# 3GPP TSG-RAN WG4 Meeting #117	R4-2521240

# Dallas, USA, Nov. 17-21, 2025

Source: 	OPPO

Agenda Item:	7.9.2

Title: 	General issues on CSI compression

Document for:	Approval

Introduction

In R20 WID, following objectives are approved for the inter-vendor training collaboration for two-sided AI/ML models

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

In this contribution, some general issues on CSI compression are analysed.

Discussion

As guided in the end of last RAN4 meeting, at least following issues should be clarified first, that may be helpful to get an aligned understanding among companies.

Assumptions on the dataset

In R19, a mixed dataset based on the UMa channel assumption was developed using inputs from five companies. Based on this mixed dataset, RAN4 evaluated the performance under different model assumptions, the compatibility between various encoders and a fixed decoder, and the impact of different encoder complexities on the performance of CSI compression use cases.

In R20, to fully specify a reference model in RAN4, this mixed dataset could be reused initially. On one hand, no significant drawbacks have been identified with the dataset after its use in R19. On the other hand, constructing a new standardized dataset would require more discussion, evaluation, and development. Therefore, for the fully defined/specified reference model ("Direction C"), in the absence of a clearly defined new standardized dataset, the existing mixed dataset could serve as a baseline for simulation and evaluation by companies, e.g., assessing the performance of candidate standardized models like transformer backbones and ensuring performance alignment.

The existing mixed dataset could serve as a baseline for simulation and evaluation by companies, e.g., assessing the performance of candidate standardized models like transformer backbones and ensuring performance alignment.

For R20, compared to R19, scalability needs to be considered as an additional factor in the standardization of models. This change will introduce new requirements for datasets.

In R19, the mixed datasets used had fixed interface parameters, such as the number of sub-bands = 13 and the number of antenna ports = 32, and the output size of the CSI encoder was fixed at 32 floating-point numbers or 64 bits.

For 6G, to address scalability considerations, a new dataset with different assumptions on 1) the number of sub-bands, 2) the number of antenna ports, and 3) CSI payload sizes, should be considered. For instance, in terms of the number of sub-bands, beyond the currently adopted 13 sub-bands, various other sub-band configurations exist, which are directly related to the frequency domain bandwidth, e.g., 5MHz, 10MHz, 20MHz. Similarly, for the number of antenna ports, configurations such as 16, and 32 ports need to be considered. As for CSI payload sizes, the protocol already supports a range of bit sizes from tens to hundreds of bits.

For 6G, to address scalability considerations, following aspects should be considered when constructing a new data set in RAN4

    - Number of sub-bands (e.g., corresponding to 5MHz, 10 MHz, 20 MHz, with 4 RBs per sub-band)

    - Number of antenna ports (e.g., 16 and 32)

    - CSI payload sizes (e.g., 64 bits, 128 bits)

Furthermore, during the R19, RAN4 created a mixed dataset comprising 500,000 samples. For 6G, more companies are encouraged to provide their respective datasets to help create a mixed dataset with broader sources. However, it is important to note that the datasets contributed by each company should be calibrated, e.g., using the RAN4 R19 CNN based CSI encoder/decoder to validate the performance on their respective datasets, to ensure the overall quality of the RAN4 standardized dataset.

The datasets contributed by each company should be calibrated, e.g., using the RAN4 R19 CNN based CSI encoder/decoder to validate the performance on their respective datasets, to ensure the overall quality of the RAN4 standardized dataset.

Assumptions on the test decoder and reference encoder structures

In R19, a CNN-based decoder was proposed and agreed to verify whether model performance alignment could be achieved. Additionally, a CNN-based CSI encoder, along with simplified transformer-based, simplified CNN-based, and MLP-based encoders, were utilized in conjunction with the proposed CNN-based decoder.

In R20, further research on model standardization is required, with the following aspects needing consideration

- UE may differ in their AI capabilities, including supported models, architectures, and computational complexities. When designing models and defining minimum performance requirements, RAN4 should consider potential limitations arising from these differing UE capabilities.

    - Option1: Standardized models with varying complexities need to be introduced,

    - Option2: Ensuring that constraints on UE deployment(e.g., available computing resource, memory size, power consumption) are fully addressed during standardized model design.

- RAN1 provided one example structure for study purposes, and RAN4 evaluations can begin with this structure. However, the structure given by RAN1 lacks detailed parameter specifications. For RAN4 evaluations, parameter designs of varying complexities can be assessed in 2025 Q4.

In [4], we give 2 different descriptions based on the transformer backbone and also provide performance analyses of the two models with different complexity levels.

- Besides the transformer backbone, the CNN-based CSI decoder model adopted in the R19 phase, along with the accompanying simplified transformer-based, simplified CNN-based, and MLP-based CSI encoders, could also serve as candidate test decoders and reference encoder structures for evaluation. The key difference from the R19 is the need to incorporate scalability evaluation.

UE may differ in their AI capabilities, including supported models, architectures, and computational complexities.

RAN4 should consider potential limitations arising from these differing UE capabilities.

    - Option1: More than one standardized model with varying complexities need to be introduced,

    - Option2: Ensuring that constraints on UE deployment(e.g., available computing resource, memory size, power consumption) are fully addressed during standardized model design.

Based on the transformer backbone that provided by RAN1, RAN4 evaluations can begin with this structure, detailed designs of varying complexities can be assessed in 2025 Q4.

Assumptions on scalability solutions

In terms of specific solutions for designing model architectures to address scalability, the following approaches should be considered:

- Pre/post-processing with truncation/padding: For example, when the input to the CSI compression model has 32 antenna ports, no additional processing is needed. However, when the input has only 16 antenna ports, 16 zeros may be padded to match the model's expected input dimensions. Similarly, for the output of the CSI recovery model, when 32 antenna ports are required, no adjustment is needed, but for 16 antenna ports, a subset may be truncated to produce the corresponding output.

- Adaptation layers in the AI/ML model: Introduce an Adaptation layers in the AI/ML model, to accommodate different input dimensions of the CSI encoder, CSI payload size (output dimensions of the CSI encoder, input dimensions of the CSI decoder), and output dimensions of the CSI decoder.

- Concatenating multiple models: If higher input or output dimensions are required in practical applications, perform inference by concatenating multiple models. For example, for a 10 MHz bandwidth, use two 5 MHz models separately.

- Above approaches can be combined.

- Additionally, when considering scalability issues, it is also necessary to evaluate whether to adopt a "single model structure + single set of model parameters" approach or a "single model structure + multiple model parameters" approach to address scalability challenges.

In terms of specific solutions for designing model architectures to address scalability, the following approaches should be considered

    - Pre/post-processing with truncation/padding

    - Adaptation layers in the AI/ML model

    - Concatenating multiple models

RAN4 to evaluate whether to adopt "single model structure + single set of model parameters" or “single model structure + multiple model parameters" to address scalability challenges

Test metric and Channel model for the requirements

For CSI prediction, Channel model for CSI prediction tests has been agreed in RAN4 #113

Agreement (RAN4 #113): For channel model for CSI prediction tests, TDL channel models will be used.

For CSI prediction, test metric has been agreed in RAN4#114bis

Agreement (RAN4 #113): Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/MLusing the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

Regarding the test metric and channel model used for CSI compression, the same assumptions as those for the CSI prediction could be reused. This is because both CSI compression and CSI prediction essentially evaluate the performance of PMI reporting in testing, only difference is through different methodologies.

RAN4 could further assess whether the performance requirement for AI based CSI compression differ from those of traditional PMI reporting. However, for the channel model and the test metric themselves, there is no need to introduce new assumptions.

Regarding the test metric and channel model used for CSI compression, the same assumptions as those for the CSI prediction could be reused.

Regarding the performance metrics for evaluating different potential model structures, system throughput can certainly be used as evaluation criteria. However, in evaluations, if all candidate models are compared based on system throughput, the evaluation efficiency may be relatively low. Therefore, when assessing the performance of different models, SGCS could still be adopted as the evaluation metric.

Regarding the performance metrics for evaluating different potential model structures, SGCS could be adopted as the evaluation metric.

How to capture the model and the dataset in 3GPP

Regarding the model structure, it could be captured in 3GPP through a description of the architecture. However, directly capturing the model parameters in 3GPP, such as explicitly listing each parameter value, would be a challenging task. For model parameters, an alternative approach could be the parameters are stored on 3GPP server, while the storage location of these model parameters could be referenced in a relevant 3GPP TS or TR documents, thereby serving as a means to capture the model parameters. Regarding the dataset, similar like the model parameter, it could be stored on 3GPP server as well. The storage location could be referenced in a relevant 3GPP TS or TR documents.

Regarding the model structure, it could be captured in 3GPP by a description of the architecture.

Regarding the model parameter and dataset, it could be stored on 3GPP server, while the storage location of the model parameter and dataset could be referenced in a relevant 3GPP TS or TR documents.

Conclusion

In this contribution, general issues on CSI compression are analysed and got the following proposals:

Proposal 1: 	The existing mixed dataset could serve as a baseline for simulation and evaluation by companies, e.g., assessing the performance of candidate standardized models like transformer backbones and ensuring performance alignment.

- Proposal 2: 		For 6G, to address scalability considerations, following aspects should be considered when constructing a new data set in RAN4

    - Number of sub-bands (e.g., corresponding to 5MHz, 10 MHz, 20 MHz, with 4 RBs per sub-band)

    - Number of antenna ports (e.g., 16 and 32)

    - CSI payload sizes (e.g., 64 bits, 128 bits)

- Proposal 3: 		The datasets contributed by each company should be calibrated, e.g., using the RAN4 R19 CNN based CSI encoder/decoder to validate the performance on their respective datasets, to ensure the overall quality of the RAN4 standardized dataset.

- Observation 1: 	UE may differ in their AI capabilities, including supported models, architectures, and computational complexities.

- Proposal 4: 		RAN4 should consider potential limitations arising from these differing UE capabilities.

    - Option1: More than one standardized model with varying complexities need to be introduced,

    - Option2: Ensuring that constraints on UE deployment(e.g., available computing resource, memory size, power consumption) are fully addressed during standardized model design.

- Proposal 5: 		Based on the transformer backbone that provided by RAN1, RAN4 evaluations can begin with this structure, detailed designs of varying complexities can be assessed in 2025 Q4.

- Proposal 6: 		In terms of specific solutions for designing model architectures to address scalability, the following approaches should be considered

    - Pre/post-processing with truncation/padding

    - Adaptation layers in the AI/ML model

    - Concatenating multiple models

- Proposal 7: 		RAN4 to evaluate whether to adopt "single model structure + single set of model parameters" or “single model structure + multiple model parameters" to address scalability challenges

- Proposal 8: 		Regarding the test metric and channel model used for CSI compression, the same assumptions as those for the CSI prediction could be reused.

- Proposal 9: 		Regarding the performance metrics for evaluating different potential model structures, SGCS could be adopted as the evaluation metric.

- Proposal 10: 		Regarding the model structure, it could be captured in 3GPP by a description of the architecture.

- Proposal 11: 		Regarding the model parameter and dataset, it could be stored on 3GPP server, while the storage location of the model parameter and dataset could be referenced in a relevant 3GPP TS or TR documents.

Reference

RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

- [4] R4-2521241, Inter-vendor training collaboration and interoperability for two-sided AIML models