3GPP TSG-RAN WG4 Meeting #117	R4-2521302

# Dallas, US, 17 - 21 Nov, 2025

Title:	Discussion on HPUE Basket WI in Rel-20

Agenda item:	13.1.2

WI/SI:	N/A

Release:	Rel-19

Source:	China Telecom

Document for:	Discussion

# 1 Background

This contribution provides our view on HPUE basket Work Items in Release 20. While we acknowledge the benefits of the newly introduced HPUE MSD Look-Up Tables (LUTs) in simplifying the specification process, we believe that discontinuing HPUE basket WIDs in Rel-20 is premature.

The current Rel-19 HPUE basket WIs still contain numerous pending combinations that are not addressed by the MSD LUTs. This contribution outlines specific cases where the LUT-based approach falls short and argues for the continuation of HPUE basket WI in Rel-20 to ensure comprehensive HPUE support.

# 2 Discussion

## 2.1 Intra-band CA

The current Rel-19 HPUE basket includes a significant number of requirements specifically for DL intra-band CA configurations. These scenarios often involve unique reference sensitivity requirements. Discontinuing the HPUE basket in Rel-20 would leave these intra-band CA combinations without a clear path for standardization.

Observation 1: The LUTs are not applicable to intra-band CA configurations.

## 2.2 Inter-band CA with single UL band 2 CCs

Another critical area is inter-band CA with single UL band multiple CCs, e.g., DL inter-band CA with UL intra-band CA configurations. The current HPUE basket WIs include numerous combinations for such configurations, which are not adequately covered by the LUTs. Without a dedicated WI, it is unlikely that these scenarios will be properly specified in Rel-20.

Observation 2: The LUTs are not applicable to DL inter-band CA with UL intra-band CA configurations.

2.3 Hybrid inter-band and intra-band UL CA

The Rel-19 HPUE basket also addresses UL hybrid inter-band and intra-band CA configurations, e.g., 2 UL band CA configurations with 3 UL CCs. These configurations combine the complexities of both inter-band and intra-band CA, resulting in MSD and power class requirements that are highly configuration specific. The current LUT framework does not cover these hybrid cases, and discontinuing the basket WI would risk leaving them unsupported.

Observation 3: The LUTs a not applicable to UL hybrid inter-band and intra-band CA configurations.

2.4 Future MSD Specification Gaps

Looking ahead, we anticipate scenarios, where the PC3 MSD was too low to be specified, while the PC2/PC1.5 MSD was substantial enough to require explicit specification. This case is beyond the scope of LUTs solution.

Observation 4: It is anticipated that certain cases may arise in which MSD requirements are not necessary for PC3 but are required for high power class. The LUTs solution is unable to accommodate this scenario.

2.5 Release 20 HPUE basket WI

As discussed above, while the introduction of HPUE MSD LUTs is a positive step toward simplifying the specification process, it is not yet sufficient to cover all HPUE-related scenarios—especially those combinations involving intra-band CA and combinations without PC3 MSD requirements.

During the previous meeting, it was suggested that HPUE combinations could be addressed within the PC3 WI. LUTs are intended solely as a solution to facilitate the introduction of HPUE band combinations. Combining PC3 combinations with those for PC2 or PC1.5 may lead to confusion in future work. We recommend continuing the inclusion of dedicated HPUE basket WIDs in Release 20 to ensure comprehensive coverage of all high-power CA configuration requests. This will provide a clear and consistent framework for HPUE support across all relevant band combinations and CA types.

Proposal 1: Around 3000 band combinations are still ongoing in current Rel-19 HPUE WI. Considering the limitations of LUTs and to establish a clear and consistent framework for HPUE support, a dedicated Rel-20 HPUE basket WI for NR CA/DC band combinations is required.

Proposal 2: Suggestions on Rel-20 basket WI

| Category | R19 WI | R20 WI |
| --- | --- | --- |
| LTE band combination (PC3) | Rel-19 LTE-Adv CA for x bands (1<=x<= 6) DL with y bands (y=1, 2) UL [RAN4 WI: LTE_CA_R19_xBDL_yBUL-Core] | Rel-20 LTE-Adv CA for x bands (1<=x<= 6) DL with y bands (y=1, 2) UL |
| LTE HPUE band (PC2/PC1 FWA) | Rel-19 HPUE (PC2) and high-power operation (PC1) for fixed-wireless/vehicle-mounted use cases in a single LTE band [RAN4 WI: HPUE_LTE_bands_R19-Core] | No need. |
| NR band combination (PC3) | Rel-19 NR CA/DC for x bands DL with y bands UL (x<7, y<3) and SUL band/CA band combinations with a single SUL or two SUL cells [RAN4 WI: NR_CADC_SUL_R19-Core] | Rel-20 NR CA/DC for x bands DL with y bands UL (x<7, y<3) and SUL band/CA band combinations with a single SUL or two SUL cells |
| NR HPUE band combination (PC2, PC1.5) | Rel-19 HPUE (PC1.5 or 2) for NR intra-band CA or NR inter-band CA/DC band combinations with/without NR SUL [RAN4 WI: HPUE_NR_CADC_SUL_R19-Core] | Rel-20 High power UE (PC1.5 or 2) for NR Carrier Aggregation (CA)/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL) |
| NR band HPUE (PC2, PC1.5, PC1 FWA) | Rel-19 HPUE (PC1.5 and 2) for NR FR1 TDD/FDD single band for handheld/FWA UEs, and HPUE operation (PC1) for fixed-wireless/vehicle-mounted use cases in a single NR band [RAN4 WI: HPUE_NR_FR1_bands_R19-Core] | Rel-20 HPUE (PC1.5 and 2) for NR FR1 TDD/FDD single band for handheld/FWA UEs, and HPUE operation (PC1) for fixed-wireless/vehicle-mounted use cases in a single NR band |
| DC of LTE and NR bands (PC3) | Rel-19 DC of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR SUL bands [RAN4 WI: DC_R19_xBLTE_yBNR-Core] | Rel-20 DC of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR SUL bands |
| HPUE for DC of LTE and NR bands (PC2, PC1.5) | Rel-19 HPUE (PC1.5 or 2) for DC combinations of LTE band(s) and NR band(s) [RAN4 WI: HPUE_DC_LTE_NR_R19-Core] | Rel-20 HPUE (PC1.5 or 2) for DC combinations of LTE band(s) and NR band(s) |

In previous meetings, it also was proposed to remove all HPUE applicability notes from clause 5. However, we have identified cases in which MSD requirements are not needed for either PC3 or PC2/PC1.5. If all such notes are removed from clause 5, there would be no clear indication of whether the configuration for PC2/PC1.5 has been completed. Therefore, if the intention is to remove all applicability notes from clause 5, it is necessary to include a row with the value “0” in the MSD tables for PC2/PC1.5 to explicitly indicate configuration completeness.

Proposal 3: Introduce a new row with the value “0” into MSD table to indicate the PC2/PC1.5 combination is completed after removing all notes from clause 5 for the combinations of which MSD requirements are not needed for both PC3 and PC2/PC1.5. This method can be further discussed in Rel-20 HPUE basket WI.

# 3 Conclusion

Observation 1: The LUTs are not applicable to intra-band CA configurations.

Observation 2: The LUTs are not applicable to DL inter-band CA with UL intra-band CA configurations.

Observation 3: The LUTs a not applicable to UL hybrid inter-band and intra-band CA configurations.

Observation 4: It is anticipated that certain cases may arise in which MSD requirements are not necessary for PC3 but are required for high power class. The LUTs solution is unable to accommodate this scenario.

Proposal 1: Around 3000 band combinations are still ongoing in current Rel-19 HPUE WI. Considering the limitations of LUTs and to establish a clear and consistent framework for HPUE support, a dedicated Rel-20 HPUE basket WI for NR CA/DC band combinations is required.

Proposal 2: Suggestions on Rel-20 basket WI:

| Category | R19 WI | R20 WI |
| --- | --- | --- |
| LTE band combination (PC3) | Rel-19 LTE-Adv CA for x bands (1<=x<= 6) DL with y bands (y=1, 2) UL [RAN4 WI: LTE_CA_R19_xBDL_yBUL-Core] | Rel-20 LTE-Adv CA for x bands (1<=x<= 6) DL with y bands (y=1, 2) UL |
| LTE HPUE band (PC2/PC1 FWA) | Rel-19 HPUE (PC2) and high-power operation (PC1) for fixed-wireless/vehicle-mounted use cases in a single LTE band [RAN4 WI: HPUE_LTE_bands_R19-Core] | No need. |
| NR band combination (PC3) | Rel-19 NR CA/DC for x bands DL with y bands UL (x<7, y<3) and SUL band/CA band combinations with a single SUL or two SUL cells [RAN4 WI: NR_CADC_SUL_R19-Core] | Rel-20 NR CA/DC for x bands DL with y bands UL (x<7, y<3) and SUL band/CA band combinations with a single SUL or two SUL cells |
| NR HPUE band combination (PC2, PC1.5) | Rel-19 HPUE (PC1.5 or 2) for NR intra-band CA or NR inter-band CA/DC band combinations with/without NR SUL [RAN4 WI: HPUE_NR_CADC_SUL_R19-Core] | Rel-20 High power UE (PC1.5 or 2) for NR Carrier Aggregation (CA)/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL) |
| NR band HPUE (PC2, PC1.5, PC1 FWA) | Rel-19 HPUE (PC1.5 and 2) for NR FR1 TDD/FDD single band for handheld/FWA UEs, and HPUE operation (PC1) for fixed-wireless/vehicle-mounted use cases in a single NR band [RAN4 WI: HPUE_NR_FR1_bands_R19-Core] | Rel-20 HPUE (PC1.5 and 2) for NR FR1 TDD/FDD single band for handheld/FWA UEs, and HPUE operation (PC1) for fixed-wireless/vehicle-mounted use cases in a single NR band |
| DC of LTE and NR bands (PC3) | Rel-19 DC of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR SUL bands [RAN4 WI: DC_R19_xBLTE_yBNR-Core] | Rel-20 DC of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR SUL bands |
| HPUE for DC of LTE and NR bands (PC2, PC1.5) | Rel-19 HPUE (PC1.5 or 2) for DC combinations of LTE band(s) and NR band(s) [RAN4 WI: HPUE_DC_LTE_NR_R19-Core] | Rel-20 HPUE (PC1.5 or 2) for DC combinations of LTE band(s) and NR band(s) |

Proposal 3: Introduce a new row with the value “0” into MSD table to indicate the PC2/PC1.5 combination is completed after removing all notes from clause 5 for the combinations of which MSD requirements are not needed for both PC3 and PC2/PC1.5. This method can be further discussed in Rel-20 HPUE basket WI.