3GPP TSG-RAN WG4 Meeting #117										    R4-2521329

Dallas, USA, Nov. 17-21, 2025

Source: 	OPPO

Title: 	On RRM requirements for (e)RedCap UE enhancement in RRM phase 6

Agenda Item:	7.4.3.2

Document for:	Discussion

Introduction

In last meeting, a WF was approved on NR RRM phase 6 [1]. RAN4 achieved multiple agreements on the scope and principle for (e)Redcap UE enhancement requirements.

In this document, we provide our views on the RRM requirements of (e)RedCap UE enhancement.

Discussion

Issue 2-2-1: Scope of relaxed RLM/BFD for (e)RedCap UE Agreement: RAN4 only considers the relaxed RLM/BFD requirements on PCell when the DRX cycle is no longer than 80 ms. Issue 2-2-2: Relaxed RLM/BFD for 2Rx (e)RedCap UE Agreement: - Reuse the existing R17 RLM/BFD relaxation requirements for both FR1 and FR2, and for full-duplex FDD and TDD.

Existing Rel-17 RLM/BFD relaxation for power saving has been specified only for non-RedCap UEs in FR1 and FR2, and it is natural to extend it to (e)RedCap UEs. The applicability rules can also apply, such as the relaxed RLM/BFD requirements on PCell when the DRX cycle is no longer than 80 ms.

For HD-FDD, we think the conditions can be reused for relaxed RLM/BFD requirements, such as

  - Minimum requirement for L1 indication: at least one RS sample must fall with DL occasion within an indication period, and

  - Scheduling and Collision: whether to apply scheduling restriction or UL transmission in case of collision between CSI-RS resources to be measured and UL transmission.

The legacy conditions for HD-FDD Redcap UE can still apply when UE support RLM/BFD relaxation, without additional conditions. The existing R17 RLM/BFD relaxation evaluation periods can be reused for 2Rx redcap UE.

For 1 Rx redcap UE, RAN4 still needs to discuss whether the similar relaxation/margin compared to 2Rx can be reused.

Observation 1: The legacy conditions for HD-FDD Redcap UE can still apply when UE support RLM/BFD relaxation, including those for minimum requirement for L1 indication and scheduling and collision.

Proposal 1: For HD-FDD Redcap UE, the R17 RLM/BFD relaxation requirements such as evaluation periods can be reused for 2Rx Redcap UE.

Issue 2-1-1: Scope for LT5 (e)RedCap UE Agreement: - For (e)RedCap UE supporting LT5: - CSI-RS based measurement is out of scope. - RAN4 only defines the requirements for PCell. - RAN4 only defines the requirements for NR FR1. - Introduce support for only 15 PRB transmission bandwidth configuration and 12 PRB SSB Issue 2-1-2: LT5 for 2Rx (e)RedCap UE Agreement: - The existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes. - Handover - RLM/BFD - Intra-frequency/Inter-frequency measurement - Further check for HD-FDD Issue 2-1-4: RLM/BFD impact of LT5 for 1Rx (e)RedCap UE Agreement: For discussing the PDCCH hypothetical parameters, RAN4 discuss whether it is feasible to target on coverage equivalence between 1Rx and 2Rx RedCap UEs.

Furthermore, it is important to extend less than 5MHz (LT5) support from normal UE to (e)RedCap UEs to reduce complexity and cost.

For LT5 2Rx (e)RedCap UE, the existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes. For HD-FDD Redcap UE, the legacy conditions for HD-FDD Redcap UE can still apply when UE support RLM/BFD relaxation.

Proposal 2: For HD-FDD Redcap UE, the existing LT5 requirements for non-RedCap UE such as evaluation periods can be reused for 2Rx (e)RedCap UE.

About the target on coverage equivalence between 1Rx and 2Rx RedCap UEs, from network side, more repetition and high aggregation level of DL resources can be configured to improve the DL coverage. It depends on the simulation results based on the simulation assumptions.

Observation 2: Whether it is feasible to target on coverage equivalence between 1Rx and 2Rx RedCap UEs, depends on network configuration and simulation assumption.

About the simulation assumption on detection Method, one shot detection (i.e., no combination for different PBCHs) is set as baseline. We are open to Soft Combining as Optional. Companies can provide simulation results for or soft combining on top of one-shot detection.

Whether the difference of RS samples between 1Rx and 2Rx Redcap UE for normal RLM or BFD requirements can apply to 1Rx and 2Rx RLM/BFD relaxation, need to be discussed and evaluated.

For RLM IS evaluation period without relaxation, there are no difference between 1Rx and 2Rx Redcap UE, as specified in clause 8.1B.2. But for RLM OOS evaluation period, 1Rx UE relaxed the measured samples by 2 times, as the tables below. The similar conclusion can be found for BFD comparing 1Rx and 2Rx.

Observation 3: For RLM IS requirements without relaxation, there are no difference between 1Rx and 2Rx Redcap UE, For RLM OOS and BFD evaluation period, 1Rx UE relaxed the measured samples by 2 times.

- 1Rx and 2Rx Redcap UE without RLM relaxation Table 8.1B.2.2-1: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 2Rx RedCap UE Configuration; TEvaluate_out_SSB,RedCap (ms); TEvaluate_in_SSB,RedCap (ms). no DRX; Max(200, Ceil(10 P) TSSB); Max(100, Ceil(5 P) TSSB). DRX cycle≤320 ms; Max(200, Ceil(15 P) Max(TDRX,TSSB)); Max(100, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle>320 ms; Ceil(10 P) TDRX; Ceil(5 P) TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length.. Table 8.1B.2.2-3: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE Configuration; TEvaluate_out_SSB,RedCap (ms); TEvaluate_in_SSB,RedCap (ms). no DRX; Max(400, Ceil(20 P) TSSB); Max(100, Ceil(5 P) TSSB). DRX cycle≤320 ms; Max(400, Ceil(30 P) Max(TDRX,TSSB)); Max(100, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle>320 ms; Ceil(20 P) TDRX; Ceil(5 P) TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length.. - 1Rx and 2Rx Redcap UE without BFD relaxation Table 8.5B.2.2-1: Evaluation period TEvaluate_BFD_SSB_Redcap for FR1 Configuration; TEvaluate_BFD_SSB_Redcap (ms) for Redcap UE with 2 Rx; TEvaluate_BFD_SSB_Redcap (ms) for 1 Rx Redcap. no DRX; Max(50, Ceil(5 P) TSSB); [ Max(50, Ceil(10 P) TSSB)]. DRX cycle ≤ 320 ms; Max(50, Ceil(7.5 P) Max(TDRX,TSSB)); [ Max(50, Ceil(15 P) Max(TDRX,TSSB)) ]. DRX cycle > 320 ms; Ceil(5 P) TDRX; [ Ceil(10 P) TDRX]. NOTE: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length..

- Normal UE with RLM relaxation Table 8.1.2.4-1: Evaluation period TEvaluate_out_SSB_Relax for FR1 Configuration; TEvaluate_out_SSB_Relax (ms). Max(TDRX,TSSB) ≤80 ms; Max(200 K3 NOTE3, Ceil(15 K1NOTE2 P) Max(TDRX,TSSB)). 80 ms < Max(TDRX,TSSB) ≤160 ms; Ceil(15 P) Max(TDRX,TSSB). NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K1 = 4 for Max(TDRX,TSSB) ≤40 ms and K1 = 2 for 40 ms<Max(TDRX,TSSB) ≤80 ms. NOTE 3: K3 = K1, if K1 ≤ 2; otherwise K3 = 1.. Table 8.1.3.4-1: Evaluation period TEvaluate_out_CSI-RS_Relax for FR1 Configuration; TEvaluate_out_CSI-RS_Relax (ms). Max(TDRX, TCSI-RS) ≤ 80 ms; Max(200 K3 NOTE3, Ceil(1.5 Mout P K1 NOTE2) Max(TDRX, TCSI-RS) NOTE1). NOTE1: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length and no longer than 80 ms. NOTE2: K1 = 2 for 40 ms < MAX(TDRX, TCSI-RS) ≤ 80 ms, K1 = 4 for MAX(TDRX, TCSI-RS) ≤ 40 ms NOTE3: K3 = K1, if K1 ≤ 2; K3 = 1 otherwise.. - Normal UE with BFD relaxation Table 8.5.2.4-1: Evaluation period TEvaluate_BFD_SSB_Relax for FR1 Configuration; TEvaluate_BFD_SSB_Relax (ms). Max(TDRX, TSSB) ≤ 80 ms; Max(50 K3, Ceil(7.5 K1 P) Max(TDRX,TSSB)). 80 ms＜Max(TDRX, TSSB) ≤ 160 ms; Max(50, Ceil(7.5 P) Max(TDRX,TSSB)). Note 1: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length and no longer than 80ms. Note 2: K1 is the relaxation factor. K1 = 2 for 40ms＜Max(TDRX, TSSB) ≤ 80 ms, K1 = 4 for Max(TDRX, TSSB) ≤ 40 ms Note 3: K3 is the relaxation factor for the lower bound. K3 = K1, if 1 < K1 ≤ 2; K3 = 1 otherwise.. Table 8.5.3.4-1: Evaluation period TEvaluate_BFD_CSI-RS_Relax for FR1 Configuration; TEvaluate_BFD_CSI-RS Relax (ms). Max(TDRX, TCSI-RS) ≤ 80 ms; Max(50 K3, Ceil(K1 × 1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)). NOTE 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K1 is the relaxation factor. K1 = 2 for 40 ms < MAX(TDRX, TCSI-RS) ≤ 80 ms, K1 = 4 for MAX(TDRX, TCSI-RS) ≤ 40 ms NOTE 3: K3 is the relaxation factor for the lower bound. K3 = K1, if 1 < K1 ≤ 2; K3 = 1 otherwise..

For RLM/BFD relaxation, scaling factor K3 for lower bound and K1 for relaxed samples, can be referred.

We prefer to reuse K1 = 2 for 40 ms < MAX(TDRX, TCSI-RS) ≤ 80 ms, K1 = 4 for MAX(TDRX, TCSI-RS) ≤ 40 ms for 2Rx Redcap UE.

Proposal 4: For 2Rx Redcap UE supporting RLM/BFD relaxation, reuse the relaxation factor K1 and K3.

- K1 = 2 for 40 ms < MAX(TDRX, TCSI-RS) ≤ 80 ms, K1 = 4 for MAX(TDRX, TCSI-RS) ≤ 40 ms.

- K3 is the relaxation factor for the lower bound. K3 = K1, if 1 < K1 ≤ 2; K3 = 1 otherwise.

For 1Rx Redcap UE supporting RLM/BFD relaxation, based on observation 4, we prefer further relaxation by 2 times on top of the relaxed requirements for 2Rx Redcap UE, in case of RLM OOS and BFD. For RLM IS, we are ok to use the same relaxed requirements for 1Rx and 2Rx Redcap UE.

Proposal 5: For 1Rx Redcap UE supporting RLM/BFD relaxation, RAN4 to define

- further relaxation by 2 times on top of the relaxed requirements for 2Rx Redcap UE in case of RLM OOS and BFD.

- the same relaxed requirements for 1Rx and 2Rx Redcap UE in case of RLM IS.

For LT5 1Rx (e)RedCap UE, we are still preparing the simulation. Whether to use the same or more relaxation as 1Rx Redcap UE, can be further discussed based on simulation input later.

Proposal 6: Further discuss whether to use the same or larger relaxation factors for LT5 UE.

Issue 2-2-4: Entry and exit criteria, singling indication, applicability rules for 2Rx Agreement: Reuse R17 RLM/BFD Entry and exit criteria from legacy UE as a baseline

For exit criteria, we think the legacy can be reused as below.

- RLM exit Criteria - The UE is no longer allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met: - The UE sends out-of sync indications to the higher layers, - The timer T310 is running. - No DRX is configured or configured DRX cycle is longer than 80 ms - BFD exit Criteria - The UE is no longer allowed to relax BFD measurements and apply the relaxed link recovery procedures provided that at least one of the following conditions is met: The timer beamFailureDetectionTimer is running. - No DRX is configured or configured DRX cycle is longer than 80 ms

For entry criteria, two cases include

- Case 1: only goodServingCellEvaluationBFD is configured and good serving cell quality criterion is fulfilled

- Case 2: both goodServingCellEvaluationBFD and lowMobilityEvaluationConnected are configured, and both low mobility criterion defined in clause 5.7.13.1 of TS 38.331 [2] and good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] are fulfilled for the serving cell configured with RLM RS or BFD RS.

For relaxed measurement Criteria, low mobility criterion and good serving cell quality criterion can be reused from R17. The similar cases are considered for Redcap UE.

Proposal 7: For entry criteria, two cases are considered as well for Redcap UE and LT5 UE supporting RLM/BFD relaxation.

- Case 1: only goodServingCellEvaluationBFD is configured and good serving cell quality criterion is fulfilled

- Case 2: both goodServingCellEvaluationBFD and lowMobilityEvaluationConnected are configured, and both low mobility criterion and good serving cell quality criterion are fulfilled

About UE capabilities, rlm-Relaxation-r17 and bfd-Relaxation-r17 can be extended to Redcap UE or LT5 UE. Whether to support early implementation or release independence, also need to be discussed in RAN4. We prefer it can start from R17.

Proposal 8: UE capabilities for RLM/BFD relaxation can be extended to Redcap UE or LT5 UE. Early implementation or release independence issues need to be clarified.

Conclusions

Based on above analysis, we have following observations and proposals.

HD-FDD

Observation 1: The legacy conditions for HD-FDD Redcap UE can still apply when UE support RLM/BFD relaxation, including those for minimum requirement for L1 indication and scheduling and collision.

Proposal 1: For HD-FDD Redcap UE, the R17 RLM/BFD relaxation requirements such as evaluation periods can be reused for 2Rx Redcap UE.

Proposal 2: For HD-FDD Redcap UE, the existing LT5 requirements for non-RedCap UE such as evaluation periods can be reused for 2Rx (e)RedCap UE.

The difference between 1Rx and 2Rx

Observation 2: Whether it is feasible to target on coverage equivalence between 1Rx and 2Rx RedCap UEs, depends on network configuration and simulation assumption.

Observation 3: For RLM IS requirements without relaxation, there are no difference between 1Rx and 2Rx Redcap UE, For RLM OOS and BFD evaluation period, 1Rx UE relaxed the measured samples by 2 times.

The relaxation factors of UE supporting RLM/BFD relaxation

Proposal 4: For 2Rx Redcap UE supporting RLM/BFD relaxation, reuse the relaxation factor K1 and K3.

- K1 = 2 for 40 ms < MAX(TDRX, TCSI-RS) ≤ 80 ms, K1 = 4 for MAX(TDRX, TCSI-RS) ≤ 40 ms.

- K3 is the relaxation factor for the lower bound. K3 = K1, if 1 < K1 ≤ 2; K3 = 1 otherwise.

Proposal 5: For 1Rx Redcap UE supporting RLM/BFD relaxation, RAN4 to define

- further relaxation by 2 times on top of the relaxed requirements for 2Rx Redcap UE in case of RLM OOS and BFD.

- the same relaxed requirements for 1Rx and 2Rx Redcap UE in case of RLM IS.

Proposal 6: Further discuss whether to use the same or larger relaxation factors for LT5 UE.

The criteria and UE capabilities of RLM/BFD relaxation

Proposal 7: For entry criteria, two cases are considered as well for Redcap UE and LT5 UE supporting RLM/BFD relaxation.

- Case 1: only goodServingCellEvaluationBFD is configured and good serving cell quality criterion is fulfilled

- Case 2: both goodServingCellEvaluationBFD and lowMobilityEvaluationConnected are configured, and both low mobility criterion and good serving cell quality criterion are fulfilled

Proposal 8: UE capabilities for RLM/BFD relaxation can be extended to Redcap UE or LT5 UE. Early implementation or release independence issues need to be clarified.

- Reference

[1]. R4-2514821, WF on NR RRM Phase 6.