3GPP TSG-RAN WG4 Meeting #117	                             R4-2521343

Dallas, USA, November 17th– 21st, 2025

Title: 	Discussion on RRM requirements for low band carrier aggregation via switching

Source: 	OPPO

Agenda item:	4.8.2

Document for:	Approval

Introduction

The core requirements for low band carrier aggregation via switching has been completed in RAN4#116 meeting. But there are still some remaining issues to be discussed, and the discussion and agreements in last meeting was captured in [1][2]. In this document, we will further discuss the remaining issues and give our proposals.

Discussion

Issue 1-1-3a: Application time of Switching Pattern in SCell activation Issue 1-1-3b: switching pattern based carrier switching in SCell activation - Recommended WF - Options for discussion. Please note the following T1 and TLBCA is up to the conclusion from issue 1-1-3a. - Option 1: immediate switching(CATT, HW, Apple, ZTE, Ericsson, QC) (using Ericsson wording) - For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(THARQ+3ms+TLBCA)/slot-length. Switching is placed at the end of the slot. - For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+( TRRC_Process+T1+TLBCA)/slot-length. Switching is placed at the end of the slot. - Option 2: Wait for next switching period (LGE, Nokia, vivo) (using vivo wording) - If UE should be on PCell according to switching pattern after THARQ+3+TLBCA ms or TRRC_Process + T1+TLBCA after MAC-CE or RRC based SCell activation PDSCH is received, UE performs switching according to the configured switching pattern. Switching is placed at the end of the slot. - If UE should be on SCell according to switching pattern after THARQ+3ms+TLBCA or TRRC_Process + T1+TLBCA after MAC-CE or RRC based SCell activation PDSCH is received, UE switches to SCell at the next switching period or UE performs switching based on dwell time on SCell. Switching is placed at the end of the slot. Agreement: - TLBCA =1ms for MAC CE based, 0 ms for RRC based.

In last meeting, it was agreed that UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length or slot n+(THARQ+3ms+TLBCA)/slot-length for SCell activation where TLBCA =1ms for MAC CE based, 0 ms for RRC based. But the UE behavior after this time is not decided. Among the above two options, we think option 1 is more reasonable, and UE can switch to the SCell on the first slot that belongs to the SCell as per the switching pattern after it is applied. In last meeting, one argument is that the reaming time PCell may be not enough for UE to switch, but we think it is a corner case and can be avoided by NW configuration. Wait for another switching period will unnecessarily extend the activation delay.

Proposal 1: The UE behavior during SCell activation after the switching pattern is applied can be defined as below:

- For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+ (THARQ+3ms+TLBCA)/slot-length. Switching is placed at the end of the slot.

- For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+( TRRC_Process+ TLBCA)/slot-length. Switching is placed at the end of the slot.

Issue 1-1-4a: Application time of disabling switching pattern in SCell deactivation Agreement: - The switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation - TLBCA =1 ms for MAC-CE based - Further discuss for timer triggered SCell deactivation Issue 1-1-4b: switching pattern based carrier switching in SCell activation Agreement: - If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation. - Following RAN1 logic, the carrier switching, if applicable, shall be placed at the end of a slot. - FFS RAN4 to capture this sub-bullet in the CR. - Further discuss for timer triggered SCell deactivation

In last meeting, it was agreed that the switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation, where TLBCA =1ms for MAC CE based, and the value for timer based is FFS. In our understanding, the time is also needed for timer based SCell deactivation since the switching pattern loading from high layer is needed as discussed in last meeting and the case is same as MAC CE based SCell activation.

The other issue is the UE behavior after this time. Since UE will not apply the switching pattern after this time and the SCell is deactivated, UE can only work on PCell unless the measurement on deactivated SCell is needed. So it is agreed in last meeting that UE will switch to PCell immediately if it is on SCell after deactivation. We think this behavior is also same for timer based SCell activation. And basically the switching is at the end of slot. The question is whether we need to capture it in the spec. Since it is aligned with RAN1 logic and after this time, the SCell is deactivated, whether or not the switching is at the end of slot does not have much impact on NW scheduling, we think it is OK not to emphasize it in the specification.

Proposal 2: The UE behavior after timer based SCell deactivation can be defined as below:

- If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation.

Proposal 3: No need to capture in the specification that the carrier switching after SCell deactivation, if applicable, shall be placed at the end of a slot.

Issue 1-4-2: UE capability for RRC based SDL SCell activation in LB CA [Way forward]: - FFS: - Proposal 1 (QC): - RAN4 introduces a new capability that allows UE to signal whether it supports RRC based direct SCell activation in Rel-19 NR LB-CA. Potential details of the capability are shown in Table 1. Table 1: Details of UE capability for RRC based Direct SCell Activation in Rel-19 NR LB-CA Features; Index; Feature group; Components; Prerequisite feature groups; Consequence if the feature is not supported by the UE; Mandatory/Optional. 55. NR_LBCA_Sw; [55-x]; RRC based Direct SCell Activation in Rel-19 NR LB-CA; [UE supports RRC based direct SCell activation in Rel-19 NR LB-CA]; 55-1; [UE will not support RRC based SCell activation in Rel-19 NR LB-CA]; [Optional with capability signaling].

For UE capability, the support of LB CA has been defined. Whether it can be applied to RRC based SCell activation depends on the support of legacy RRC based SCell activation. We think we do not need to define a new UE capability. The argument in last meeting is that the legacy capability is for MCG and SCG which is not for CA, but after checking with our RAN2 delegates, we think the capability for MCG is applied to CA. If we cannot reach consensus on the interpretation, we can send LS to RAN2 for clarification. Before that, we think we do not need to add new UE capability.

Proposal 4: Do not introduce new capability for the support of RRC based direct SCell activation in Rel-19 NR LB-CA.

Conclusions

In this paper, we give our views on remaining issues for low band carrier aggregation via switching. And the following proposals are provided.

Proposal 1: The UE behavior during SCell activation after the switching pattern is applied can be defined as below:

- For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+ (THARQ+3ms+TLBCA)/slot-length. Switching is placed at the end of the slot.

- For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+( TRRC_Process+ TLBCA)/slot-length. Switching is placed at the end of the slot.

Proposal 2: The UE behavior after timer based SCell deactivation can be defined as below:

- If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation.

Proposal 3: No need to capture in the specification that the carrier switching after SCell deactivation, if applicable, shall be placed at the end of a slot.

Proposal 4: Do not introduce new capability for the support of RRC based direct SCell activation in Rel-19 NR LB-CA.

Reference

- R4-2514870 WF on NR_LBCA_Sw_RRM, Apple

- R4-2513508 Topic summary for [116bis][202] NR_LBCA_Sw_RRM, Apple