3GPP TSG-RAN WG4 Meeting #117	R4-2521364

# Dallas, US, November 17th-21st, 2025

Title: 	Discussion on 6Rx release independency issues

Source: 	Huawei, HiSilicon

Agenda item:	4.6.1.3

Document for:	Discussion

Background

A Reply LS on Release independence of 6Rx[1] was agreed. In this contribution we provide our views.

- Discussion

RAN4 sent a LS to RAN2 to ask the feasibility of early implementing UE capability signalling on the support of maximum 6 DL MIMO layers for PDSCH. RAN2 answer that the feasibility was confirmed and it is up to RAN4 to make the decision:

So to answer the RAN4 question, it is possible to specify the early implementation of the UE capability signalling on the support of maximum 6 DL MIMO layers for PDSCH. RAN2 assumes that it is up to RAN4 to decide whether the feature should be made early implementable in the standard or not.

We don’t support introducing release independency for max 6 DL MIMO layers. Firstly, early Base station must change the scheduling strategy to accommodate 6 layers, which makes the scheduling complicate. Secondly, supporting max 6 DL layers is much more complicate than 4 layers from UE baseband processing respective, it’s questionable that how many UE support max DL 6 layers in early release. It is not attractive for BS vendors and operators to upgrade the NW to support max 6 DL MIMO layers with big scheduling strategy changes for those very limited number of UE that support 6 DL MIMO layers in early release. Actually how many Rel-19 UE will support max DL 6 MIMO layers is also questionable.. But considering that some UE may support 6Rx and 4 MIMO layers to improve the DL performance, and no impact to RAN1/RAN2 specification and also no impact to network scheduling strategy, RAN4 can consider that RAN4 requirements for 6Rx and 4 MIMO layers can be release independent from Rel-18.

Proposal 1:

- Not to introduce release independency for support of max DL 6 MIMO layers.

- RAN4 requirements for 6Rx and 4 MIMO layers can be release independent from Rel-18.

- Conclusion

In this contribution we provide our views on open issues for 6Rx release independency issues, the proposal is:

Proposal 1: Not to introduce release independency for 6 MIMO layers.

- Reference

[1] R2-2507743 Reply LS on Release Independence of 6Rx.