3GPP TSG-RAN WG4 Meeting #117	R4-2521366

# Dallas, US, November 17th-21st, 2025

Title: 	Discussion on 6Rx demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.1.2

Document for:	Discussion

Background

A WF [1] for 6Rx demodulation was agreed. In this contribution we provide our views on open issues.

- Discussion

One issue is test applicability rules:

Issue 1-2: Test applicability rule for 6Rx 6 layers performance requirements - Agreement: - Agree to define the applicability rules that 6 MIMO layers performance requirements are applicable for UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6. - Proposals: - Discuss whether to explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4

It is very clear that RF made an agreement that “The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications” [2]:

Issue 2-1-1: Way forward for 6-layers support for handheld UE and FWA devices Way Forward: - RAN4 Chair to take note of the following in the meeting report: - “RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19” - 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications.

This agreement shall be respected.

Proposal 1: Capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4

One issue is test applicability rule for support different number Rx antenna ports:

Issue 1-4: Test applicability rule for support different number Rx antenna ports - Agreements - Implement the following proposal into the CR, and further check the CR can be agreeable or not. - Corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass. - Under the condition that there is a similar test cases for different numbers of UE antennas. - Companies to further check which test cases can be skipped.

We support following new applicability rules corresponding to 6Rx:

If UE support both 8Rx and 6Rx, 8Rx tests shall be applied and 6Rx tests shall be skipped.

If UE passes 6Rx tests, 4Rx with rank4 test can be skipped

The proposed applicability rules are captured in Table 2-1:

Table 2-1: Proposed applicability rules

| UE support |  |  |  | PDSCH cases UE should pass | Applicability rules |
| --- | --- | --- | --- | --- | --- |
| 8Rx | 6Rx | 4Rx | 2Rx |  |  |
|  |  |  | √ | 2Rx test |  |
|  |  | √ | √ | 4Rx test |  |
|  |  | √ |  |  |  |
|  | √ | √ | √ | 6Rx+4Rx test | If UE passes tests in Clause 5.2.5, UE can skip Test 4-1 and Test 4-2 in Clause 5.2.3 Table 5.2.3.2.1-6 |
|  | √ | √ |  |  |  |
|  | √ |  | √ | 6Rx+2Rx test |  |
|  | √ |  |  | 6Rx test |  |
| √ | √ | √ | √ | 8Rx+4Rx test | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
| √ | √ | √ |  |  |  |
| √ |  | √ | √ |  |  |
| √ |  | √ |  |  |  |
| √ | √ |  | √ | 8Rx+2Rx test | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4 |
| √ |  |  | √ |  |  |
| √ | √ |  |  | 8Rx test |  |
| √ |  |  |  |  |  |

Proposal 2: Introduce applicability of requirements for different number of RX antenna ports listed in Table 2-1.

SDR requirements for 6Rx

One issue is MCS values for 64QAM/256QAM/1024QAM with 6 layers with scaling factor.

Issue 3-1-2: MCS value for 256QAM/ 6 layers with scaling factor 1 - Proposals - Option 1: 27 - Option 2: 24 - Option 3: 25 Issue 3-1-3: MCS value for 64QAM/ 6 layers with scaling factor 1 - Proposals - Option 1: 28 - Option 2: 26 Issue 3-1-4: MCS value for 1024QAM/ 2 layers with scaling factor 1 - Proposals - Option 1: 23 - Option 2: 24

Based on our simulation results in [1], we think the upper bound SNR should be 25dB, hence we propose MCS 25 for 256QAM/ 6 layers, MCS 26 for  64QAM/ 6 layers and MCS 23 for 1024QAM/ 2 layers

Proposal 3: Consider MCS 25 for 256QAM/ 6 layers, MCS 26 for  64QAM/ 6 layers and MCS 23 for 1024QAM/ 2 layers

CQI requirements for 6Rx

Based on our simulation results, we propose to use 5dB,6dB and 11dB,12dB.

Proposal 4: Use 5dB,6dB and 11dB,12dB for test SNR points for CQI tests.

- Conclusion

In this contribution we provide our views on open issues for 6Rx, the proposals are:

Proposal 1: Capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4

Proposal 2: Introduce applicability of requirements for different number of RX antenna ports listed in Table 2-1.

Proposal 3: Consider MCS 25 for 256QAM/ 6 layers, MCS 26 for  64QAM/ 6 layers and MCS 23 for 1024QAM/ 2 layers

Proposal 4: Use 5dB,6dB and 11dB,12dB for test SNR points for CQI tests.

- Reference

[1] Way Forward for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx. Huawei, HiSilicon

[2] WF on 6Rx requirements. AT&T