3GPP TSG-RAN WG4 Meeting # 117	R4-2521370

# Dallas, US, November 17th- 21st ,2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on 38.101-4 Introducing multiple PMI requirements with 64TX Enhanced TypeII Codebook (2Rx, FDD) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-10-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 shall introduce multiple PMI requirements with 64Tx Enhanced TypeII codebook for 2Rx FDD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce multiple PMI requirements with 64Tx Enhanced TypeII codebook for 2Rx FDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.12(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.3	Reporting of Precoding Matrix Indicator (PMI)

6.3.1	1RX requirements

6.3.2	2RX requirements

6.3.2.1	FDD

6.3.2.1.12	Multiple PMI with 64TX Enhanced Type II Codebook

For the parameters specified in Table 6.3.2.1.12-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.12-2.

Table 6.3.2.1.12-1: Test parameters (dual-layer)

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |
| Subcarrier spacing |  |  | kHz | 15 |
| Duplex Mode |  |  |  | FDD |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | XP Medium 64 x 2 (N1,N2) = (8,4) |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(9) |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | The number of CSI-RS ports |  |  | 64 |
|  | Number of aggregated NZP CSI-RS resources |  |  | 2 |
|  | portMappingMethod |  |  | ‘method1’ |
|  | Configuration for each NZP CSI-RS resource | CSI-RS resource Type |  | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  |  | First OFDM symbol in the PRB used for CSI-RS resource 1 (l0, l1) |  | Row 17, (1, 7) |
|  |  | First OFDM symbol in the PRB used for CSI-RS resource 2 (l0, l1) |  | Row 17, (5, 12) |
|  |  | CSI-RS periodicity and offset |  | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) |
|  | CSI-IM timeConfig interval and offset |  | slot | Not configured |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured |
| Sub-band Size |  |  | RB | 4 |
| csi-ReportingBand |  |  |  | 1111111111111 |
| CSI-Report interval and offset |  |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 5 |
| CSI request |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration |  | Codebook Type |  | eTypeII-r19 |
|  |  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  |  | R(numberOfPMI-SubbandsPerCQI-subband-r19) |  | 1 |
|  |  |  | N1,N2 value (n1-n2-typeII-r19) |  | (8,4) |
|  |  | (X1(c),X2(c)) configuration (valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) |  | Not configured |
|  |  |  | CBSR configuration (typeII-CBSR-r19) |  | Not configured |
|  |  |  | RI Restriction (typeII-RI-restriction-r19) |  | 0010 |
| Physical channel for CSI report |  |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 11 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| Measurement channel |  |  |  | [R.PDSCH.x-y.z] FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |

Table 6.3.2.1.12-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| [SNR(dB) corresponding to 90% of maximum throughput] | TBD |