3GPP TSG-RAN WG4 Meeting #117	R4-2521378

# Dallas, US, November 17th-21st, 2025

Title: 	Discussion on 6Rx demodulation requirements with interference

Source: 	Huawei, HiSilicon

Agenda item:	7.5.4

Document for:	Discussion

Background

A WF [1] for 6Rx demodulation was agreed. In this contribution we provide our views on open issues.

- Discussion

One issue is coverage of UE types, the open issue is captured as following:

Coverage of UE types - Observation: - It is clearly stated in the approved WID that the 6Rx requirements with interference will cover both FWA and handheld UEs. - Proposals: - Proposal 1: FFS whether to define one set or two sets of requirements for handheld and FWA UEs until there is clear agreement in Rel-19. - Proposal 2: Define only a single requirement for both handheld and FWA devices. - Proposal 3: Define separate requirements for handheld and FWA devices

In the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’, it has been agreed to define single set of requirement for both FWA and handheld UE and select α = 0 and β = 0.07179 for MIMO correlation matrix.  The agreements shall be applied for this WI.

Proposal 1: RAN4 to define single set of requirements for 6Rx requirements with interference.

Antenna correlation, Rank, MCS

One issue is antenna correlation, the proposals are:

- Candidate options: - Proposal 1: Follow the decision in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’. - Proposal 2: Evaluate the performance with α = 0 and β = 0.07179 for handheld UE and low correlation for FWA - Proposal 3: For initial simulation alignment, cover both ULA Low and ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx.

In the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’, it has been agreed to use α = 0 and β = 0.07179 for both FWA and handheld UE. We did some initial evaluation shown in figure 2-1. In the simulation, MCS 13 with TDLA30-10 was assumed, we compare the performance for low correlation with medium correlation with Beta=0.07179 under Rank 1,2 and 4:

Figure 2-1: Initial simulation results for HomNet

It can be observed that correlation with Beta=0.07179 exhibits reasonable SNR range for up to rank2.  Low correlation exhibits reasonable SNR range for up to Rank4. We propose RAN4 to select one case from following two cases:

TDLA30-10, Rank4, Low correlation, Rank4, MCS13

TDLA30-10, Rank2, correlation with Beta=0.07179, Rank2, MCS13.

Observation 1: Correlation with Beta=0.07179 exhibits reasonable SNR range for up to rank2.  Low correlation exhibits reasonable SNR range for up to Rank4.

Proposal 2: RAN4 to select one case from following two cases for inter cell interference:

TDLA30-10, Rank4, Low correlation, MCS13

TDLA30-10, Rank2, correlation with Alpha=0.0, Beta=0.07179, MCS13.

MU-MIMO

We evaluate the performance for Rank 2+2, MCS13, TDLA30-10, which is shown in Figure 2-2:

Figure 2-2: Simulation results for MU-MIMO, Rank 2+2

It is shown that correlation with Beta=0.07179 exhibits reasonable operating SNR. Hence, we propose the following:

Proposal 3: RAN4 to consider following test setup for MU-MIMO 6Rx requirements:

Rank 2+2

MC13

TDLA30-10

Alpha=0.0, Beta=0.07179

Whether to introduce CQI reporting requirements with inter-cell interference

One issue is whether to introduce CQI reporting requirements with inter cell interference:

- Candidate options: - Option 1: No matter how RAN4 will design the CQI reporting tests for basic 6Rx, there is necessity to define CQI reporting requirements with inter-cell interference. - Option 2: Don’t define CQI reporting requirements with inter-cell interference for 6Rx

We don’t see the need to introduce such requirements since the combination gain is quite limited when number of Rx increases from 4 to 6. Considering high antenna correlation at UE side for 6Rx, the degree of freedom for interference mitigation is also limited. Furthermore, RAN4 didn’t introduce CQI requirements under fading channel without interference, we don’t think RAN4 should jump to more advanced requirements without basic requirements defined.

Proposal 4: Not to introduce CQI requirements with inter cell interference.

- Conclusion

In this contribution we provide our views on open issues for 6Rx, the proposals and observations are:

Observation 1: Correlation with Beta=0.07179 exhibits reasonable SNR range for up to rank2.  Low correlation exhibits reasonable SNR range for up to Rank4.

Proposal 2: RAN4 to select one case from following two cases for inter cell interference:

TDLA30-10, Rank4, Low correlation, MCS13

TDLA30-10, Rank2, correlation with Alpha=0.0, Beta=0.07179, MCS13.

Proposal 3: RAN4 to consider following test setup for MU-MIMO 6Rx requirements:

Rank 2+2

MC13

TDLA30-10

Alpha=0.0, Beta=0.07179

Proposal 4: Not to introduce CQI requirements with inter cell interference.

- Reference

[1]  WF on NR_demod_Ph6_Ph5,  China Telecom