# 3GPP TSG-RAN WG4 Meeting #117	R4-2521419

# Dallas, USA, Nov. 17-21, 2025

Title:	Discussion on co-existence for 6GR

Source:	Samsung

Agenda item:	8.4.3

Document for:	Approval

Introduction

In last RAN4#116bis meeting, RAN4 had initially discussed the contributions for 6GR BS RF requirements and co-existence from multiple companies. The meeting had agreed on some high-level guidance on some aspects and documented in [1], but the detailed proposals from each company still require more discussion. In this paper, we will continue to provide our views on the co-existence aspects for 6GR.

Discussion

In #116bis meeting, in our paper [2], it discussed considerations for 6G radio co-existence studies, highlighted significant gaps between previous RAN4 studies and current 6G deployment scenarios. Key observations include differences in carrier frequencies, deployment scenarios, BS antenna assumptions, and the need for new co-existence studies to address these gaps, particularly for the 7GHz carrier frequency. Proposals include conducting new co-existence studies, starting with 7GHz, reviewing antenna assumptions, and adopting sub-array based AAS models for 6G base stations.

  - General review

In this paper, we will skip all those tables provided in [2] with detailed comparison between existing co-ex studies carried out in RAN4 and the potential deployment and typical antenna array adopted in 6G to not duplicate work. From the tables in [2], we have made the following observations:

Observation 1: The gaps between co-ex studies performed in RAN4 and 6GR target deployment scenarios in different carrier frequencies are quite large, and they are shown as below.

Observation 2: The previously available co-existence studies on 700MHz and 2GHz carrier frequencies are majorly from LTE study, which differs significantly with later 5G NR and the current 6G Radio in both system parameters and deployment scenarios.

Observation 3: The previously available co-existence studies on 4GHz and 7GHz carrier frequencies assumed much less antenna array elements and only partial of the deployment scenarios as given in latest 6G deployment discussion.

Observation 4: The previously available co-existence studies on 15GHz and 30GHz carrier frequencies also assumed much less antenna array elements, and different scenario (UMa) other than scenarios targeted in latest 6G deployment discussion.

In general for 6G radio, the previous co-ex studies performed in RAN4 can only provide very limited reference. And with the development of new scenarios for different carrier frequencies, larger antenna array size for 6G, different conducted power per sub-array/elements, different ISD/topology to each scenarios considered, the best way to understand the RF requirements to ensure the co-existence in adjacent channel is to perform new co-existence studies to these carrier frequencies.

Among all the candidate frequencies in [3], in our view, we can pick 7GHz with higher priority, given it has most major changes compared to previous studies in both deployment scenario and BS antenna assumption aspects.

Proposal 1: It is proposed to perform new co-existence studies to the carrier frequencies suggested for 6G Radio.

Proposal 2: The new 6G Radio co-existence study can start with 7GHz to re-evaluate ACLR and ACS given its significant difference to previous studies in both deployment scenario and BS antenna assumption aspects.

  - BS antenna array and modelling

In our last paper [2], we had performed a numerical simulation over the differences of BS antenna assumption between TR 38.921 and TR 38.922 for around 7GHz carrier. The observations were made as follows:

Observatoin 5: For 7GHz, TR 38.921 studied co-ex with 8x16x2 array size for Urban Macro, TR 38.922 concluded (8x16)x(3x1)x2 array size for Urban Macro, while latest 6G deployment discussion suggested ”up to 2304” elements.

R4-2513045 [2] The figures below depicted a difference between TR 38.921 and TR 38.922 assumptions in 7GHz BS antenna. The difference will be significant larger if we consider larger array as 2304 elements. Figure 2.2-1 Differences in antenna pattern between TR 38.921 and TR 38.922 for 7GHz Left: Horizontal domain; Right: Vertical domain

The simulated antenna pattern from TR 38.921 and TR 38.922 simply reflects the difference in antenna gain in different directions of both horizontal and vertical domain, it does not consider the difference introduced by different Tx power. And yet, with a (3x1) sub-array assumed in TR 38.922, its side lobes in vertical domain already shows better suppression compared to TR 38.921. With larger array size as documented in [3] in 7GHz, we are expecting larger difference in antenna pattern in new co-existence study.

Observation 6: With more elements considered, for example 3x1 sub-array in a same 8x16 antenna, the difference in antenna pattern is already significantly large. The 2304 elements proposed in latest 6G deployment discussion would introduce greater impacts in antenna pattern in new co-existence studies alone.

Proposal 3: RAN4 to review and discuss antenna assumptions in TR 38.921, 38.922 and RP-252888 for 7GHz, and to perform new co-existence study with the agreed typical antenna assumption for 7GHz.

In addition, in the Figure 2.2-1, the pattern from TR 38.922 considered a sub-array modelling, which was not usually considered in previous co-existence studies as listed in [2] in RAN4. With more and more antenna elements considered as expressed in [3], and a broader adoption of having sub-array assumed in the BS AAS antenna model in TR 38.922, we tend to suggest RAN4 to use sub-array based AAS model as a baseline assumption at least for 6G base station in 6G Radio co-existence study.

Proposal 4: In 6G Radio co-existence study, use sub-array based AAS model as a baseline assumption for 6G base stations.

  - Deployment scenario

In our last paper [2], we had listed the differences in deployment parameters between the legacy RAN4 co-ex studies and the on-going 6G discussions. And then the following observations had been made:

Observation 7: The previous co-existence study in TR 38.921 does not study Urban micro, Rural scenarios for 7GHz carrier, which is listed in latest 6G deployment scenario discussion.

Observation 8: The previous assumptions for Urban macro in TR 38.921 assumed one layer of hexagonal grid macro stations, while 6G deployment scenario discussion suggested both one layer and two layer layouts. But two layers in latest 6G deployment discussion assumed different carriers in different layers.

Observation 9: The previous assumption for sub-urban macro ISD is 900m, which is much smaller than the assumed 1299 or 1732 meters ISD as discussed in latest 6G deployment scenario discussion.

From the above observations, together with the difference from BS parameters and antennas discussed in previous sections, the adjacent channel interference would be very much different under new 6G Radio co-existence study.

Proposal 5: RAN4 to consider the deployment related agreements (e.g. ISD, layout) from 6G deployment scenario discussion as the new 6G Radio co-existence study assumptions.

Moreover, the channel model have been further developed and updated in the Rel-19 study item “Study on channel modelling enhancements for 7-24GHz for NR”. In our view, the major changes to RAN4 co-ex study would be the introduction of Sub-urban scenario, which is also aligned with the 6G deployment discussions.

TR 38.901 SMa In suburban macro-cells base stations are located above the surrounding environment to allow wide area coverage, and mobile stations are outdoors at street level and within commercial and residential buildings. Buildings are typically low residential detached houses with one or two floors, or blocks of apartments/condos or commercial buildings with a few floors. Occasional open areas such as parks or playgrounds between the houses make the environment rather open. Streets do not form urban-like regular strict grid structure. In suburban areas, vegetation is more prevalent than in urban areas with a high variability of foliage density across different suburban areas. Details on SMa scenarios are listed in Table 7.2-5.

The Sub-urban macro (SMa) related updates in TR 38.901 includes new assumptions of base station height, UE dropping and new channel models dedicated for SMa including probability and models of LoS and non-LoS models, etc. And the related contents can be found in section 7.2, 7.3 and 7.3 in TR 38.901.

Observation 10: In updated TR 38.901 from Rel-19 7-24GHz channel model study, the sub-urban macro (SMa) was introduced, and aligned with the 6G deployment discussion.

Proposal 6: To adopt the updated channel model in TR 38.901, especially for SMa scenario, to the new 6G Radio co-existence study in applicable example carrier.

  - System level simulation assumptions for 7GHz

The antenna technology has developed significantly so far. In particular, BSs that can achieve further beamforming gain by increasing the number of AEs (antenna elements) have been commercialized. However, for practical handheld devices, there are constraints on antenna placement due to limited size of form-factor which is caused by portability and weight. Nevertheless, antenna placement technology has advanced, and commercial handheld devices which enable to receive up to 4 layers for DL are widely used. On the other hand, while high-end handheld devices now support up to 2 TX, 1Tx handhelds which are equipped with one high-power PA remain practical. Therefore, 1TX should be a baseline assumption for handheld devices.

Accurate performance evaluation of 6GR necessitates clearly defined antenna assumptions to ensure meaningful comparisons across frequency bands, particularly when assessing coverage equivalence between the mid-band (e.g., 3.5 GHz) and 7~24GHz spectrum (as mentioned in SID). Baseline evaluations should consider a BS architecture that selects both the number of horizontal and vertical antenna element, alongside sub-array structures defining antenna port configurations – representative examples include 192 elements for 4 GHz and 768 elements for 7 GHz scenarios for the total number of antenna elements. With the increased number of antenna elements for 7 GHz scenarios, more antenna ports, i.e., 256 enabling more precoding gain as well as the same number of antenna ports as NR Rel-19, i.e. 128 can be supported. Therefore, the antenna configurations for BS and UE in Table 4.2-1 can be supported as a baseline for antenna models in 7 GHz.

Proposal 7: To consider the Table 2.4-1 below as starting point for system level simulation for 7GHz.

Table 2.4-1: System-level assumptions related to around 7GHz evaluation

|  | UMa | InH | RMa |
| --- | --- | --- | --- |
| Deployment | 21 cells (7 sites w/ 3 sectors) | 12 cells (12 sites w/ 1 sector) | 21 cells (7 sites w/ 3 sectors) |
| ISD | 200 | 20 | 1732 |
| Carrier freq. [GHz] | 7 | 7 | 7 |
| BW [MHz] | [20, 100, 200] | [20, 100, 200] | [20, 100, 200] |
| SCS [kHz] | 30 kHz | 30 kHz | 30 kHz |
| Indoor ratio | 20 | 100 | 50 |
| Probability of Low loss model | 20% high loss, 80% low loss | - | 100% low loss |
| BS height [m] | 25 | 3 | 35 |
| TRP (dBm) | 44 dBm for 20 MHz 51 dBm for 100 MHz 54 dBm for 200 MHz | 24 dBm for 20 MHz 31 dBm for 100 MHz 34 dBm for 200 MHz | 49 dBm for 20 MHz 56 dBm for 100 MHz 59 dBm for 200 MHz |
| UE power class | 1) 23 dBm; 2) 26 dBm | 1) 23 dBm; 2) 26 dBm | 1) 23 dBm; 2) 26 dBm |
| BS Element gain [dBi] | 8 | 5 | 8 |
| BS noise figure [dB] | 5 | 5 | 5 |
| UE Element gain [dBi] | 0 (Isotropic antenna) |  |  |
| UE noise figure [dB] | 7 | 7 | 7 |
| BS Antenna Configuration | 1) (M,N,P,Mp,Np) = (24,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.8)λ 2) (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ | 1) (M,N,P,Mp,Np) = (8,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.5)λ 2) (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ | 1) (M,N,P,Mp,Np) = (24,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.8)λ 2) (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ |
| UE Antenna Configuration | 1T4R: (M,N,P,Mp,Np) = (1,2,2,1,2); 4 ports, (dH,dV) = (0.5,0.5)λ |  |  |
| Mechanical tilt | 12 deg | 90 deg | 1.5 deg |
| Traffic Model | Full buffer |  |  |

Conclusion

In this contribution, we proposed following for the 6G radio co-existence:

Observation 1: The gaps between co-ex studies performed in RAN4 and 6GR target deployment scenarios in different carrier frequencies are quite large, and they are shown as below.

Observation 2: The previously available co-existence studies on 700MHz and 2GHz carrier frequencies are majorly from LTE study, which differs significantly with later 5G NR and the current 6G Radio in both system parameters and deployment scenarios.

Observation 3: The previously available co-existence studies on 4GHz and 7GHz carrier frequencies assumed much less antenna array elements and only partial of the deployment scenarios as given in latest 6G deployment discussion.

Observation 4: The previously available co-existence studies on 15GHz and 30GHz carrier frequencies also assumed much less antenna array elements, and different scenario (UMa) other than scenarios targeted in latest 6G deployment discussion.

Proposal 1: It is proposed to perform new co-existence studies to the carrier frequencies suggested for 6G Radio.

Proposal 2: The new 6G Radio co-existence study can start with 7GHz to re-evaluate ACLR and ACS given its significant difference to previous studies in both deployment scenario and BS antenna assumption aspects.

Observatoin 5: For 7GHz, TR 38.921 studied co-ex with 8x16x2 array size for Urban Macro, TR 38.922 concluded (8x16)x(3x1)x2 array size for Urban Macro, while latest 6G deployment discussion suggested ”up to 2304” elements.

Observation 6: With more elements considered, for example 3x1 sub-array in a same 8x16 antenna, the difference in antenna pattern is already significantly large. The 2304 elements proposed in latest 6G deployment discussion would introduce greater impacts in antenna pattern in new co-existence studies alone.

Proposal 3: RAN4 to review and discuss antenna assumptions in TR 38.921, 38.922 and RP-252888 for 7GHz, and to perform new co-existence study with the agreed typical antenna assumption for 7GHz.

Proposal 4: In 6G Radio co-existence study, use sub-array based AAS model as a baseline assumption for 6G base stations.

Observation 7: The previous co-existence study in TR 38.921 does not study Urban micro, Rural scenarios for 7GHz carrier, which is listed in latest 6G deployment scenario discussion.

Observation 8: The previous assumptions for Urban macro in TR 38.921 assumed one layer of hexagonal grid macro stations, while 6G deployment scenario discussion suggested both one layer and two layer layouts. But two layers in latest 6G deployment discussion assumed different carriers in different layers.

Observation 9: The previous assumption for sub-urban macro ISD is 900m, which is much smaller than the assumed 1299 or 1732 meters ISD as discussed in latest 6G deployment scenario discussion.

Proposal 5: RAN4 to consider the deployment related agreements (e.g. ISD, layout) from 6G deployment scenario discussion as the new 6G Radio co-existence study assumptions.

Observation 10: In updated TR 38.901 from Rel-19 7-24GHz channel model study, the sub-urban macro (SMa) was introduced, and aligned with the 6G deployment discussion.

Proposal 6: To adopt the updated channel model in TR 38.901, especially for SMa scenario, to the new 6G Radio co-existence study in applicable example carrier.

Proposal 7: To consider the Table 2.4-1 below as starting point for system level simulation for 7GHz.

Table 2.4-1: System-level assumptions related to around 7GHz evaluation

|  | UMa | InH | RMa |
| --- | --- | --- | --- |
| Deployment | 21 cells (7 sites w/ 3 sectors) | 12 cells (12 sites w/ 1 sector) | 21 cells (7 sites w/ 3 sectors) |
| ISD | 200 | 20 | 1732 |
| Carrier freq. [GHz] | 7 | 7 | 7 |
| BW [MHz] | [20, 100, 200] | [20, 100, 200] | [20, 100, 200] |
| SCS [kHz] | 30 kHz | 30 kHz | 30 kHz |
| Indoor ratio | 20 and 80 | 100 | 50 |
| Probability of Low loss model | 20% high loss, 80% low loss | - | 100% low loss |
| BS height [m] | 25 | 3 | 35 |
| TRP (dBm) | 44 dBm for 20 MHz 51 dBm for 100 MHz 54 dBm for 200 MHz | 24 dBm for 20 MHz 31 dBm for 100 MHz 34 dBm for 200 MHz | 49 dBm for 20 MHz 56 dBm for 100 MHz 59 dBm for 200 MHz |
| UE power class | 1) 23 dBm; 2) 26 dBm | 1) 23 dBm; 2) 26 dBm | 1) 23 dBm; 2) 26 dBm |
| BS Element gain [dBi] | 8 | 5 | 8 |
| BS noise figure [dB] | 5 | 5 | 5 |
| UE Element gain [dBi] | 0 (Isotropic antenna) |  |  |
| UE noise figure [dB] | 7 | 7 | 7 |
| BS Antenna Configuration | 1) (M,N,P,Mp,Np) = (24,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.8)λ 2) (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ | 1) (M,N,P,Mp,Np) = (8,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.5)λ 2) (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ | 1) (M,N,P,Mp,Np) = (24,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.8)λ 2) (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ |
| UE Antenna Configuration | 1T4R: (M,N,P,Mp,Np) = (1,2,2,1,2); 4 ports, (dH,dV) = (0.5,0.5)λ |  |  |
| Mechanical tilt | 12 deg | 90 deg | 1.5 deg |
| Traffic Model | Full buffer |  |  |

Reference

[1] R4-2514638, WF on 6G BS RF and co-existence, Feature Lead (Ericsson)

[2] R4-2513045, Discussion on BS RF and co-existence for 6GR, Samsung

[3] RP-252888, pCR on Deployment scenarios for TR 38.914, Deutsche Telekom