3GPP TSG RAN WG4 Meeting #117	R4-2521445

Dallas, USA, Nov. 17-21, 2025

Agenda item:			7.12.4

Source:	vivo

Title:	Genernal discussion on RRM impact on measurement event prediction

Document for:	Discussion

- Introduction

In RAN#109 meeting, the latest WID [1] was approved. The description of objectives is copied as:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

Among them, the RAN4 task specifies the highlighted sections above, primarily covering both RRM measurement prediction and measurement event prediction for the UE-sided model.

The RAN4 task specifies the highlighted sections above, primarily covering both RRM measurement prediction and measurement event prediction for the UE-sided model.

In the last meeting, RAN4 conducted a preliminary discussion on issues related to scenarios and sub-use cases, as well as simulation scenarios and assumptions. The outcome of the discussion were captured in WF [2]. In this contribution, we further provide our views on issues regarding RRM requirements for measurement event prediction for AI mobility based on the summary reached in the last meeting[2].

- Discussion

According to the WID description, RAN4 will focus only on discussions related to indirect event prediction until Q2 2026. For reference, the following are the conclusions reached by RAN4 during the SI phase.

#### 6.2.2.2 Potential RRM requirements For indirect event prediction, the requirements for the predicted event triggered reporting including, but not limited to, delay and accuracy requirements and the performance metrics are considered. If the report includes the predicted RSRPs corresponding to the predicted event occurrence time, the absolute and/or relative accuracy requirement for the predicted RSRP will be defined.

  - Core part

As mentioned in the WID, there is a relevant Note regarding the core part, which states that RAN4 needs to study whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction. Regarding this aspect, we believe it is necessary to wait until RAN4 reaches a preliminary conclusion on the core requirement for RRM measurement prediction before discussing whether it can be applied to indirect measurement event prediction, or if any additional requirements need to be defined specifically for indirect measurement event prediction.

Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction

Proposal 1: On whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction or if any additional requirements need to be defined specifically, it can be discussed after RAN4 reaches a preliminary conclusion on the core requirement for RRM measurement prediction

  - KPI/metric related

For the KPI part, according to the green text in the above RAN4 conclusions, RAN4 has agreed that the absolute and/or relative accuracy requirement for the predicted RSRP will be defined.

Regarding the metrics related to the prediction of measurement events themselves, during the SI phase, companies provided some analyses. These included the following metrics:

- How early can the UE predict the occurrence of the measurement time

This metric is defined as the time gap between the report time of the predicted event and the actual occurrence time of the ground truth event. It primarily measures whether the UE can predict the event in advance and how early it can do so. We understand the motivation behind this metric, but the challenge lies in determining whether a larger time gap is better or whether it should fall within a specific range to meet requirements. Defining an appropriate value for this metric may be difficult.

- Event prediction accuracy

For the accuracy of measurement event prediction, the following describes the methods and metrics previously used by RAN2 during the evaluation phase.

### 5.3.1 Evaluation methodology, metrics and assumptions The performance metric F1 score is defined as following: F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3) For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: - Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa; - Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD]; - Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD]. The ETD, i.e. timing difference between ground-truth event and predicted event, is illustrated in Figure 5.3.1-1: Figure 5.3.1-1: illustration of ETD As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected.

We believe this metric is similar to the F1 score currently considered in RAN2 simulation evaluations, which verifies whether the timing difference between the ground-truth event and the predicted event falls within a certain range. RAN4 can use this as a starting point for further discussion.

Possible details include: The F1 score defined by RAN2 treats true event prediction, false event detection, and missed event detection as equally important and incorporates them into the calculation of the F1 score. When defining the metric for RAN4, considerations may include whether all these detection types need to be accounted for and their respective weights. Additionally, the current ETD only requires the timing difference between the ground-truth event and the predicted event to be within a certain range, without distinguishing whether the predicted event occurs before or after the ground-truth event. RAN4 may need to consider whether only predicted events occurring before the ground-truth event should be regarded as correct predictions.

Proposal 2: RAN4 considers metrics related to the prediction of the measurement event itself. Specifically, RAN4 can use the performance metric in RAN2 simulations as a starting point and further discuss the definition of the metric. Possible directions include:

- For true event prediction, false event detection and missed event detection defined in F1 score, whether to consider them in KPI definition and their corresponding weights.

- Whether to only consider predicted events occurring before the true event as correct predictions, rather than following the ETD definition where correctness is based on the timing difference between the ground-truth event and the predicted event falling within a certain range.

Additionally, for performance monitoring of measurement event prediction, the definition of metrics also falls within the scope of RAN4 discussions. Therefore, RAN4 also needs to discuss whether and how to define relevant metrics for performance monitoring, which may depend on further discussions in RAN2 regarding performance monitoring mechanisms.

Proposal 3: RAN4 to discuss whether and how to define relevant metrics for performance monitoring, which may depend on further discussions in RAN2 regarding performance monitoring mechanisms.

- Summary

Proposal 1: On whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction or if any additional requirements need to be defined specifically, it can be discussed after RAN4 reaches a preliminary conclusion on the core requirement for RRM measurement prediction

Proposal 2: RAN4 considers metrics related to the prediction of the measurement event itself. Specifically, RAN4 can use the performance metric in RAN2 simulations as a starting point and further discuss the definition of the metric. Possible directions include:

- For true event prediction, false event detection and missed event detection defined in F1 score, whether to consider them in KPI definition and their corresponding weights.

- Whether to only consider predicted events occurring before the true event as correct predictions, rather than following the ETD definition where correctness is based on the timing difference between the ground-truth event and the predicted event falling within a certain range.

Proposal 3: RAN4 to discuss whether and how to define relevant metrics for performance monitoring, which may depend on further discussions in RAN2 regarding performance monitoring mechanisms.

- References

- RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital

- R4-2514827, WF on NR_AIML_Mob_RRM, OPPO