3GPP TSG-RAN WG4 Meeting #117            	R4-2521490

Dallas, USA, 17th-21th, Nov, 2025

Agenda item:	6.11.3

Source:	Samsung

Title:	View on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

- Introduction

In the last RAN4#116bis meeting, CSI reporting requirement framework for CSI prediction was under discussed. The related agreed agreements are captured in the WF [1] as following

- CSI reporting requirement framework for CSI prediction - No performance monitoring delay requirements are specified in RAN4 for CSI prediction - CSI requirements for PMI prediction reporting - Introduce a new clause under Clause 6 (6.X) in 38.101-4 as “Reporting of predicted PMI” - Doppler and MCS - Introduce tests for 20Hz Doppler with MCS [17or19] with 16Tx ports and 2Rx in FDD - Introduce test for TDD with the details FFS - Generalization - Test setup for generalization will be further discussed based on the following options - Option 1: Introduce multiple tests with different MCS, which will lead to different SNR points - Option 2: Introduce multiple tests with different channel models. The details of the channel models are FFS. - Option 3: The combination of option 1 and 2 - It is FFS on how to quantify the generalization - Activation - No activation requirement in RAN4 spec for CSI prediction will be introduced - Note: it is a general understanding that UE has to be ready to start measurements for inference after sending RRC reconfiguration complete. No details will be specified in RAN4

For Generalization, the following is agreed in previous meeting as

- SNR levels for training data and generalization - No need to capture anything explicit in RAN4 about training data in the spec, - For simulation calibration, SNR range alignment for training can be discussed separately - UE performance at more than one MCS should be tested - Model generalization can be discussed separately - Each individual test is expected to involve in single MCS

In this contribution, the view on the remain issue of CSI reporting requirement and testing framework for CSI prediction is provided.

# 2	LCM related requirement

Reporting accuracy for performance monitoring

According to RAN1 agreement, two SGCS are included to reflect the Quantity reported, including the SGSC of predicted CSI and ground truth CSI, and the SGCS of non-predicted CSI and ground truth CSI.

Based on RAN1 agreement, with type 3 monitoring, UE will report the performance metric to network, based on the configured resources for monitor and report, as well the prediction instance. For performance monitoring metric report, both semi-persistent and aperiodic can be supported. For report value, the SGCS will be used and is quantized with 4-bit. For each report, two SGCS are included to reflect the Quantity reported, including the SGSC of predicted CSI and ground truth CSI, and the SGCS of non-predicted CSI and ground truth CSI. Each calculated SGCS value is quantized to a 4-bit field, where the first codepoint ‘0000’ is used to indicate (0 0.3] value range, and the value range [0.3, 1] is quantized in a linear scale with 0.05 step size. Detailed mapping is specified in following table

Table 1: SGCS Quantization Mapping

| Code Point | Range |
| --- | --- |
| 0 | [0.0000, 0.3000] |
| 1 | (0.3000, 0.3500] |
| 2 | (0.3500, 0.4000] |
| 3 | (0.4000, 0.4500] |
| 4 | (0.4500, 0.5000] |
| 5 | (0.5000, 0.5500] |
| 6 | (0.5500, 0.6000] |
| 7 | (0.6000, 0.6500] |
| 8 | (0.6500, 0.7000] |
| 9 | (0.7000, 0.7500] |
| 10 | (0.7500, 0.8000] |
| 11 | (0.8000, 0.8500] |
| 12 | (0.8500, 0.9000] |
| 13 | (0.9000, 0.9500] |
| 14 | (0.9500, 1.0000] |
| 15 | Reserved |

Observation 1:  For performance monitoring metric report, SGCS will be used and is quantized with 4-bit. For each report, two SGCS are included to reflect the Quantity reported.

The measurement accuracy requirements specified is to verify the measurement errors from the true value. For SGCS reporting, the predicted CSI, non-predicted CSI and ground truth CSI is measured and calculated by UE, TE has no information about the absolute value of predicted CSI, non-predicted CSI and ground truth CSI. Therefore, it is difficult to verify the measurement accuracy of SGCS1 and SGCS2 in the TE side.

Then, how to test in RAN4 specification should be further discussed based on the reported SGCS.

For legacy PMI accuracy test, RAN4 takes the throughput ratio of achieved by following PMI and random PMI, where the requirement is based on 90% of the maximum throughput obtained by following PMI.

For reported CSI, since the SGCS with quantized will be reported for AI based predicted CSI and non-predicted CSI, respectively, one possible of test metric is to use the ratio of reported SGCS1 and reported SGCS2. During the whole testing time, TE can check whether it is higher than the pre-defined threshold Y with X% of test time at current test SNR. At same time, TE can check throughput ratio of predicted PMI with random type I PMI together to provide the information on the CSI prediction performance

Proposal 1:  RAN4 could consider to define the report accuracy requirement based on test metric of ratio of reported SGCS1 and SGCS2, TE can check whether the ratio is higher than pre-defined threshold Y with X% of test time at current test SNR. At same time, TE can check the throughput ratio of predicted PMI with random type 1 PMI to provide the information on CSI prediction performance at current test SNR.

# 3	Generalization

For generalization testing, the purpose is to guarantee the AI model training on specific scenario/condition can still work well in other scenario/condition. Based on RAN4 meeting, multiple test cases with different configurations are expected to be introduced.

As the configurations for generalization, at least for difference MCS and difference channel models will be considered. For different MCS, it is related with different SNR. To ensure the generalization of SNR, RAN4 can define requirement with different MCS, targeting for different SNR range. Since the purpose of AI/ML based CSI prediction is to track the channel correlation property and prediction the channel information in the future slots instead of denoising, the specific SNR range associated with MCS for generalization test should be further discussed. High MCS is preferred to be considered for robust testing SNR.

For different channel model, the main purpose of CSI prediction is to track the channel variation and correlation in the time domain considering the doppler impact. RAN4 have evaluated the performance with various UE speeds (20Hz, 50Hz, 100Hz). The gain of AI based CSI prediction is minor under high doppler scenario. Therefore, 20HZ will be considered for requirement. To verify the generalization, different channel condition can be considered. In current legacy test, other channel models are also considered for requirement, such as TDLC300/TDLB100. To check the robustness, our preference is to consider TDLC300-20.

To minimize the test effort, we would like to test the generalization with different MCS and channel model combination. Based on existing evaluation for FDD scenario, TDLA30-20 with MCS 17 or MCS 19 will be considered. Therefore, we can consider two cases, one is for TDLA30-20 with MCS 19, another is for TDLC300-20 with MCS 17 for FDD scenario.

For TDD scenario, the current RAN4 evaluation mainly focuses on the FDD for AI based CSI prediction, the feasible test condition needed to be investigated.

Proposal 1:  RAN4 can introduce the generation test with different MCS and channel model combination

For FDD

- TDLA30-20 with MCS 19

- TDLC300-20 with MCS 17

For TDD scenario, further discuss pending on feasibility evaluation

FFS on how to quantify the generalization

As agreed, the requirements will be defined for PMI reporting requirement based on AI based CSI prediction as following

Issue 1-4: Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML with random precoding. - trnd is obtained with Rel-15 type I single panel codebook Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

Since different MCS with different channel models will be involved into the individual testing for generalization testing, the expected Throughput ratio is different for individual tests. Therefore, we think it is more proper to define the requirement based on the TP ratio for each case to quantify the generalization performance

Proposal 3:  RAN4 can still use the TP ratio as test metric for individual case testing to quantify the generalization performance

# 4	Conclusion

In this contribution, the view on the remaining issue of CSI reporting requirement and testing framework for CSI prediction was provided with following observations and proposals

Reporting accuracy for performance monitoring

Observation 1:  For performance monitoring metric report, SGCS will be used and is quantized with 4-bit. For each report, two SGCS are included to reflect the Quantity reported.

Proposal 1:  RAN4 could consider to define the report accuracy requirement based on test metric of ratio of reported SGCS1 and SGCS2, TE can check whether the ratio is higher than pre-defined threshold Y with X% of test time at current test SNR. At same time, TE can check the throughput ratio of predicted PMI with random type 1 PMI to provide the information on CSI prediction performance at current test SNR.

Generalization

Proposal 2:  RAN4 can introduce the generation test with different MCS and channel model combination

For FDD

- TDLA30-20 with MCS 19

- TDLC300-20 with MCS 17

For TDD scenario, further discuss pending on feasibility evaluation

Proposal 3:  RAN4 can still use the TP ratio as test metric for individual case testing to quantify the generalization performance

# References

R4-2514631, WF on AI/ML for NR air interface1