3GPP TSG-RAN WG4 Meeting #117		R4-2521508
Dallas, USA, Nov. 17-21, 2025

Title:	R17 FDD duty cycle

Source:	OPPO

Agenda item:	10.2.2

Document for:	Approval

Introduction

In the past RAN4 meeting there was discussion on whether the FDD HPUE shall be limited by the duty cycle due to SAR or thermal issue. And there is no conclusion on this. This paper share our view on this.

Discussion

SAR limitation

As captured in TR38.861, the duty cycle based SAR solution was discussed but no conclusion as can be seen below. But unfortunately, when the FDD HPUE was introduced into spec, the duty cycle was mistakenly extended to FDD bands in 38.101-1. This makes the spec is broken between what is written in the spec and what is agreed. This error shall be corrected.

There is no conclusion in introducing duty cycle based SAR solution to FDD bands, i.e., FDD HPUE shall not be limited by duty cycle due to SAR issue.

It is an error in the spec that FDD HPUE is limited by duty cycle.

Correct the error of duty cycle limitation to FDD HPUE in 38.101-1.

It was well recognized that duty cycle based SAR solution has never been implemented by UE up to now. This correct should be ok from the earliest release, i.e., Rel-17.

This error correction can be made from Rel-17 considering duty cycle based SAR solution has never been implemented by UE up to now.

Thermal issue

Thermal issue was also discussed a bit in previous meetings, and it seems this is also one of the reason some companies want to limit the FDD HPUE duty cycle. However, in our view, this should be discussed separately from above SAR issue and related error correction.

Thermal related duty cycle restriction should be discussed separately from SAR related duty cycle error correction.

Conclusion

This paper shares our view on the FDD duty cycle and error correction issue.

Observation 1: 	There is no conclusion in introducing duty cycle based SAR solution to FDD bands, i.e., FDD HPUE shall not be limited by duty cycle due to SAR issue.

Observation 2: 	It is an error in the spec that FDD HPUE is limited by duty cycle.

Proposal 1: 	Correct the error of duty cycle limitation to FDD HPUE in 38.101-1.

Proposal 2: 	This error correction can be made from Rel-17 considering duty cycle based SAR solution has never been implemented by UE up to now.

Proposal 3: 	Thermal related duty cycle restriction should be discussed separately from SAR related duty cycle error correction.