3GPP TSG-RAN WG4 Meeting #117	R4-2521529

Dallas, USA, November 17th – 21st, 2025

Source:	Nokia

Title:	Discussion on Test Decoder Specification

Agenda item:	7.9.3

Document for:	Discussion

Introduction

In the RAN4#116bis meeting, the following recommendations were captured in the WF for topics to discuss in RAN4#117. In this contribution, we share our views on the assumptions for dataset creation, assumptions for test decoder and reference encoder models, and means to capture models and datasets in RAN4 specifications.

Recommendations for RAN4#117 The following are not agreements, but are recommendations for contributions and discussions at RAN4#117: - Proposals for assumptions for the dataset including how to cover scalability, channel models, number of samples - Proposals for how to evaluate the dataset including potential metrics to compare different dataset contributions - Proposals on model hyperparameters, considering in particular complexity, considering also quantization size - Proposals and reasoning for scalability solutions - Proposals and reasoning for training parameters - Proposals for performance metrics for evaluating the different potential model structures - Proposals for how to capture the model and the dataset in 3GPP (format, how to store etc.)

Discussion

Assumptions for dataset creation

As stated below, RAN4 agreed in the last meeting to introduce a new mixed dataset in the WI.

Dataset for model evaluation RAN4 will discuss how to introduce the new mixed dataset for model structure evaluation purpose at the beginning of this WI, by taking into consideration of the scalability and/or various configurations. - Details are FFS including the channel model(s).

Steps to create the new mixed dataset were also agreed in RAN4#116bis, as shown below.

Steps to develop the test decoder and reference encoder: - Step 1: Create dataset - Step 1-1: Align on configurations for mixed dataset construction, taking factors such as scalability over Tx port number and subband size and payload size into account - Step 1-2 Decide how align and on which metrics can be used for evaluating dataset alignment, if any - Step 1-3 Companies create datasets - Step 1-4 Identify which company datasets to include in the mixed dataset(s) - Step 1-5 Create new dataset(s)

Simulations for dataset creation

There are three options for the simulations to be used for dataset creation: (1) SLS samples only, (2) LLS samples only, or (3) both SLS and LLS samples. We believe that the second option (i.e., using LLS only for generating the dataset) may be acceptable for RAN4 testing purposes, but may not be the optimal strategy, especially if the expectation is that RAN1 may reuse the models trained on this dataset for inter-vendor training collaboration. The first option is the one adopted by RAN4 during the SI, while the third option offers a middle ground for both purposes. Hence, we propose to construct the dataset using samples from link-level and system-level simulations.

Using only LLS for dataset creation may be acceptable for RAN4 testing purposes, but may not be the optimal strategy, especially if the expectation is that RAN1 may reuse the models trained on this dataset for inter-vendor training collaboration.

RAN4 to consider using samples from both link-level and system-level simulations to construct the mixed dataset.

We also believe that generalization over LLS samples should be verified in RAN4 simulations in both cases where the training dataset is created using SLS samples only or a combination of LLS and SLS samples. This is needed to make sure that the pre-trained decoder model can work well for testing purposes.

Generalization over LLS samples should be verified in RAN4 simulations in both cases where the training dataset is created using SLS samples only or a combination of LLS and SLS samples.

Configurations for mixed dataset creation

For the system-level parameters, RAN4 can initially adopt the configuration agreed for mixed dataset generation in the SI. The main system-level parameters that are needed for dataset creation are summarized in Table 1.

Table 1: System-level parameters for dataset creation following the configuration agreed in the SI.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, 4GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR 36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 30kHz for 4GHz |
| Simulation bandwidth |  | 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| UE distribution |  | 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Channel estimation |  | Realistic or ideal channel estimation |

RAN4 can initially adopt the system-level parameter configuration agreed for mixed dataset generation in the SI to generate the SLS samples needed for creating the new mixed dataset.

The mixed datasets used by RAN1 in their generalization evaluations were created assuming different deployment scenarios and configurations. Examples of deployment scenarios include InH, Umi, and UMa, while the configurations include carrier frequencies, UE distributions, and TxRU mappings [1].

In relation to this, RAN4 should discuss whether to follow the same principle used during the SI for the mixed dataset creation (i.e., assuming a single configuration for the SLS) or to consider different system-level parameter configurations. If the latter option is adopted, then a limited number of selected combinations of parameters from Table 2 should be agreed and whether a separate mixed dataset will be created for every agreed combination of parameters should also be clarified.

Table 2: Possible values for system-level parameters considered in RAN1 study.

| Parameter | Possible values |
| --- | --- |
| Deployment scenario | UMa, UMi, InH |
| Indoor/outdoor UE distributions | 80%/20%, 0%/100% |
| Carrier frequency RANK-1 | 2GHz, 3.5/4GHz |
| TxRU mappings RANK-1 | [2,8,2], [4,4,2], [8,2,2] |
| Tx port number | 16, 32 |
| Bandwidth | 10 MHz, 20 MHz |

We believe that the clarification of whether a separate mixed dataset will be created for every agreed combination of parameters is necessary, especially in relation to “Step 1-1: Align on configurations for mixed dataset construction, taking factors such as scalability over Tx port number and subband size and payload size into account”. For example, considering different Tx port numbers would mean having datasets with different dimensions. Hence, it would be beneficial to clarify whether “taking factors such as scalability over Tx port number and subband size and payload size into account” means creating separate mixed datasets for each configuration of these parameters.

RAN4 to discuss whether different configurations of system-level parameters, including Tx port number and subband size, are to be considered for mixed dataset creation and clarify whether a separate mixed dataset will be created for every agreed combination of parameters.

For the link-level parameters, the TDL channel model is used for defining performance requirements in legacy PMI reporting and is also used for defining performance requirements for CSI prediction. Hence, the TDL channel model can be potentially used for defining the AI/ML-based CSI compression performance requirements and thus can be adopted to generate the LLS samples needed for creating the new mixed dataset.

RAN4 to adopt the TDL channel model to generate the LLS samples needed for creating the new mixed dataset.

Assumptions for test decoder and reference encoder models

As stated below, RAN4 agreed in the last meeting to adopt a transformer-based backbone for the test decoder and the reference encoder.

Test decoder and reference encoder backbone The test decoder and the reference encoder in Option 3 will have the same backbone. - It is FFS if a single test decoder will be specified. - The test decoder and the reference encoder can have different structures, e.g. # of layers Model backbone Transformer is assumed as the baseline Only Quantized parameters are specified

Steps to develop the test decoder and the reference encoder were also agreed in RAN4#116bis, as shown below.

Steps to develop the test decoder and reference encoder: - Step 2: Align on the model backbone, model hyperparameters and the scalability solution. - Step 2-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution. - This can be in parallel with step 1. - Transformer is agreed as baseline for backbone - Same backbone for test decoder and reference encoder - Step 2-2: Preliminary selection of structures, taking into account complexity - [Step 2-2a Align on training parameters] - Step 2-3: Agree on metrics and steps for evaluation - Step 2-4: Check on performance alignment -> see simulation results from contributing companies. - Step 2-5: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed. - Step 2-6: Select one or more model structures considering complexity and robustness and decide how many models. - Step 2-7: Select one or more scalability solutions. This step can be done in parallel with Steps 2-2 to 2-4.

Model structure and parameters

As quoted in the below agreement, a reference transformer-based model structure was agreed in RAN1#120. We propose to adopt this same model structure and hyperparameters for the test decoder and the corresponding reference encoder in RAN4.

RAN1#120 Agreement For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure as one example structure for study purpose, Encoder description: Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer); Number of tokens Feature dimension of each token Output dimension dmodel per token. Positional encoding; Number of tokens positions Dimension of positional encoding for each token position: dmodel. Transformer blocks; Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice. Output linear layer; Reshape the matrix to a vector Output dimension zdim. Quantization; Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg - Total payload size for Vector quantization = Number of segments * bits per segment. Decoder description: The decoder has a mirroring design as the encoder. Details are to be discussed.

RAN4 to adopt the following transformer-based model structure for the test decoder and the corresponding reference encoder:

We provide in Table 3, the hyperparameters that we consider in our simulations. For the above model structure and the hyperparameters stated in Table 3, the obtained number of FLOPs is 244 M, and the number of parameters is 2.6 M.

| Table 3: Hyperparameters for the transformer-based models. Hyperparameter; Value. Number of tokens Ntoken; 13. Dimension of each token dtoken; 64. Dimension of transformer block dmodel; 128. Number of transformer blocks NTF; 7. Number of self-attention heads Nhead; 8. Dimension of latent space inside feedforward module dFF; 256. Output dimension zdim; 64. |  |
| --- | --- |

Scalability alternatives

As stated below, RAN4 agreed in the last meeting to consider the scalability options agreed in RAN1.

Scalability Unless the significant issues are identified, RAN4 is going to only discuss the scalability issues and options, which have been agreed in RAN1.

The following alternatives were considered in RAN1 for model structure scalability study.

RAN1#120 Agreement For model structure scalability study for temporal domain Case 0, - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters

For the choice of token dimension and feature dimension, we propose to adopt Alt 1 in RAN4, i.e., to use subband as the token dimension and Tx port as the feature dimension. Each token in Alt 1 represents an eigenvector per subband.

For scalability over the feature dimension, we propose that RAN4 adopts Alt 2, i.e., to use a common embedding layer with padding (e.g., zero-padding or other techniques for padding values). Using this alternative, the data of the larger antenna port count can be directly input into the encoder and output from the decoder to compress and reconstruct the data. For the data of smaller antenna port numbers, zero-padding and truncation can be applied to extend the data’s shape to the shape of the larger antenna port number data so that the data of smaller antenna port number can be treated similarly as the data of the larger antenna port number.

For scalability over the token dimension, and similarly to the alternative for scalability over the feature dimension, we propose to adopt Alt 2 in RAN4, i.e., to use padding at the input.

For scalability over payload configurations, we propose to adopt Alt3 in RAN4, i.e., varying quantization parameters.

RAN4 to adopt the following assumptions for test decoder and reference encoder model scalability:

- Use subband as token dimension and Tx port as feature dimension

- For scalability over feature dimension, use a common embedding layer with padding (e.g., zero-padding or other techniques for padding values)

- For scalability over token dimension, use padding at the input

- For scalability over payload configurations, vary quantization parameters

Metrics for model evaluation

Similarly to the metric considered during the SI, we believe that SGCS can also be used for model evaluation in the WI.

RAN4 can initially adopt the SGCS metric for model evaluation.

Means to capture models and datasets in 3GPP

To define requirements for the encoder assuming test decoder Option 3, RAN4 needs to specify a test decoder, the corresponding reference encoder, and a dataset. A part of the steps to develop these, RAN4 needs to agree on how and where to capture the models and the dataset.

Steps to develop the test decoder and reference encoder: - Step 3: Specify the model for the following work in WID. Where and how to specify may need further discussion. Note: At this stage, models using the agreed structures, dataset, scalability have already been created by most companies - Step 3-1 Decide how to select from the trained model(s) from each company - Step 3-2: Freeze model(s) - Step 3-3: Agree on how and where to capture the models, the dataset and relevant information

Links to specified locations of the models and the dataset can be used in RAN4 specifications to refer to the models and the dataset used for defining the requirements.

The models for the test decoder and the reference encoder and the dataset can be captured in RAN4 specifications using links to specified locations.

Conclusion

The following observations and proposals were made in this contribution:

Observation 1: Using only LLS for dataset creation may be acceptable for RAN4 testing purposes, but may not be the optimal strategy, especially if the expectation is that RAN1 may reuse the models trained on this dataset for inter-vendor training collaboration.

Proposal 1: RAN4 to consider using samples from both link-level and system-level simulations to construct the mixed dataset.

Proposal 2: Generalization over LLS samples should be verified in RAN4 simulations in both cases where the training dataset is created using SLS samples only or a combination of LLS and SLS samples.

Proposal 3: RAN4 can initially adopt the system-level parameter configuration agreed for mixed dataset generation in the SI to generate the SLS samples needed for creating the new mixed dataset.

Proposal 4: RAN4 to discuss whether different configurations of system-level parameters, including Tx port number and subband size, are to be considered for mixed dataset creation and clarify whether a separate mixed dataset will be created for every agreed combination of parameters.

Proposal 5: RAN4 to adopt the TDL channel model to generate the LLS samples needed for creating the new mixed dataset.

Proposal 6: RAN4 to adopt the following transformer-based model structure for the test decoder and the corresponding reference encoder:

Proposal 7: RAN4 to adopt the following assumptions for test decoder and reference encoder model scalability:

	Use subband as token dimension and Tx port as feature dimension

	For scalability over feature dimension, use a common embedding layer with padding (e.g., zero-padding or other techniques for padding values)

	For scalability over token dimension, use padding at the input

	For scalability over payload configurations, vary quantization parameters

Proposal 8: RAN4 can initially adopt the SGCS metric for model evaluation.

Proposal 9: The models for the test decoder and the reference encoder and the dataset can be captured in RAN4 specifications using links to specified locations.

- References

- TR 38.843 V19.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 19)”.