# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2521545

# Dallas, USA, 17th – 21th Nov, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements for LP-WUS

Agenda item:	6.16.2

Document for:	Approval

Introduction

RRM core requirements for Rel-19 LP-WUS are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. In this paper we will provide our initial views on RRM performance requirements on LP-WUS.

Discussion

- In previous meeting, we mainly discuss the core part on LP-WUS and the agreements are stable. In this stage, we would like to do some analysis on performance part. The following analysis could be considered from our side.

Topic 1 Test case design for higher priority frequency layer search

- In core part, we also defined the requirements for higher priority frequency layer search. The way forward for test cases on higher priority frequency layer search could be seen as below:

- Proposals - Option 1: Introduce test case for high priority frequency search in case 1. - Option 2: For RRM measurement offloading, not to introduce test case for higher priority carrier frequency measurement.

- In legacy test cases for cell re-selection, the higher priority search has been tested as below:

- Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1.

- We deem that the UE behaviour of measurement higher priority frequency shall be tested as legacy. The detailed test procedure could be reused as starting point.

- Proposal 1: RAN4 shall introduce test case for high priority frequency search in case 1.

- Proposal 2: The legacy test procedure on high priority frequency measurement shall be reused as starting point.

Topic 2 Test case design for entry/exit conditions for case1/case3

- The latest way forward could be seen as below:

- Entry/exit condition evaluation verification - Option 1: RAN4 to consider only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions (China Telecom Apple vivo Huawei QC) - Option 2: Define test case for entry/exit scenario (oppo ZTE Nokia) - Option 3: Not to define test cases for the evaluation requirements for Entry/Exit between different cases (Case#1 and Case#3).(MTK)

- We find that the majority of companies think even if UE satisfies the entry condition, it is up to UE to enter another modes or not. On the contrary, if UE satisfies the exit threshold, UE must exit the current states to another states.Thus, defining test cases for exit conditions is basic. However, to our understanding, we deem that the test cases for entry conditions shall also be defined. If network configures the entry threshold and UE would like to change the current state, UE has to ensure the measurement results is good enough to satisfy entry condition, and then switches to another state. That means we would like to define test cases for the sake of verifying the current requirements not only for entry evaluation condition but for exit evaluation condition. Consequently, we do think the tests cases for entry evaluation condition shall be considered.

- Proposal 3: RAN4 shall defines test cases for entry conditions:

- Scenario 1: UE enters Case 3 from legacy state.

- Scenario 2: UE enters Case 1 from Case 3.

- Scenario 3: UE enters Case 3 from Case 1.

Topic 3 Test case design for MR wake up delay

- In LP-WUS monitoring, if the UE receives LP-WUS, MR will be woke up and perform legacy paging monitoring. In previous meeting, we confirm the wake up delay defined in RAN1. To our understanding, the test case defined for verifying MR wake up delay shall be considered.

- Proposal 4: RAN4 shall define test case to verify the MR wake up delay once UE receives LP-WUS.

Conclusion

In this paper we provided our views on RRM requirements on R19 LP-WUS:

- Proposal 1: RAN4 shall introduce test case for high priority frequency search in case 1.

- Proposal 2: The legacy test procedure on high priority frequency measurement shall be reused as starting point.

- Proposal 3: RAN4 shall defines test cases for entry conditions:

- Scenario 1: UE enters Case 3 from legacy state.

- Scenario 2: UE enters Case 1 from Case 3.

- Scenario 3: UE enters Case 3 from Case 1.

- Proposal 4: RAN4 shall define test case to verify the MR wake up delay once UE receives LP-WUS.

- 4 References

- TS38.133