3GPP TSG-RAN WG4 Meeting #117		R4-2521570
Dallas, USA, 17 – 21 Nov. 2025

Title:	Further considerations on 6GR testability and OTA

Source:	OPPO

Agenda item:	8.11

Document for:	Approval

Introduction

In RAN4 #116bis meeting, the discussion on 6GR testability and OTA was triggered as per RAN4 guideline of 6GR SI [1] which was approved in RAN #108. This paper further presents our views and considerations on this aspect.

Discussion

Device type impacts to OTA tests

The high-level agreement was reached in RAN4 #116bis meeting [2] on the topic of device types.

Generally, the device types for 6G are not converged to a limited scope at the current early stage of 6G, and the device types includes hand-held UE, XR, wearable device, FWA, vehicle UE, NTN device, AI agent, etc. These different device types and form factors may have varying impacts and requirements on OTA testing systems, methods and procedures. However, once the hardware of the OTA testing system is determined, it is difficult to make changes or upgrades to match the demands from other device types. Therefore, clearly identifying the device types and form factors of 6G terminals is the primary task to study on 6G OTA test.

Furthermore, for the hardware setup of OTA test system, the most concerned issues are the following two aspects.

- What size of quiet zone / test zone is needed?

The size and quality of the quiet zone / test zone are highly related to the hardware setup of OTA chamber. To ensure the accuracy and repeatability of OTA testing, it is required that the EUT and any associated phantom combination are placed within the quiet zone / test zone. For current SISO OTA test chambers, the default size of quiet zone is a sphere with diameter D=30cm. Obviously, in the 6G stage, the size of some types of devices may exceed the 30cm diameter in this quiet zone, e.g. XR helmet associated with head phantom. So, to determine the requirements for the size of the quiet zone / test zone, the fundamental issue is to clarify the device type and form factor of 6GR terminal, as well as typical usage scenarios which is related to the associated phantoms.

- The requirements for the measurement antennas/probs and communication antennas/probs, e.g. the number of antennas, the polarization of the antennas, etc.

For SISO OTA test, the minimum number of measurement antennas is one. Multi measurement antennas can be used depending on OTA chamber implementation. The typical measurement antenna used in OTA chamber is a set of orthogonal linear polarized antennas. However, due to the fact that satellite antennas typically use circular polarization for communication, we have to consider whether OTA testing systems need to introduce circular polarized measurement antennas.

For MIMO OTA test for FR1 bands, determining the number of measurement antennas/probs is a controversial issue. Regarding 5G static MIMO OTA, the permitted chamber setup is multi-probe anechoic chamber (MPAC) system with 16 uniformly-spaced dual polarized probes. This setup builds up a cylinder test zone with diameter D=20cm and height H=20cm. The size of this quiet zone can not even fit into head phantom or hand phantom.

The device types and form factors for 6G are not converged to a limited scope at the current early stage of 6G, which may have varying impacts and requirements on OTA testing systems, methods and procedures.

Regarding the legacy OTA system setups, SISO OTA has the sphere quiet zone with diameter D=30cm, and MIMO OTA has cylinder test zone with diameter D=20cm and height H=20cm. Some form factors of devices can not be completely contained with the quiet zone / test zone.

Clearly identify the targetedform factors (e.g., size) of 6G terminals to be considered in OTA tests is the primary task at this moment.

Harmonizing OTA test for TN and NTN

It has been agreed to target a harmonized test system / test method for TN and NTN for FR1 devices in RAN4 #116bis meeting [2].

As a terminal enterprise, we are happy to see the harmonization of OTA test methodologies for TN and NTN, but the premise is that the complexity and cost of the testing system and methods are controllable, or in other words, the above goals can be achieved without significantly increasing test system complexity and cost. We will elaborate on our views on this issue through the following two aspects.

One aspect is the measurement metrics for TN and NTN.

For most TN devices, especially hand-held UEs, the OTA performance is evaluated by TRP and TRS, i.e. the radiated performance on the full sphere. Meanwhile, there are also some types of devices considering to adopt partial sphere metrics or metrics with specified weights on spherical test points, e.g. NHPRP/NHPRS are not precluded as XR OTA performance metrics[3].

For NTN devices, the UE may have various usage scenarios. And some usage scenarios require specific radiating orientation, while some other usage scenarios allow UE to transmit in any arbitrary orientation. Therefore, both full sphere TRP / TRS and partial sphere metrics will be used to evaluate the performance of NTN devices in different usage scenarios. RAN4 #114bis summaries the NTN usage scenarios and reaches the following agreement [4].

As can be seen from the above discussion, a compatible OTA test system for both full sphere metrics and partial sphere metrics is not only necessary for the harmonization of TN and NTN, but also for different types of TN devices. From OTA test system implementation perspective, there are no technical barriers to support both full sphere scanning and partial sphere scanning. And no additional hardware upgrade is introduced for the legacy SISO OTA chambers.

A compatible OTA test system for both full sphere metrics and partial sphere metrics is not only necessary for the harmonization of TN and NTN, but also for different types of TN devices.

No additional hardware upgrade is introduced for the legacy SISO OTA chamber to support both full sphere scanning and partial sphere scanning.

The other aspect to be considered is the polarization of measurement antennas for TN and NTN.

For OTA test of TN devices, the combined power from orthogonal dual-linear polarized antennas is used as the EIRP of a certain measurement point on spherical surface. This method has been proven to be stable and reliable through 20 years of industrial practice and application. And almost all OTA chambers worldwide use this kind of orthogonal dual-linear polarized antennas / probs as their hardware implementation.

On the other hand, satellite communication commonly uses circular-polarized antennas, mainly because they can maintain stable communication in complex environments, such as attitude adjustment, ionospheric interference, rain attenuation and so on. In 6G stage, hand-held UE that support NTN capability are likely to still use linear polarization or irregular elliptical polarization as communication antennas. Based on previous study and analysis during 5G OTA WI, the mismatch in antenna polarization between the transmitting side and receiving side may result in power loss. In detail, for downlink communication, the satellite base stations radiate via left-handed circular polarization (LHCP) or right-handed circular polarization (RHCP), and the UEs use orthogonal dual-linear polarization antennas to receive the signals. In this case, the UEs can fully receive all the power transmitted by the satellite base station without power loss. For uplink communication, the UEs radiate via linear polarization or irregular elliptical polarization, and the satellite base stations capture the signal via LHCP or RHCP. The power loss will occur in this situation, i.e. 3dB loss if linear polarization on the UE side, 0-3dB loss if elliptical polarization on the UE side. In summary, the power loss due to polarization mismatch occurs in uplink communication.

The power loss due to polarization mismatch (linear polarization and circular polarization) mainly occurs in uplink communication.

From the above discussion, the legacy OTA chamber using dual-linear polarized antennas as measurement antennas can perform UE sensitivity measurement without any hardware upgrade. However, for transmitting performance of UE, using orthogonal dual-linear polarization or single circular polarized polarization (LHCP or RHCP) will yield different results. Therefore, OTA chamber may need to upgrade hardware with capability of measuring circular polarization. And there will be diverse OTA chamber requirements for TN OTA test and NTN OTA test. RAN4 should study the system complexity and cost for specifying the unified OTA chamber measurement antenna hardware that can meet both TN and NTN requirements.

The system complexity and cost should be studied for specifying the unified OTA chamber measurement antenna hardware that can meet both TN and NTN requirements.

6GR AI/ML OTA testability

It is agreed that treating the AI related testability issues under “6G testability and OTA agenda” in RAN4 116bis meeting [5].

Under 6G testability and OTA topic, a high-level agreement is reached on 6GR AI/ML test method [2].

In 5G-A stage, the test case of AI Beam Management (BM) in FR2 is prioritized for developing AI OTA test methodology. And multi-AoA based RRM test setup is considered as baseline, and single-AoA test setup is not precluded. However, by the end of 5G-A, less progress has been made and there is no clear understanding and solution on how to setup and handle the AI BM test environment.

Technically, building up the channel model which can simulate beam sweep requires a large number of FR2 test probes in OTA chamber, which is not only a burden for hardware implementation, but also a complex channel simulation process from software perspective. That's why a lot of time was spent on studying and analyzing simplified channel models during the 5G-A WI. On the other hand, for AI/ML, it is not necessary to perform end-to-end evaluation through OTA approach, even for FR2 frequency range. In 6G, another approach for AI tests (especially for model test) could be that RAN4 may explore whether/how to decouple the signal acquisition testing (e.g., or model inputs) from model performance evaluation (e.g., for model performance) for AI use cases.

- Signal acquisition could continue to be tested via OTA methods, e.g., evaluating BM Set B measurement accuracy.

- Model performance could be assessed using standardized datasets defined by RAN4 (RAN4 specified datasets), enabling a virtual testing environment (maybe somehow like a Digital Twin setup). For instance, RAN4 study and specify test dataset for models under test. TE would provide the DUT with a predefined test dataset. The DUT would then return inference results to the TE for performance evaluation. This approach would reduce the need for complex OTA tests, accelerate model validation, and bring testing closer to real-world conditions depending on the quality of the test dataset.

For AI/ML, it is not necessary to perform end-to-end evaluation through OTA approach, even for FR2 frequency range.

Consider the decoupling test method (signal acquisition and AI model performance) for AI BM test case.

Conclusion

This paper discussed the considerations on 6GR testability and OTA issues.

- The device types and form factors for 6G are not converged to a limited scope at the current early stage of 6G, which may have varying impacts and requirements on OTA testing systems, methods and procedures.

Regarding the legacy OTA system setups, SISO OTA has the sphere quiet zone with diameter D=30cm, and MIMO OTA has cylinder test zone with diameter D=20cm and height H=20cm. Some types of devices can not be completely contained with the quiet zone / test zone.

A compatible OTA test system for both full sphere metrics and partial sphere metrics is not only necessary for the harmonization of TN and NTN, but also for different types of TN devices.

No additional hardware upgrade is introduced for the legacy SISO OTA chamber to support both full sphere scanning and partial sphere scanning.

The power loss due to polarization mismatch (linear polarization and circular polarization) mainly occurs in uplink communication.

For AI/ML, it is not necessary to perform end-to-end evaluation through OTA approach, even for FR2 frequency range.

- Clearly identify the targeted form factors (e.g., size) of 6G terminals to be considered in OTA tests is the primary task at this moment.

- The system complexity and cost should be studied for specifying the unified OTA chamber measurement antenna hardware that can meet both TN and NTN requirements.

- Consider the decoupling test method (signal acquisition and AI model performance) for AI BM test case.

Reference

RP-251881	New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

R4-2514647 WF for [116bis][110] 6G testability and OTA, Feature lead (vivo)

R4-2512693 Way Forward for [116][337] TRP_TRS_MIMO_OTA_Ph3, vivo

R4-2504763 WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3, vivo

R4-2514636_6G_AIML_FL_summary#3_v19, Moderator (Samsung)