3GPP TSG-RAN WG4 Meeting #117	R4-2521645

Dallas, USA, 17 – 21 November, 2025

Title: 	Discussion on performance requirements for RedCap NTN

Source: 	Huawei, HiSilicon

Agenda item:	6.19.3.1

Document for:	Discussion

Introduction

RRM performance requirements for RedCap NTN are discussed in RAN4#116-bis, and the outcomes are captured in WF [1]. In our view, following issues need to be further discussed.

- Accuracy requirements

- Test cases

In this paper, we will provide our views on RRM performance requirements for RedCap NTN.

Discussion

Accuracy requirements

In RAN4#116-bis, following was agreed for accuracy requirements for RedCap NTN.

#### Issue 1-1-1: Measurement accuracy requirement Online agreement < Agreement >: - RAN4 to define accuracy requirements for following measurements for RedCap in NTN: - Intra- and inter-frequency RSRP - Intra- and inter-frequency RSRQ - Intra- and inter-frequency SINR - L1-RSRP - Rx-Tx for NW verified location - The following side conditions are applicable for 1 Rx and 2Rx RedCap UE with FR1-NTN: Measurement type; 1Rx (e)RedCap UE; 2Rx (e)RedCap UE. Intra- and inter-frequency RSRP; clauses 10.1A.2 and 10.1A.4; clauses 10.1.2C and 10.1.4C. Intra- and inter-frequency RSRQ; clauses 10.1A.6 and 10.1A.8; clauses 10.1.7C and 10.1.9C. Intra- and inter-frequency SINR; clauses 10.1A.10 and 10.1A.12; clauses 10.1.12C and 10.1.14C. L1-RSRP; clause 10.1A.14; clause 10.1.19C. Rx-Tx for NW verified location; clause 10.1A.18; clause 10.1.25C. - RAN4 to reuse the NR operating band groups defined in NTN accuracy requirements for 1 Rx and 2Rx RedCap UE with FR1-NTN.

One remaining issue in accuracy requirements for RedCap NTN is the exact accuracy numbers.

- For 2Rx RedCap, it is straightforward to re-use the accuracy numbers from NTN clauses 10.1.xC

- For 1Rx RedCap, there is no NTN reference, but if we compare the accuracy requirements between NTN and TN for normal UE or 2Rx RedCap, we find that NTN is re-using the accuracy from TN. In this case, we suggest to re-use the accuracy numbers from TN RedCap clauses 10.1A.x.

Proposal 1: The following accuracy numbers are applicable for 1 Rx and 2Rx RedCap.

| Measurement type | 1Rx (e)RedCap UE | 2Rx (e)RedCap UE |
| --- | --- | --- |
| Intra- and inter-frequency RSRP | clauses 10.1A.2 and 10.1A.4 | clauses 10.1.2C and 10.1.4C |
| Intra- and inter-frequency RSRQ | clauses 10.1A.6 and 10.1A.8 | clauses 10.1.7C and 10.1.9C |
| Intra- and inter-frequency SINR | clauses 10.1A.10 and 10.1A.12 | clauses 10.1.12C and 10.1.14C |
| L1-RSRP | clause 10.1A.14 | clause 10.1.19C |
| Rx-Tx for NW verified location | clause 10.1A.18 | clause 10.1.25C |

Test cases

In RAN4#116-bis, following was agreed for test configuration for RedCap NTN.

#### Issue 1-1-3: Test configuration and parameters for (e)RedCap UE with FR1-NTN Online agreement < Agreement >: - Both FDD and HD-FDD duplex mode should be considered in test configuration for 1Rx and 2Rx (e)RedCap UE with FR1-NTN. - 15 kHz SCS and 10MHz bandwidth can be configured for 1Rx and 2Rx (e)RedCap UE with FR1-NTN in test cases. - Both GSO and NGSO are configured in test cases for 1Rx and 2Rx RedCap UE with FR1-NTN. - If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. According to the above, the following table is just a reference for 1Rx and 2Rx RedCap UE with FR1-NTN for cell reselection requirements. Table 1: Supported test configurations Configuration; Description. 1; GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW. 2; NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW. 3; GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW. 4; NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW. NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both.. - RAN4 need to further discuss whether to follow the test principle in NOTE2 case by case for different core requirements. - For the agreed test configuration, the existing RMC related parameters, Io values, SSB configuration and SMTC configurations defined for NTN can be reused for RedCap UE with FR1-NTN. - RAN4 NOT to consider NCD-SSB related configuration and RedCap specific UL/DL BWP configuration in test cases for (e)RedCap UE with NTN.

One remaining issue is how to test HD-FDD specific requirements. For example, the collision handling between UL and CSI-RS L1 measurement is a specific requirement for HD-FDD. In existing test case for RedCap, HD-FDD is added as a test configuration, and if UE supports both FD-FDD and HD-FDD, UE is only required to pass the test with one of them. It means the HD-FDD specific requirements will not be tested if UE pass the test with FD-FDD.

One option to resolve the issue is to require UE to pass the test with HD-FDD in test cases which involve HD-FDD specific requirements, e.g. test case for L1 measurement with CSI-RS. Another option is to define new test cases for HD-FDD specific requirements so that FD- and HD-FDD are tested if UE supports both. We slightly prefer the first option to reduce the number of test cases.

Proposal 2: For test cases HD-FDD specific requirements, define the test configuration such that UE supporting HD-FDD has to pass the test with HD-FDD configuration.

In RAN4#116-bis the test case list was agreed for RedCap NTN with some FFS.

- We support to define the test cases marked as FFS in Issue 1-1-5-2 in [1]. These test cases are defined for normal UE in NTN in clause A.14, and it is reasonable to keep the same test coverage for RedCap.

- On whether to define separate test cases for 2Rx and 1Rx, we slightly prefer not to do so. We understand separate test cases are defined for 1Rx and 2Rx in A.16 for RedCap in TN, but if we found that the test configurations and procedures are almost same for 1Rx and 2Rx, except that it may happen that due to longer delay for 1Rx the time duration needs to be increased. Therefore, we do not see strong need to have separate test cases. It is noted that whether we define separate test cases for 1Rx and 2Rx does not impact the number of test cases a specific UE needs to pass but only impact number of test case clauses in the spec.

Proposal 3: For the test case list,

- Define all test cases marked as FFS in in Issue 1-1-5-2 in [1]

- Define single test case for 2Rx and 1Rx with different test requirements

Conclusions

In this paper we provided our views on RRM performance requirements for RedCap NTN.

Proposal 1: The following accuracy numbers are applicable for 1 Rx and 2Rx RedCap.

| Measurement type | 1Rx (e)RedCap UE | 2Rx (e)RedCap UE |
| --- | --- | --- |
| Intra- and inter-frequency RSRP | clauses 10.1A.2 and 10.1A.4 | clauses 10.1.2C and 10.1.4C |
| Intra- and inter-frequency RSRQ | clauses 10.1A.6 and 10.1A.8 | clauses 10.1.7C and 10.1.9C |
| Intra- and inter-frequency SINR | clauses 10.1A.10 and 10.1A.12 | clauses 10.1.12C and 10.1.14C |
| L1-RSRP | clause 10.1A.14 | clause 10.1.19C |
| Rx-Tx for NW verified location | clause 10.1A.18 | clause 10.1.25C |

Proposal 2: For test cases HD-FDD specific requirements, define the test configuration such that UE supporting HD-FDD has to pass the test with HD-FDD configuration.

Proposal 3: For the test case list,

- Define all test cases marked as FFS in in Issue 1-1-5-2 in [1]

- Define single test case for 2Rx and 1Rx with different test requirements

Reference

- R4-2515056, Way Forward for [116bis][327] Rel-19 RRM performance_Part3, Moderator (CATT)