3GPP TSG-RAN WG4 Meeting #117	R4-2521653

Dallas, USA, 17 – 21 November, 2025

Title: 	Discussion on performance requirements for IoT NTN TDD

Source: 	Huawei, HiSilicon

Agenda item:	6.21.2

Document for:	Discussion

Introduction

RRM performance requirements for IoT NTN TDD are discussed in RAN4#116-bis, and the outcomes are captured in WF [1]. In our view, following issues need to be further discussed.

- Principle of testing TDD and FDD frame structure

- TSI_NB-IoT and TPRACH_NB-IoT

In this paper, we will provide our views on RRM performance requirements in IoT NTN TDD.

Discussion

Principle of testing TDD and FDD frame structure

#### Issue 4-2-4: Principle of testing TDD and FDD frame structure WayForward - Further discuss the principle of testing TDD and FDD frame structure. - Option 1: A UE capable of both TDD and FDD, the UE is required to pass either TDD or FDD per the DUT’s declaration. - Option 2: define a test case list to be tested for each configuration for the UE that supports both TDD and FDD frame structures - Other options are not precluded.

One issue raised up in RAN4#116-bis is the test applicability for UE supporting both FDD and TDD, with 2 options listed in [1] but no conclusion. Our view is that UE supporting both FDD and TDD should pass the test cases with each of the FDD and TDD configurations as default, and RAN4 can discuss the exception list of test cases, for which UE is required to pass with one of the FDD and TDD configurations.

Technically, most of the RRM test cases are related to measurement, and the measurement behaviour is quite different between FDD and TDD. In FDD UE can do measurement contiguously across multiple subframes, but for TDD the measurement has to be performed in burst manner. Even the measurement delay is similar for FDD and TDD, we believe it is meaningful to test the measurement behaviour separately.

In addition, the side condition for FDD and TDD requirements are different, i.e. -6dB for FDD and -1dB for TDD. In our view, they correspond to different coverage levels and leading to different delay and accuracy combination. It is noted that RAN4 has defined separate test cases for normal and enhanced coverage.

On the other hand, we are open to discussing whether some test cases can be tested with only one of the FDD and TDD configurations, to reduce the test burden for NB-IoT UEs. For example, intra-frequency neighbour cell measurement performance is tested in A.13.5.1, so the test case in A.13.5.3, which is testing location based intra-frequency measurement, can be tested with one of FDD and TDD because the location-based measurement triggering is not different for FDD and TDD.

Proposal 1: UE supporting both FDD and TDD should pass the test cases with each of the FDD and TDD configurations as default.

TSI_NB-IoT and TPRACH_NB-IoT

During CR drafting, we find one test requirement may need to be discussed. In test cases for intra- or inter-frequency neighbor cell measurement, we have following test requirements.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%. NOTE: The RRC re-establishment delay in the test is derived from the following expression: Tre-establish_delay= TUL_grant + TUE-re-establish_delay_NB-IoT. Where: - TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The NPRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used. - TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB-IoT + TSI_NB-IoT + TPRACH_NB-IoT - NNB-Iot-freq = 1 - Tsearch_NB-IoT = 0 ms - TSI_NB-IoT = 8320 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT FDD cell. - TPRACH_NB-IoT = 80 ms; it is the additional delay caused by the random access procedure.

One is the time required for receiving all the relevant system information. In our understanding, it includes MIB, SIB1, SIB31 and SIB33. For FDD it is defined as 8320ms. It is not clear how this value was derived, and whether the assumptions behind this value apply for TDD. For example, if the transmission periodicity of SIB31 was assumed as 160ms, it may not apply for TDD.

Another is the additional delay caused by the random access procedure. We understand it is the waiting time for PRACH, and it is unclear whether the existing value of 80ms can be re-used for TDD.

It is noted that above 2 parameters may also be needed in other test cases, e.g. cell reselection and RRC re-establishment. We suggest RAN4 to align the values for them.

Proposal 2: RAN4 to discuss the applicable value for the following 2 parameters in TDD.

- TSI_NB-IoT, the time required for receiving all the relevant system information (8320 ms for FDD).

- TPRACH_NB-IoT, the additional delay caused by the random access procedure (80 ms for FDD).

Conclusions

In this paper we provided our views on RRM performance requirements in IoT NTN TDD.

Proposal 1: UE supporting both FDD and TDD should pass the test cases with each of the FDD and TDD configurations as default.

Proposal 2: RAN4 to discuss the applicable value for the following 2 parameters in TDD.

- TSI_NB-IoT, the time required for receiving all the relevant system information (8320 ms for FDD).

- TPRACH_NB-IoT, the additional delay caused by the random access procedure (80 ms for FDD).

Reference

- R4-2515055, Way Forward for [116bis][310] Rel-19 RRM performance_Part2, Moderator (MediaTek inc.)