3GPP TSG-RAN WG4 Meeting #117	R4-2521715

# Dallas, USA, Nov. 17-21, 2025

Source:	Ericsson

Title:	(NR_newRAT-core) On gradual timing adjustment for CA and DC

Agenda item:	10.4.3

Document for:	Approval

# 1	Introduction

In this contribution we forward our proposal for this meeting, RAN4# 117.

# 2	Discussion

The requirements in TS 38.133 are minimum requirements. We do not want to impose unnecessary restrictions on a capable UE and force a specific restriction to actual transmission time difference < MTTD for gradual adjustment and for SCell transmission.

We therefore arrive to a guiding framework for our proposal:

- A UE which can manage MTTD > minimum requirement can continue transmission.

- Use ‘shall’, instead of ‘may’.

Observation 1: A framework for our proposal is:

- A UE which can manage MTTD > minimum requirement can continue transmission.

- Use ‘shall’, instead of ‘may’.

In our CR [1] we apply observation 1 to both the gradual adjustment and to the MTTD parts of the specification.

… #### 7.1.2.1 Gradual timing adjustment Requirements in this section shall apply regardless of whether the reference cell is on a carrier frequency subject to CCA or not. When the transmission timing error between the UE and the reference timing exceeds Te then the UE shall adjust the timing such that timing error is to within Te. The reference timing shall be NTA+NTA offset×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules: 1) The maximum amount of the magnitude of the timing change in one adjustment shall be Tq. 2) The minimum aggregate adjustment rate shall be Tp per second. 3) The maximum aggregate adjustment rate shall be Tq per 200 ms for SCS of UL signals smaller or equal to 120 kHz and 100 ms for SCS of uplink signals larger or equal to 480 kHz. where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in table 7.1.2.1-1. For multi-DCI based multi-TRP operation with two TAs, gradual timing adjustment specified in this clause applies for each TAG and shall be met for each TAG. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1. Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate Frequency Range; SCS of uplink signals (kHz); Tq; Tp. 1; 15; 5.5*64*Tc; 5.5*64*Tc. 30; 5.5*64*Tc; 5.5*64*Tc. 60; 5.5*64*Tc; 5.5*64*Tc. 2-1; 60; K*64*Tc; 2.5*64*Tc. 120; K*64*Tc; 2.5*64*Tc. 2-2; 120; 2.5*64*Tc; 2.5*64*Tc. 480; 0.8*64*Tc; 0.8*64*Tc. 960; 0.8*64*Tc; 0.8*64*Tc. NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] NOTE 2: When highSpeedMeasFlagFR2-r17 is configured for UE supporting power class 6, K = 4.5; otherwise, K = 2.5.. In any of the configurations below the UE shall stop transmitting in the sTAG if such adjustment results in transmit timing difference larger than the maximum timing difference in section 7.6 and the UE cannot continue to regulate its transmit timing. - Inter-band CA - Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support asynchronous operation - Intra-band EN-DC if UE doesn’t support asynchronous operation - Inter-band NE-DC if UE doesn’t support asynchronous operation - Inter-band NR-DC if UE doesn’t support asynchronous operation …

We apply the same idea to the SCell transmission in MTTD part of the specification.

… ## 7.5 Maximum Transmission Timing Difference … #### 7.5.2.1 Minimum Requirements for inter-band synchronous EN-DC A UE configured with E-UTRA PCell and PSCell shall stop transmitting on the PSCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and PSCell exceeds the specified minimum required MTTD and the UE cannot continue to regulate its transmit timing. … ### 7.5.3 Minimum Requirements for intra-band EN-DC A UE configured with E-UTRA PCell and PSCell shall stop transmitting on the PSCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and PSCell exceeds specified minimum required MTTD and the UE cannot continue to regulate its transmit timing. … ### 7.5.4 Minimum Requirements for NR Carrier Aggregation A UE configured with pTAG and sTAG shall stop transmitting on the SCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and SCell exceeds specified minimum required MTTD and the UE cannot continue to regulate its transmit timing. … #### 7.5.5.1 Minimum Requirements for inter-band synchronous NE-DC A UE configured with PCell and E-UTRA PSCell shall stop transmitting on the PSCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and PSCell exceeds specified minimum required MTTD and the UE cannot continue to regulate its transmit timing. … ### 7.5.6 Minimum Requirements for inter-band NR DC A UE configured with PCell and PSCell shall stop transmitting on the PSCell if after timing adjusting due to received TA command the uplink transmission timing difference between PCell and PSCell exceeds specified minimum required MTTD and the UE cannot continue to regulate its transmit timing. …

The complete proposal can be found in [6].

Proposal 1: 	Approve CR R4-25xxxxx, CR, (NR_newRAT-Core, TEI18) SCell transmission based on MTTD [UL_Timing_req]

# 3 	Summary

Observation 1: A framework for our proposal is:

- A UE which can manage MTTD > minimum requirement can continue transmission.

- Use ‘shall’, instead of ‘may’.

Proposal 1: 	Approve CR R4-2521716, CR, (NR_newRAT-Core, TEI18) SCell transmission based on MTTD [UL_Timing_req], from rel-18.

# References

[1] 		R4-2521716, CR, (NR_newRAT-Core, TEI18) SCell transmission based on MTTD [UL_Timing_req], Ericsson