3GPP TSG-RAN WG4 Meeting#117                                R4-2521759

Dallas, USA, 17-21 Nov, 2025

Agenda item:	6.13.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on conformance testing for the new RF requirements for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some initial views on conformance testing for SBFD BS.

- Sub-topic 5-1: General assumptions of conformance testing - Issue 5-1-1: System parameters of conformance testing Agreement: - For system parameters of conformance testing: - The operating bands supported by SBFD need to be added to TS 38.141-1 and TS 38.141-2. - The channel bandwidths of SBFD capable BS and SBFD subband configuration(s) per supported channel bandwidth need to be added to TS 38.141-1 and TS 38.141-2. - Issue 5-1-2: BS type for SBFD conformance testing Agreement: - For BS type for SBFD conformance testing: - The general principle that SBFD doesn’t support type 1-C needs to be captured in TS 38.141-1. - Issue 5-1-3: Declaration of supported SBFD configuration Agreement: - Declaration of supported SBFD configuration: - At least the following declarations shall be captured in TS 38.141-1/-2: - TX power for SBFD; - Bands supported for SBFD; - Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; - SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands. - Other declarations are not precluded - Issue 5-1-4: Testing applicability for supported SBFD configuration Way forward: - Further study on the following proposals on test configuration: - For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required. For the restriction on guardband size of SBFD - Based on SBFD core requirements definition, test complexity reduction study is deemed necessary given that e.g. it is going to cover all the declared SBFD configurations. - Issue 5-1-5: RF channels to be tested for SBFD operation Way forward: - Further study on the following proposals on RF channels to be tested for SBFD operation: - When testing the SBFD mode, it is recommended to configure the SBFD carrier at the bottom and top positions of the operating frequency band. - Sub-topic 5-2: Conformance testing for new RF requirements - Issue 5-2-1: Conformance testing related to transient period between SBFD and non-SBFD Way forward: - Further study on the following proposals on conformance test for transient period between SBFD and non-SBFD: - For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD. - Issue 5-2-2: Conformance testing related to in-channel adjacent subband leakage Way forward: - Further study on the following proposals on conformance test for in-channel adjacent subband leakage: - For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges. - For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2. - For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2. - Issue 5-2-3: Conformance testing related to in-channel adjacent subband blocking Way forward: - Further study on the following proposals on conformance test for in-channel adjacent subband blocking: - For in-channel adjacent subband blocking testing of SBFD capable BS, the transmitters must be turned on. - For in-channel adjacent subband blocking testing of SBFD capable BS, the desired Rx input signal and its frequency location are the same as those used for in-band blocking requirement. - For SBFD capable BS, the test frequency range of in-channel adjacent subband blocking is necessary to added to both TS 38.141-1 and TS 38.141-2.

Discussion

- 2.1. Transient period between SBFD and non-SBFD

Regarding the transition period between SBFD and non-SBFD symbols, as captured in the WF, a new test model is needed. In order to simplify the additional test model configuration for TDD BS, we propose to use the existing TDD pattern as following as baseline configuration for SBFD configuration.

Table 4.9.2.2-1: Configurations of TDD for BS type 1-C and BS type 1-H test models

| Field name | Value |  |  |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 | 5 | 5 |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

According to the RAN1/RAN2 signalling configuration, both SBFD time and frequency location would be indicated by SIB information, however from RAN4 specification, only the time domain configuration (e.g. starting slots/symbols, ending slots/symbols) are needed in the specification since frequency domain configuration (e.g. DL/UL sub-band size and location) are up to vendor’s declaration. In other words, this is not necessary to be specified clearly which is captured in the declaration part instead.

Proposal 1: For the transient period between SBFD and non-SBFD, propose to use the exiting TDD configuration as baseline and configure the additional SBFD time domain configuration and leave the SBFD frequency domain configuration up to the declaration.

Proposal 2: For the transition period between SBFD and non-SBFD symbols, propose to use the exiting test procedure for ON-OFF transition period as starting point and make the necessary modification for it.

- 2.2. In-channel adjacent subband leakage

Since in-channel adjancent sub-band leakage is a new requirement, RAN4 need to discuss the corresponding test model for it. However this requirement should be tested with the same test model for out-band emission requirement just with different measurement frequency range.

For test model for SBFD symbol in the specification, the existing test model in the specification is based on the transmission bandwidth configuration of the test signal, however for SBFD symbols, this NRB should be updated as the declared as DL sub-band.

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

In the following Figure 2.2-1, we would like to share the understanding the test mode for NR-FR1-TM1.1 in the visualized way.

- Figure 2.2-1. the visualized resource allocation for the legacy TM1.1

Considering the DU/UD and DUD configurations, the test model could be defined as shown in Figure 2.2-2.

- Figure 2.2-2. the visualized resource allocation for  the updated TM1.1 for SBFD operation.

- Proposal 3: propose to consider the following updated test model for NR-FR1-TM1.1 and NR-FR2-TM1.1 for SBFD operation for ISCL requirement.

| Parameter | Value [DU] |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

| Parameter | Value [DU case] |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | NRB -NRB,SBFD,DL |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | NRB - 3 |

| Parameter | Value [DUD] |
| --- | --- |
| # of PRBs PDSCH nRNTI=03 | NRB,SBFD,DL,2 |
| Modulation PDSCH nRNTI=03 | QPSK |
| Starting RB location of PDSCH nRNTI=03 | NRB -NRB,SBFD,DL,2 |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL,1 - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

- Proposal 4: For test procedure and MU for In-channel adjacent subband leakage, propose to use the test procedure for absolute ACLR limit as baseline and make the necessary modification for it.

- 2.3. In-channel adjacent subband blocking

In-channel adjacent subband blocking is a new Rx requirement, however this is similar as in-band blocking requirement with slightly different power level and frequency position, therefore we propose to use the existing test procedure for in-band blocking requirement as baseline and make the necessary modification for it.

- Proposal 5: For test procedure and MU for In-channel adjacent subband blocking, propose to use the existing test procedure for in-band blocking requirement as baseline and make the necessary modification for it.

Conclusions

In this contribution, we want to share some further views on conformance testing for SBFD BS and proposals/observations are made as following:

Proposal 1: For the transient period between SBFD and non-SBFD, propose to use the exiting TDD configuration as baseline and configure the additional SBFD time domain configuration and leave the SBFD frequency domain configuration up to the declaration.

Proposal 2: For the transition period between SBFD and non-SBFD symbols, propose to use the exiting test procedure for ON-OFF transition period as starting point and make the necessary modification for it.

- Proposal 3: propose to consider the following updated test model for NR-FR1-TM1.1 and NR-FR2-TM1.1 for SBFD operation for ISCL requirement.

| Parameter | Value [DU] |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

| Parameter | Value [DU case] |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | NRB -NRB,SBFD,DL |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | NRB - 3 |

| Parameter | Value [DUD] |
| --- | --- |
| # of PRBs PDSCH nRNTI=03 | NRB,SBFD,DL,2 |
| Modulation PDSCH nRNTI=03 | QPSK |
| Starting RB location of PDSCH nRNTI=03 | NRB -NRB,SBFD,DL,2 |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL,1 - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

- Proposal 4: For test procedure and MU for In-channel adjacent subband leakage, propose to use the test procedure for absolute ACLR limit as baseline and make the necessary modification for it.

- Proposal 5: For test procedure and MU for In-channel adjacent subband blocking, propose to use the existing test procedure for in-band blocking requirement as baseline and make the necessary modification for it.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2409958, Way forward for [111][313] NR_duplex_evo, Approved.

- R4-2413283, On modification of existing TX RF requirements for SBFD, Huawei

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Approved

- R4-2411640,On the potentially new requirements for SBFD operation for FR1 and FR2-1, Samsung

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Approved.

- R4-2419907	Way Forward for [113][306] NR_duplex_evo_General, Approved

- R4-2419908	Way Forward for [113][307] NR_duplex_evo_BSRF, Approved.

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.

- R4-2512665,Way Forward for [116][306] NR_duplex_evo_General

- R4-2512680,Way Forward for [116][307] NR_duplex_evo_BSRF

- R4-2515158, Way Forward for SBFD BS conformance, Ericsson, approved.

- R4-1905121, draft CR to TS38.141-1 on test model(Section 4.9.2), Huawei, Ericsson, ZTE, endorsed