3GPP TSG-RAN4 Meeting #117	R4-2521764

Dallas, US, Nov. 17 – 21, 2025

Source:	Nokia

Title:	OTA and testability for 6G

Agenda item:	8.11

Document for:	Discussion

Introduction

In the last RAN4 meeting #116-bis in Prague, the WF for OTA and testability for 6G in [1] were endorsed. In this paper, we would like to present our views related to below Topics:

# Topic #2: New test methodologies for new 6GR frequencies

### Sub-topic 2-1 Testability for new 6GR frequency

Issue 2-1-1: Testability for New Frequencies between FR1 and FR2

Agreements:

RAN4 study the testability including conducted testing and radiated testing for the new frequencies. The following aspects can be considered:

- The frequency range definition will be discussed under the spectrum agenda.

- For conducted test, use existing test method as a starting point

- For OTA test

# Topic #3: OTA test methods for Multi-Tx and CA

### Sub-topic 3-1 OTA for 6GR multi-carrier

Issue 3-1-1: OTA test methodology for FR1 CA

Agreements:

- RAN4 consider the study of testability for single carrier (with single or multi-Tx/Rx) as 1st priority.

# Topic #4: Testability for different Device types

### Sub-topic 4-1 OTA testability for different device types

Issue 4-1-1: OTA testability applicability for different UE types in 6GR day-1

Agreements:

- RAN4 consider the study of OTA test system to better accommodate different UE types, new feature/functionalities, new performance metrics and test cases.

- The UE types and form factor considered in testability can be further discussed in this SI.

- RAN4 study the feasibility of developing a single system to cover above aspects.

# Topic #5: AI/ML OTA testability

### Sub-topic 5-1 Enhanced OTA test method for 6GR AI/ML cases

Issue 5-1-1: RAN4 study on enhancement of AI/ML testability for 6GR

Agreements:

- The corresponding test method including both OTA and conducted, to verify the AI/ML features (FFS details) should be studied in RAN4 in 6G SI.

  - The methodologies identified in 5G will be considered as the starting point for both conducted and OTA.

# Topic #11: Others

### Sub-topic 11-3 General procedure on introducing alternative test method in RAN4

Issue 11-3-1: General procedure on introducing alternative test method in RAN4

Agreements:

- RAN4 could work on a general principle on introducing alternative methods.

Discussion

Discussions

### OTA test requirement for new 6G frequency bands

Studies of channel properties in the 7–24 GHz range, considered a potential 6G mid-band, have been documented in 38.901. These studies form a fundamental basis for 6G wireless communications, and related investigations for 6G new bands are ongoing in 3GPP.

In the current discussion, one of the key issues is whether to use conductive tests or OTA tests for these new 6G bands. Two main concerns have been raised:

- The complexity of the conductive test. For example, performing 4Tx conductive tests requires connecting four cables to the UE and modifying corresponding emulators, which is difficult in practice.

- Impact of physical connectors: Soldering physical connectors directly to the PCB can significantly affect UE RF performance, especially at higher frequencies. This is a primary reason why FR2 frequencies rely on OTA tests rather than conductive tests.

Given that 6G frequencies are higher than the FR1 range, the influence of physical connectors on RF performance, particularly the impedance of transmission lines between the radio chip and the antenna, cannot be ignored. Therefore, we propose considering OTA tests as the starting point for testing in 6G frequency bands.

There are no conductive tests in the FR2 frequency range because physical connectors on the PCB can significantly impact RF radiation performance.

Conductive tests for multiple transmitters, such as 4Tx, are complex due to the need to connect multiple cables to the UE.

We propose considering OTA tests as the starting point for the 6G frequency range.

### OTA test methods for Multi-Tx and CA

In the current 3GPP specifications (38.151 and 38.827), MIMO OTA test requirements for downlink (DL) use cases are defined. However, corresponding OTA test specifications for uplink (UL) MIMO use cases have not yet been developed.

In 5G, the UE RF architecture with 2Tx/4Tx is well studied and defined. For 6G UEs, a similar RF architecture can be reused and prioritized. The related TRP and MIMO OTA requirements should be studied first, and the corresponding OTA test method development should be given priority.

As agreed in the previous WF meeting, the study of testability for single-carrier scenarios (with single or multi-Tx/Rx) should be treated as the first priority. Therefore, we propose starting with single-carrier scenarios using 2Tx and 4Tx configurations for this topic.

OTA test specification related to UL MIMO use cases has not been developed yet in 38.151.

TRP and MIMO OTA requirements for 2Tx/4Tx should be studied first.

### Testability for different Device types

- The absence of specific UE categories initially (in Rel-15) made it challenging to manage the smooth introduction of a new device type with distinct capabilities, such as eMBB, RedCap UE types. In the current 5G NR UE definition, lack of solid reference and baseline led to proceeding in standard with devices requirements and capabilities scattered across different domains (RAN1, RAN4, RAN2), with challenging identification of a device type, making it impossible to recognize actual support of a baseline and/or complete set of device capabilities. However, instead of narrowing device design, this wide scope drove the development of an increasingly varied set of device categories (RedCap, eRedCap, XR, CPE, V2X, UAV), each shaped by distinct constraints in power, form factor and connectivity.

- For OTA testing, this implies that corresponding test method adjustments may be necessary to properly address the new 6G UE category definitions. However, this work can only proceed once the 6G UE type framework has been clearly defined. Until then, it may be more practical to defer the start of OTA work to align with the finalized 6G UE category definitions.

New 6G UE category definition will be studied, addressing the development of an increasingly varied set of device categories. OTA test requirements and test methods should be correspondingly adjusted.  However, we can wait until those works are done in other parts of 3GPP.

- It is has been agreed in the last meeting that

- RAN4 consider the study of OTA test system to better accommodate different UE types, new feature/functionalities, new performance metrics and test cases.

In 6G, UE sizes can vary significantly, including folded phones, IoT devices, standard-size phones (size 1, size 2), XR devices, VSAT terminals, and more. The size difference between the smallest and largest UEs can be several times. However, in a given physical OTA chamber, the size of the quiet zone is fixed, which may make it challenging to accommodate such a wide range of UE sizes unless the quiet zone is sufficiently large.

The fixed size of the quiet zone in a physical chamber may make it difficult to accommodate different UE types with varying sizes..

We propose consulting the TE vendor to determine whether the quiet zone can be made large enough to accommodate UEs of varying sizes.

#### Testability for foldered UE

- The folded UE has become popular in the market. The CTIA has defined the OTA specification for both folded mode and unfolded mode since there are use scenarios for both modes.

- In CTIA, testing is required in both the folded and unfolded states, as communication functionality may be utilized in either configuration. Hand phantom placements differ depending on the device in the unfolded state and folded state. If the unfolded device dimensions exceed the allowable size of the hand phantom (e.g., approximately double the standard size), testing in that state shall be conducted only in free space. If the folded phone is half of the size 1 UE, then it should be placed at the top of the hand phantom [2].

- 3GPP has not discussed these details related to the folded phone OTA test. It only specifies that testing should be based on the UE declaring a primary mechanical mode. In principle, it does not exclude the requirement for testing in either the folded or unfolded state, but only one of the two needs to be selected. However, specific details such as the hand phantom placement for the folded configuration have not yet been incorporated.

Folded phones are in high demand in the market. CTIA has developed the OTS test specification for a folded phone based on the unfolded and folded states. However, there is no folded phone OTA test specification defined yet in 3GPP.

Refer to the CITA folded phone OTA test plan, study and develop a folded phone OTA test plan related to measurement setup, test procedure, and preliminary MU for 3GPP.

### Testability for High Power UEs

For NTN communication with satellites, it is beneficial to use high-power UEs to compensate for the attenuation of the signals traveling through the extremely long distances between the UE and the satellite. However, high-power operation comes with trade-offs. Potential issues include chip heating, human exposure to radiation (MPE concerns), and interference with other gNBs and UEs. To manage these risks, mechanisms such as MPR, duty cycle limitations, and other technologies are employed.

Beyond NTN scenarios, high-power UE studies above 31 dBm are also being considered for UAVs and other 6G UE devices. As high-power UEs are expected to become more common in Rel-20 and beyond, OTA testing for high-power operation becomes necessary.

Careful study of high-power UE mechanisms, such as power headroom reporting and other power control methods, is ongoing in RAN4 under other WIs. OTA test development will need to closely follow these studies to define appropriate test methods that address high-power UE considerations.

High-power UEs may cause chip heating, MPE concerns, and coexistence issues. Well-controlled transmit power, using mechanisms such as MPR and duty cycle, is required, and these considerations need to be more carefully addressed than for non-high-power UEs.

Consider developing OTA test methods to properly address high-power UE transmit power requirements. This may include mechanisms such as MPR, duty cycle, power headroom reporting, and monitoring to control the UE’s maximum output power across different scenarios.

### Enhanced OTA test method for 6GR AI/ML cases

- The development of OTA test methods for AI/ML-based beam management and mobility is already under discussion in Rel-19, and further enhancements are expected to continue in Rel-20.

- Specifically, the OTA test method for beam management (BM) has been developed in relation to FR2 use cases, while the OTA test method for mobility has been developed in relation to FR1 use cases. These two test methods can serve as flagship examples and provide reference baselines for their respective frequency ranges (FR1 and FR2).

- Accordingly, they can be leveraged as reference points for the continued development of OTA test methods for AI/ML use cases in future releases.

The OTA test methods developed for AIML mobility is for FR1 use case, and AIML BM is for FR2 use case. They can be leveraged as reference points for the continued development of OTA test methods for AI/ML use cases in future releases.

### Others: General procedure on introducing alternative test method in RAN4

We often see both online and offline discussions struggling with how to handle new technologies and new OTA test methods when they are brought up by companies to the 3GPP meetings. When new technologies or new OTA test methods are proposed in 3GPP, they should be seen not as a source of confusion but as opportunities for necessary evolution, with a clear and standardized procedure guiding their study. By documenting pros and cons, comparisons to baseline methods, and typical use cases in TPs, we can provide guidance for selecting which methods may ultimately become standards while also recording the evolution steps of OTA testing in 3GPP.

New OTA test method proposals brought by companies to 3GPP should be regarded as opportunities for advancing and evolving the standard. There should be no struggle in handling them.

With TPs documenting pros and cons, baseline comparisons, and use cases to support clear decisions on future standards while recording the evolution of OTA testing.

Standardize the procedure of studying the new OTA test methods proposed in the 3GPP meetings.

Standardize the TP structure to capture pros and cons, baseline comparisons, and use cases of new methods, so that the confusion and struggles often faced in 3GPP meetings when new methods are proposed can be transformed into a clear and transparent decision-making process.

Conclusion

The following Observation and Proposals were made on the OTA topics:

- There are no conductive tests in the FR2 frequency range because physical connectors on the PCB can significantly impact RF radiation performance.

Conductive tests for multiple transmitters, such as 4Tx, are complex due to the need to connect multiple cables to the UE.

- We propose considering OTA tests as the starting point for the 6G frequency range.

OTA test specification related to UL MIMO use cases has not been developed yet in 38.151.

TRP and MIMO OTA requirements for 2Tx/4Tx should be studied first.

New 6G UE category definition will be studied, addressing the development of an increasingly varied set of device categories. OTA test requirements and test methods should be correspondingly adjusted.  However, we can wait until those works are done in other parts of 3GPP.

The fixed size of the quiet zone in a physical chamber may make it difficult to accommodate different UE types with varying sizes..

We propose consulting the TE vendor to determine whether the quiet zone can be made large enough to accommodate UEs of varying sizes.

Folded phones are in high demand in the market. CTIA has developed the OTS test specification for a folded phone based on the unfolded and folded states. However, there is no folded phone OTA test specification defined yet in 3GPP.

Refer to the CITA folded phone OTA test plan, study and develop a folded phone OTA test plan related to measurement setup, test procedure, and preliminary MU for 3GPP.

High-power UEs may cause chip heating, MPE concerns, and coexistence issues. Well-controlled transmit power, using mechanisms such as MPR and duty cycle, is required, and these considerations need to be more carefully addressed than for non-high-power UEs.

Consider developing OTA test methods to properly address high-power UE transmit power requirements. This may include mechanisms such as MPR, duty cycle, power headroom reporting, and monitoring to control the UE’s maximum output power across different scenarios.

The OTA test methods developed for AIML mobility is for FR1 use case, and AIML BM is for FR2 use case. They can be leveraged as reference points for the continued development of OTA test methods for AI/ML use cases in future releases.

New OTA test method proposals brought by companies to 3GPP should be regarded as opportunities for advancing and evolving the standard. There should be no struggle in handling them.

With TPs documenting pros and cons, baseline comparisons, and use cases to support clear decisions on future standards while recording the evolution of OTA testing.

Standardize the procedure of studying the new OTA test methods proposed in the 3GPP meetings.

Standardize the TP structure to capture pros and cons, baseline comparisons, and use cases of new methods, so that the confusion and struggles often faced in 3GPP meetings when new methods are proposed can be transformed into a clear and transparent decision-making process.

- References

- R4-2514647, WF for [116bis][110] 6G testability and OTA

- RP-251881; New SID: Study on 6G Radio

- CTIA, Test Plan for Wireless Device Over-the-Air Performance, v3.8.2, April 2019.

- R5-247967, Device Positioning for FR2 MIMO OTA Testing.