3GPP TSG-RAN WG4 Meeting #117                                                      R4-2521790

# Dallas, US, 17 – 21 Nov. 2025

Source:		Nokia

Title:	Reply R5-253653 LS on Beam Correspondence Initial Access

Agenda item:		11.3

Document for:	Discussion

# Introduction

Based on the LS On Beam Correspondence Initial Access from RAN5 to RAN4 in RAN4 #115 meeting [1] and the LS draft in the previous RAN4 #116-bis meeting [2], we’d like to present our view on related issues.

1. Overall Description (LS from RAN5): RAN5 has discussed the test procedure and feasibility of 6.6.4 Beam Correspondence in RRC_INACTIVE requirements. RAN5 seeks the clarification from TS38.101-2 sub clause 6.6.4 Beam correspondence for power class 3 to support test procedure with UE Beamlock test function in the IDLE mode and without UE Beamlock test function in the IDLE mode. As stated in 6.6.4.1, UE shall meet the spherical coverage requirement for initial access and RRC_INACTIVE according to Table 6.2.1.3-3, with its autonomously chosen UL beams and without uplink beam sweeping, using the side conditions for initial access and in RRC_INACTIVE as defined in Clause 6.6.4.3.4. RAN5 seeks the following clarifications: - RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements? - Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams? - Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such? - Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined? 2. Actions: To RAN4 group. ACTION: RAN5 respectfully requests RAN4 to respond to the above questions.

# Discussion

  - RAN4 conclusion in the Rel-18 for beam correspondence requirement for initial access

RAN4 sent LS [3] to RAN5 following the conclusion of the WI “NR_RF_FR2_Ph3-Core” in Rel-18, which addressed testability issues related to the beam correspondence requirement for initial access. The LS noted that RAN4 had agreed to inform RAN5 to develop the beamlock function for initial access. However, during RAN5 discussions, both test methods, with and without the beamlock function, were brought up, raising the question of whether the beamlock function should be the primary focus.

Based on the RAN4 conclusion from the WI “NR_RF_FR2_Ph3-Core” in Rel-18, the beamlock function was considered for developing the RAN4 requirements. This choice is justified because the beamlock function had already been developed in RAN5 and was standardized as part of the beam correspondence requirement test procedures. Therefore, we simply echo the conclusions of WI “NR_RF_FR2_Ph3-Core” from RAN4 in Rel-18.

We echo the conclusion of WI “NR_RF_FR2_Ph3-Core” in RAN4 from Rel-18 that the beamlock function was considered for developing the RAN4 requirements.

We have previously submitted the Tdoc containing the draft LS in [4], and our position remains unchanged. Here, we reiterate our suggested wording for the LS reply as follows:

RAN4 cannot prevent RAN5 from developing new test methods. If UE vendors believe they can meet the already defined requirements without beam lock function, it is not precluded as long as the core requirements are met.

Beam lock function does not prevent UE from autonomously choosing other beams in the test step 1 and step 3. But TE has to indicate/signal the UE that it should not change its beam in step 2 and step4 during measurement of that beam, this because TE need time to do measurement. Beam lock function is only an aid for the specific test case.

The core requirement is independent from which test method will be used to verify the spherical coverage. As long as core requirements are met, either test method can be used.

To agree that there will be no further debate regarding the facts stated in LS R4-2317764, which RAN4 sent to RAN5 following the conclusion of the WI “NR_RF_FR2_Ph3-Core” in Release 18.

Include the views previously submitted by Nokia in R4-2511273 in the LS reply to RAN5.

  - New Approach for Developing Test Methods for Rel-20 and 6G

The use of test methods without the beamlock function for evaluating the RAN4 beam correspondence requirement for initial access has been proposed in RAN5. However, according to the RAN4 conclusion of the WI and the LS sent to RAN5 [3], the beamlock function was assumed when RAN4 developed the requirement for testing. As a result, the discussion and debate over testing without beamlock have been escalated in RAN5, and an LS was sent to RAN4 for clarification.

We observe the difficulties in directly replying to the LS. At the same time, we view this as an opportunity to discuss and clarify the approach for developing new test methods. This can help ensure that future work is carried out more efficiently and consistently.

The discussion on the beamlock function can be viewed as an example highlighting the way of working. It underscores the need to improve collaboration and enhance efficiency in developing test methods.

In the evolution of 3GPP toward 6G, new test methods will be developed to address emerging technologies such as AI, sensing, MRSS in 5G and 6G, and integrated TN and NTN networks. Establishing clear guidelines for the approach to new technologies and test method development can significantly enhance efficiency within 3GPP.

Clarify the way of working for developing new test methods to support the evolution of 3GPP requirements and future technologies. Enhance coordination with RAN5 to improve efficiency.

# 3	Conclusions

The following observations and proposals were made:

- We echo the conclusion of WI “NR_RF_FR2_Ph3-Core” in RAN4 from Rel-18 that the beamlock function was considered for developing the RAN4 requirements.

RAN4 cannot prevent RAN5 from developing new test methods. If UE vendors believe they can meet the already defined requirements without beam lock function, it is not precluded as long as the core requirements are met.

Beam lock function does not prevent UE from autonomously choosing other beams in the test step 1 and step 3. But TE has to indicate/signal the UE that it should not change its beam in step 2 and step4 during measurement of that beam, this because TE need time to do measurement. Beam lock function is only an aid for the specific test case.

The core requirement is independent from which test method will be used to verify the spherical coverage. As long as core requirements are met, either test method can be used.

- To agree that there will be no further debate regarding the facts stated in LS R4-2317764, which RAN4 sent to RAN5 following the conclusion of the WI “NR_RF_FR2_Ph3-Core” in Release 18.

Include the views previously submitted by Nokia in R4-2511273 in the LS reply to RAN5.

The discussion on the beamlock function can be viewed as an example highlighting the way of working. It underscores the need to improve collaboration and enhance efficiency in developing test methods.

Clarify the way of working for developing new test methods to support the evolution of 3GPP requirements and future technologies. Enhance coordination with RAN5 to improve efficiency.

# References

- R5-253653, LS On Beam Correspondence Initial Access

- R4-2513626/R4-2515067, Draft reply LS on Beam Correspondence Initial Access

- R4-2317764, LS on testability issues of beam correspondence requirements

- R4-2511273, Reply R5-253653 LS on Beam Correspondence Initial Access