3GPP TSG RAN WG4 Meeting #117	 R4-2521914

Dallas, USA, 17 Nov. 2025 – 21 Nov. 2025

Agenda Item:	4.18.2

Source: 	Ericsson

Title:	maintenance of RRM core requirements for UE-initiated beam management

Document for:	Discussion

# Introduction

In this contribution we analyzed the UE initiated beam reporting (UE IBR) requirements for NR MIMO Phase 5.

# Discussion

During the last meeting, an issue was raised regarding the M value used in UE-initiated beam management, specifically in the context of the current beam versus the new beam.

The configuration for UE initiated beam reporting is provide in CSI-ReportConfig. Parts of CSI-ReportConfig is provided here for reference.

CSI-ReportConfig ::= SEQUENCE { reportConfigId CSI-ReportConfigId, carrier ServCellIndex OPTIONAL, -- Need S resourcesForChannelMeasurement CSI-ResourceConfigId, csi-IM-ResourcesForInterference CSI-ResourceConfigId OPTIONAL, -- Need R >> Some of the configuration is skipped. Please check TS 38.331 v19.0 >>> timeRestrictionForChannelMeasurements ENUMERATED {configured, notConfigured}, timeRestrictionForInterferenceMeasurements ENUMERATED {configured, notConfigured}, csi-ReportUE-IBR-r19 CSI-ReportUE-IBR-r19 OPTIONAL, -- Need R

For defining the event triggered reporting delay requirement, RAN4 has used the parameter M for determining the number of samples or the measurement period.

The value of M depends on whether the parameter timeRestrictionForChannel is configured:

- If timeRestrictionForChannel is enabled, then M = 1

- If timeRestrictionForChannel is not enabled, then M = 3

Currently, the M value is defined per CSI-ReportConfig, and the current beam is not explicitly included in the configuration. As can be seen in the highlighted text above, only resourcesForChannelMeasurement is included in the CSI-ReportConfig and the resourcesForChannelMeasurement corresponds to either new beams or current beams (current beam may be associated with another set of RS). At present, the CSI-ReportConfig may include the new beams or current beams, and the parameter timeRestrictionForChannel, which influences the M value, is only applicable to these new beams or current beams.

As a result, there is ambiguity regarding which M value should be applied to the current beam measurement delay requirements. Since the current beam is not directly part of the new beam's reporting configuration, its M value cannot be inferred from the same configuration. The M value for the current beam is determined by the CSI-ReportConfig linked to its RS, not by the configuration used for the new beams. If the RS associated with the current beam belongs to a different CSI-ReportConfig than the one used for the new beams, it may have a different M value.

Moreover, the situation becomes trickier when the current beam is associated with multiple CSI ReportConfig instances. This leads to further ambiguity in determining the correct M value, as it is unclear which configuration should take precedence or how the UE should resolve conflicting M values across different configurations.

We expect the UE to perform measurements according to all relevant CSI-ReportConfig instances. However, for delay requirements, we propose that the configuration with the lowest M value should be considered. We are also fine with one of the following options.

- If current beam is configured in multiple ReportConfig, then UE shall select the lowest M value and measure as per that for event evaluation delay requirements.

- If current beam is configured in multiple ReportConfig, then UE shall select the same M value as the new beam and measure as per that for event evaluation delay requirements.

- RAN4 to agree on one of the following.

  - If current beam is configured in multiple ReportConfig, then UE shall select the lowest M value for the measurement delay of the event triggered reporting.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to agree on one of the following.

  - If current beam is configured in multiple ReportConfig, then UE shall select the lowest M value for the measurement delay of the event triggered reporting.

# References

[1]		R4-2420096, “WF on RRM requirements for NR_MIMO_Ph5”, Samsung

[2] 	RP-234007, New WID: NR MIMO Phase 5