3GPP TSG-RAN WG4 Meeting #117	R4-2521933

# Dallas, USA, Nov. 17-21, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Perf part of R19 LB CA

Agenda Item:	6.3.2

Document for:	Discussion

- Introduction

During last meeting, good progress around the perf part of R19 LB CA were achieved and summarized in [1]. Almost all test configurations and test scope were determined, with still two test cases are suspending. In this paper, we provide our views.

- Discussion

- In last meeting, the following test configurations were approved as below:

Agreement: - General configuration: - For test configuration and parameters, existing FR1 with 15kHz SCS test configuration and parameters could be baseline - CA between FDD PCell and SDL SCell is configured - PCell and SCell are co-located deployed and keep sync between them (Max RTD is 3us) - The SCS is 15kHz and the frequencies of PCell and SCell are lower than 1GHz - As baseline, test partially overlapped scenario for LB CA RRM, i.e., SSB/SMTC is partially overlapped with LB-CA switching pattern. - Working assumption for TC: SSB/SMTC periodicity is 20ms and switching pattern periodicity is 40ms; - The details of switching pattern can be FFS. - RAN4 will pick one TC in the list to cover fully overlapping and partially overlapping cases

- Based on the test configuration, the following test cases were determined with two test cases still suspending:

Agreement:

| TC Index | Requirement | RS used for test | Target Cell for testing | Support this TC |
| --- | --- | --- | --- | --- |
| 1 | Intra-frequency L3 measurement for activated SDL SCell or PCell | SSB | Both PCell and SCell in the same TC PCell and SCell can be configured with different events, details can be FFS. PCell is in partially overlapped case. SCell is in fully overlapped case. |  |
| 2 | Deactivated SDL SCell measurement and interruption requirement | SSB | Verify measurement requirement on SDL SCell. And in the same TC verify interruption requirement to the PCell. |  |
| 3 | SDL SCell activation/deactivation | SSB(for sync and L1-RSRP) and CSI-RS for CQI | SDL SCell. Only MAC CE based SCell activation is verified. |  |
| 4 | BFD/CBD for SDL SCell | Option 1: SSB (Apple) Option 2: CSI-RS Option 3: both |  | FFS whether this TC shall be introduced. |
| 6 | PL-RS switch delay for PCell | Option 1: SSB (Apple) Option 2: CSI-RS Option 3: both |  | FFS whether this TC shall be introduced. |
| 7 | L1-RSRP for SDL SCell | SSBe | SCell only |  |
| 8 | RLM requirement for PCell | SSB | FDD PCell |  |

- If we look at all the identified test cases for PCell verification, #1 could verify the L3 measurement for PCell based on switching pattern, #2 could verify the interruption on PCell due to deactivated SCell measurement, #8 could verify the RLM requirement on PCell. It could be observed that all the measurements in #1, #2 and #8 are semi-statically configured by RRC signaling. For the MAC CE triggered RS measurement on PCell, which could only be verified in #6.

- In last meeting, during the discussion on core part maintenance, the timeline of disabling switching pattern in MAC CE based SCell deactivation was approved as below:

Issue 1-1-4a: Application time of disabling switching pattern in SCell deactivation Agreement: - The switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation - TLBCA =1 ms for MAC-CE based - Further discuss for timer triggered SCell deactivation

- So before UE receiving the MAC CE based SCell deactivation command, UE should follow both the switching pattern and PL-RS configuration to determine the available PL-RS occasions for PL-RS switching. If the SCell deactivation MAC CE comes together with the PL-RS switching MAC CE or later than the PL-RS switching MAC CE, all PL-RS occasions are available as long as the timeline of (THARQ+3ms+ 1ms)/slot-length meets.

- Therefore, in #6, at least the former case should be verified, i.e. after UE receiving the PL-RS switching MAC CE, UE picks the available PL-RS occasions to measurement and measurement extension is necessary. Meanwhile, we suggest to verify the latter case as well, i.e. during the PL-RS switching procedure, the SCell deactivation MAC CE is received and UE should follow the timeline to disable the switching pattern and then all PL-RS occasions are available.

- Proposal 1: TC#6 is the only test case to verify the MAC CE triggered procedure for PCell, which is necessary.

- Proposal 2: The timeline of disabling the switching pattern by MAC CE based SCell deactivation is defined in core part but not any already approved test case to verify it.

- Proposal 3: In TC#6, the following three test motivations could be verified:

- Before the switching pattern being disable, UE follows the switching pattern to pick the available RS occasions;

- During the PCell measurement(PL-RS based), verify the timeline of (THARQ+3ms+ 1ms)/slot-length to disable the switching pattern when receiving the SCell deactivation;

- All RS occasions are available in PCell after the processing time of disabling the switching pattern.

- Another suspending test case is #4. We do not have strong view, it aims to verify the impact of switching pattern on the BFD/CBD.

- The Conclusion

In this contribution, we have the following proposals on the performance part of LB CA:

- Proposal 1: TC#6 is the only test case to verify the MAC CE triggered procedure for PCell, which is necessary.

- Proposal 2: The timeline of disabling the switching pattern by MAC CE based SCell deactivation is defined in core part but not any already approved test case to verify it.

- Proposal 3: In TC#6, the following three test motivations could be verified:

- Before the switching pattern being disable, UE follows the switching pattern to pick the available RS occasions;

- During the PCell measurement(PL-RS based), verify the timeline of (THARQ+3ms+ 1ms)/slot-length to disable the switching pattern when receiving the SCell deactivation;

- All RS occasions are available in PCell after the processing time of disabling the switching pattern.

- Reference

- R4-2514870, “WF on RRM requirements for NR_LBCA_Sw”, Apple,3GPP RAN4#116bis, 2025.