3GPP TSG-RAN4 Meeting #117	R4-2521954

# Dallas, United States, 17th - 21st November, 2025

Agenda item:		8.7

Source:				Keysight Technologies UK Ltd

Title:					On 6G Demod testability

Document for:		Discussion

# Introduction

In RAN4#116bis meeting, 6G Demod discussions were kicked off and a way forward in [1] was agreed.

This contribution intends to add some views on 6G Demod from testability perspective.

# Discussion

During RAN4#116bis meeting several companies proposed to study some testability aspects differently in 6G in comparison with 5G for making more realism on 6G testing. Proposals were captured in [2].

In Way Forward [1] it was agreed to continue discussing about some testability aspects that makes more realism for 6G testing:

### 1.2.7 Sub-topic: New TE functionalities

OLLA with link adaptation

- Feature Lead notes

  - Option 1 discussion has been captured in Chairman and Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on OLLA in the next meeting as several companies see value to study OLLA as part of 6G demod SI.

- Proposals

  - Option 1: Study to include OLLA in ATP requirements.

    - Option 1A: Use proposed OLLA model from R4-2300703 as a starting point.

    - Option 1B: Study the feasibility to include OLLA in ATP requirements.

SRS based precoding

- Feature Lead notes

  - Option 1 discussion has been captured in Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on SRS based precoding in the next meeting as several companies see value to study SRS based precoding as part of 6G demod SI.

- Proposals

  - Option 1: Study whether and how to define baseline SRS based precoding procedure in TE to enable aligned simulation assumptions.

Time/frequency/phase offset precompensation

- Feature Lead notes

  - Discussion of Option 1 has been captured in Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on time/frequency/phase offset precompensation in the next meeting as several companies see value to study time/frequency/phase offset precompensation as part of 6G demod SI.

- Proposals

  - Option 1: Study inclusion of higher layer aspects in demodulation requirements via dynamic TE decisions using known algorithms, e.g., applying timing offset reports (CJT).

Other new TE functionalities

- Feature Lead notes

  - Discussion of following options has been captured in Ad-hoc minutes.

- Proposals

  - Option 1: Study inclusion of higher layer aspects in demodulation requirements via increased and dynamic application of DUT feedback in the TE.

  - Option 2: Study inclusion of higher layer aspects in demodulation requirements via dynamic TE decisions using known algorithms, e.g., SU/MU scheduling, dynamic resource allocation/slots.

  - Option 3: Explore utilizing test equipment algorithms to evaluate features that rely on network-side processing, enabling realistic UE performance assessment without limiting network implementation flexibility.

  - Option 4: Identify features dependent on network-side processing and aim to define corresponding performance requirements, where feasible, using test equipment-based evaluation methods.

Additionally, another proposal to be discussed is how FR1 testing will be performed, below proposal was submitted to continue performing conducted testing for FR1 [1]:

Conducted and radiated testing

- Proposals

  - Option 1: Conducted test method can be considered for FR1 frequency range, and OTA test method can be considered for FR2 frequency range for both UE and BS demodulation requirements.

  - Option 2: Define test methodologies for FR3, including whether measurements are conducted or OTA.

However, if RAN4 suggests to study different algorithms and mechanisms for 6G testing with the goal of replicating scenarios exercised in real network deployments during conformance testing, conducted testing for FR1 and potentially FR3 will not be close to realistic conditions. In line with having more realism in FR1/FR3 testing, we have sumbitted some proposals in [3] and [4] under 6G OTA and testability agenda item where RAN4 could consider moving some Demod and/or CSI requirements from conducted to radiated testing to assess end-to-end performance. Examples of such test cases could introduce the maximum sustained data rate which would indeed benefit from more realistic RF conditions.

Observation 1: Some proposals in [1] are in line with defining more realistic algorithms and mechanisms for 6G conformance testing

Proposal 1: Study the transition of select conducted TCs to OTA for FR1 range to introduce realism

Additionally, other proposals in [1] suggests to study improvements in SNR definition to ensure SNR levels defined for conformance testing is closer to SNR that UE measured in the field:

### 1.2.4 Sub-topic: TxEVM and SNR

TxEVM aspects

- Feature Lead notes

  - TxEVM related discussion has been captured in Ad-hoc minutes.

- Proposals

  - Option 1: Study what EVM simulations assumptions should be used in demodulation and CSI requirements.

    - Option 1A: Study impact of TX EVM for higher modulation order/ MIMO layers on Demodulation requirements.

    - Option 1B: Abandon the SNR operating point limitations via fixed 20dB rule, or fixed TE TxEVM assumptions, and adopt a SNR limitation derivation based on actual TDRA/FDRA configuration.

    - Option 1C: Study whether the TxEVM requirements for the base station, at least in the simulations for deriving the demodulation requirements, could be tightened.

SNR aspects

- Feature Lead notes

  - SNR related discussion has been captured in Ad-hoc minutes.

- Proposals

  - Option 1: To ensure good field coverage, the SNR values in the demodulation requirements should follow the SSB SNR values that can be measured in existing 5G NR scenarios and those SSB SNR values that can be expected in upcoming 6G deployments.

  - Option 2: Study whether the coverage range for relevant field scenarios can be extended by defining demodulation requirements for larger SNR values as currently being used in 5G NR.

  - Option 3: Study whether the coverage range for relevant field scenarios can be extended for carrier aggregation. It should also be studied whether the SNR requirement should be dependent on the number of component carriers.

  - Option 4: Abandon the SNR operating point limitations via fixed 20dB rule, or fixed TE TxEVM assumptions, and adopt a SNR limitation derivation based on actual TDRA/FDRA configuration.

SNR improvements will depend on state of art from Test Equipment, Test Systems (radiated vs conducted) including the required device types that need to be covered. Hence, a detailed analysis on TE dynamic range is needed eventually to ensure which is the max testable SNR. Test system harmonization will need to be addressed as well which likely be addressed in the 6G OTA and testability agenda item.

Observation 2: Detailed analysis on TE dynamic range is needed to ensure which is the max testable SNR

Proposal 2: TE vendors to study the dynamic range/max testable SNR for conducted and OTA test systems when device types, 6GR operating frequencies, etc. are decided.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: Some proposals in [1] are in line with defining more realistic algorithms and mechanisms for 6G conformance testing

Observation 2: Detailed analysis on TE dynamic range is needed to ensure which is the max testable SNR

Proposal 1: Study the transition of select conducted TCs to OTA for FR1 range to introduce realism

Proposal 2: TE vendors to study the dynamic range/max testable SNR for conducted and OTA test systems when device types, 6GR operating frequencies, etc are decided. Test system harmonization will need to be addressed as well.

# References

- R4-2514645, WF on [116bis][106] 6G Demod, Feature Lead (MediaTek Inc.), 3GPP TSG-RAN4 Meeting #116-bis, October 2025

- R4-2514513, Topic Summary for [116bis][106] 6G Demod, Moderator (MediaTek Inc.), 3GPP TSG-RAN4 Meeting #116-bis, October 2025

- R4-2513347, On UE 6G OTA Testability Aspects, Keysight Technologies, 3GPP TSG-RAN4 Meeting #116-bis, October 2025

- R4-2522030, On UE 6G OTA Testability Aspects, Keysight Technologies, 3GPP TSG-RAN4 Meeting #117, Nov 2025