3GPP TSG-RAN4 Meeting # 117	R4-2521956

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | – | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on test case of L3 intra-frequency measurement with SSB adaptation_R19 NES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add the test case of L3 intra-frequency measurement with SSB adaptation in FR1. In this test case, the assumptions were taken into account: General principle Agreement: - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only - Both FR1 and FR2 - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement - Only test non-DRX mode - without SSB time index detection - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. SMTC and SSB configuration Agreement (in AH): - For L1-RSRP measurement test case: - SMTC periodicity from 20ms to 80ms, and from 80ms to 40ms. - SSB periodicity from 20ms to 80ms, and from 80ms to 40ms. - For other test cases: - SMTC periodicity from 80ms to 20ms. - SSB periodicity from 80ms to 20ms. How to verify SSB adaption Agreement: - For L3 Intra-frequency serving Cell measurement: - UE is configured with Event A1 or A2. - Condition of serving Cell is changed to satisfy the Event threshold at the instance after DCI 2-9 received + the transition period. - It is verified whether UE can trigger event within the delay based on the mapped additional SMTC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the test case of L3 intra-frequency measurement with SSB adaptation in FR1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relevant test case is vacant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

A.6.6.1.x	SA event triggered reporting tests with SSB adaptation without SSB time index detection without gap under non-DRX

A.6.6.1.x.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct measurement when SSB adaptation happens for an SCell and reporting of an event. This test will partly verify the intra-frequency cell search requirements for serving cell in clauses 9.2.5.1 and 9.2.5.2.

A.6.6.1.x.2	Test parameters

There are two carriers and one cell on each carrier in the test, NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as SCell in FR1 on NR RF channel 2. The testing configurations for NR cells are in table A.6.6.1.x.1-1. The test parameters for the Cell 1 and Cell 2 are given in table A.6.6.1.x.2-1, table A.6.6.1.x.2-2 and table A.6.6.1.x.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered reporting with Event A2 is used. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. At the starting of T2, the UE receives DCI format 2_9 in the PCell, and the SSB adaptation of SCell is indicated by the DCI format 2-9. As long as UE receives the indication of SSB adapatation of SCell, UE follows the transition requirements defined in Clause 9.1.6 and the transition period ends at the end of T2. At the starting of T3, the channel condition changes and the condition of Event A2 meets.

Table A.6.6.1.x.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.x.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1: Cell 1 2: Cell 2 | Two NR radio channel (1, 2) are used for this test |
| PCell |  | 1, 2, 3 | Cell 1 |  |
| SCell |  | 1, 2, 3 | Cell 2 | Cell 2 is the cell with SSB adaptation |
| SSB configuration |  | 1 | SSB.9 FR1 |  |
|  |  | 2 | SSB.9 FR1 |  |
|  |  | 3 | SSB.10 FR1 |  |
| SMTC configuration |  | 1 | SMTC.11 |  |
|  |  | 2 | SMTC.10 |  |
|  |  | 3 | SMTC.10 |  |
| A2-threshold | dBm | 1, 2, 3 | -130 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between Cell 1 and Cell 2 |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1 |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 1 |  |
| T3 | s | 1, 2, 3 | 1 |  |

Table A.6.6.1.x.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| SSB configuration |  | 1 | SSB.9 FR1 |  | SSB.9 FR1 | SSB.1 FR1 |
|  |  | 2 | SSB.9 FR1 |  | SSB.9 FR1 | SSB.1 FR1 |
|  |  | 3 | SSB.10 FR1 |  | SSB.10 FR1 | SSB.2 FR1 |
| SMTC configuration |  | 1 | SMTC.11 |  | SMTC.11 |  |
|  |  | 2 | SMTC.10 |  | SMTC.10 |  |
|  |  | 3 | SMTC.10 |  | SMTC.10 |  |
| SMTC associated with SSB adaptation |  | 1 | NA |  | SMTC.2 |  |
|  |  | 2 | NA |  | SMTC.1 |  |
|  |  | 3 | NA |  | SMTC.1 |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -1.46 | -Infinity |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | 4 | -Infinity |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -94 | -135 |
|  |  | 2 | -94 | -94 | -94 | -135 |
|  |  | 3 | -91 | -91 | -91 | -135 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.6.1.x.3	Test Requirements

The UE shall send one Event A2 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change #1>