3GPP TSG-RAN WG4 Meeting #117	R4-2522011

Dallas, US, November 17th – 21st, 2025

Source: 	KDDI, Murata, Samsung

Title:	TP to TR 38.746 for adding HPUE feature to CA_n40-n41-n77

Agenda item:	5.7.3

Document for:	Approval

- Introduction

This paper is a Text Proposal to TR 38.746 for adding HPUE UL feature to CA_ n40A-n41A-n77A with sBCS4 and 5.

PC2 fallbacks for this band combination have been approved. And the TP for the PC3 case for this 3-band combination was submitted to this meeting in R4-2521987.

- Text proposal

< Start of Text Proposal>

# 6	High Power UE CA of 3 bands DL with 1 band UL and 2 bands UL

## 6.x	CA_ n40-n41-n77

### 6.x.1	Configurations

Table 6.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n40A-n41A-n77A CA_n40A-n41B-n77A | n407,9 n417,9 n777,9 CA_n40A-n41A7,9 CA_n40A-n77A7,9 CA_n41A-n77A7,9 | n40 | n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |

NOTE 7:	Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination

NOTE 9:	Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination

### 6.x.2	Maximum output power

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n40-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |
| CA_n40-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |
| CA_n41-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

Table 6.X.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 1.5 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n40-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |
| CA_n40-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |
| CA_n41-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.x.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

For single UL n40, n41 and n77, there is neither IMD issue nor harmonic mixing issue, hence there is no MSD for the each of the three single UL for both PC2 and PC1.5.

For the three 2-band UL cases, IMDs exist:

-UL CA_n40A-n41A has IMD5, IMD6 and IMD7 into n77, and the PC3 IMD5 MSD is 5.2dB.

-UL CA_n40-n77A has IMD4, IMD6 and IMD7 into n41, and the PC3 IMD4 MSD is 13.1dB

-UL CA_n41-n77A has IMD4, IMD6 and IMD7 into n40, and the PC3 IMD4 MSD is 15dB

Therefore, the MSD values for HPUE need to be defined.

#### 6.x.3.1	Power class 2 case a, b, c, d

Based on the approved WF on MSD simplification for HPUE [4],

- The PC2 MSD value corresponding to IMD5 due to UL CA_n40A-n41A falls into n77 DL is calculated by adding 7.1dB to the PC3 MSD value, resulting in 12.3dB.

- The PC2 MSD value corresponding to IMD4 due to UL CA_n40A-n77A falls into n41 DL is calculated by adding 9dB to the PC3 MSD value, resulting in 22.1dB.

- The PC2 MSD value corresponding to IMD4 due to UL CA_n41A-n77A falls into n40 DL is calculated by adding 9dB to the PC3 MSD value, resulting in 24dB.

Table 6.x.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| n40-n41-n77 | n40 | 2305 | 10 | 50 | 2305 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3445 | 12.3 | TDD | IMD5 X |
|  | n40 | 2305 | 10 | 50 | 2305 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2600 | 22.1 | TDD | IMD4 X |
|  | n77 | 3605 | 10 | 50 | 3605 | N/A | TDD | N/A |
|  | n40 | N/A | 10 | N/A | 2350 | 24 | TDD | IMD4 X |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | TDD | N/A |
|  | n77 | 3725 | 10 | 50 | 3725 | N/A | TDD | N/A |

NOTE X:	This band is subject to IMD6 also which MSD is not specified.

#### 6.x.3.2	Power class 1.5 case a, b, c, d, e

Based on the approved WF on MSD simplification for HPUE [4],

- The PC1.5 MSD value corresponding to IMD5 due to UL CA_n40A-n41A falls into n77 DL is calculated by adding 15.8dB to the PC3 MSD value, resulting in 21dB.

- The PC1.5 MSD value corresponding to IMD4 due to UL CA_n40A-n77A falls into n41 DL is calculated by adding 18dB to the PC3 MSD value, 13.1dB, resulting in 31.1dB.

- The PC1.5 MSD value corresponding to IMD4 due to UL CA_n41A-n77A falls into n40 DL is calculated by adding 18dB to the PC3 MSD value, 15dB, resulting in 33dB.

Table 6.x.3-2: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| n40-n41-n77 | n40 | 2305 | 10 | 50 | 2305 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3445 | 5.2 | TDD | IMD5 X |
|  | n40 | 2305 | 10 | 50 | 2305 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2600 | 13.1 | TDD | IMD4 X |
|  | n77 | 3605 | 10 | 50 | 3605 | N/A | TDD | N/A |
|  | n40 | N/A | 10 | N/A | 2350 | 15 | TDD | IMD4 X |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | TDD | N/A |
|  | n77 | 3725 | 10 | 50 | 3725 | N/A | TDD | N/A |

NOTE:	This band is subject to IMD6 also which MSD is not specified.

### 6.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

< End of Text Proposal>

- References

[1] 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[2] 	RP-241679_New WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR SUL (supplementary uplink)

[3] R4-2521987, TP to TR 38.719-03-01 for adding CA_n40A-n41A-n77A

[4] R4-2505143, WF on MSD simplification for HPUE

[5] R4-2514599, Draft CR for 38.101-1 on enabling HPUE support for band combinations without HPUE applicability notes in clause 5