3GPP TSG-RAN WG4 Meeting #117				                           R4-2522044

Dallas, USA, Nov. 17-21, 2025

Source:	Sony

Title:	Further views on waveform evaluation PA modelling of 6GR

Agenda item:	8.2.1

Document for:	Approval

# Background

In RAN4#116bis, initial discussion on system parameter for 6GR from RAN4 side was carried out [1] and a WF was agreed [2].  In this contribution, we share our further views on on waveform evaluation and PA modelling of 6GR based on the scope that was agreed in the WF.

# Framework study for waveform

For waveform study, the following agreement has been made in RAN4#116bis:

- The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1

- To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects

  - Tx assumption including PA model

  - Related RF requirements which should be taken into consideration

  - Both UL and DL are considered

  - Take the candidate waveforms identified/discussed in RAN1 as the baseline

  - Identify the evaluation metric, e.g. net gain

  - Implementation constraints

- Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs

  - Evaluation cases could be selected step-by-step, based on RAN1 progress

  - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement’

In our view, net gain can be used as the primary metric on the evaluation of waveform, but do not preclude other parameters like cubic metric (CM) at this stage, where net gain defined as:

Net gain = MPR reduction + link loss (SINR) relative to the reference @x% BLER

For the MPR reduction evaluation, the relevant RF requirements, e.g., SEM, ACLR, EVM needs to be considered where 5G NR requirements can be taken as a starting point for existing modulations. In addition, the spectral efficiency or spectral utilization is to be used as a side condition to also be considered when evaluating the waveform to reduce the PAPR.

Proposal 1: Adopt net gain can be used as the primary metric on the evaluation of waveform, but do not preclude other parameters like cubic metric (CM) at this stage, where net gain defined as:

Net gain = MPR reduction + link loss (SINR) relative to the reference @x% BLER

Proposal 2: For the MPR reduction evaluation, the relevant RF requirements, e.g., SEM, ACLR, EVM needs to be considered where 5G NR requirements can be taken as a starting point for existing modulations.

Proposal 3: Considering the spectral efficiency or spectral utilization as a side condition, e.g., the spectral utilization must be above y%, when evaluating the feasibility of waveform to reduce the PAPR.

# PA modelling for waveform

Considering the diverse device types that 6GR needs to support, the PA models may differ between different device types. For example, Massive IoT PA may be optimized against a narrower bandwidth, while a smartphone PA needs to cover a much wider range of bandwidths. In addition, more advanced semiconductor technologies, e.g., GaN PA, may be considered for FWA/CPE devices, which focus on high performance and high-power applications. Therefore, it is proposed that RAN4 should consider different PA models for different device types.

Proposal 4: RAN4 should consider different PA models for different device types.

In addition, the following agreement has been made in RAN4#116bis:

- Consider PA modelling at least for the following aspects for RAN4 discussion

  - Memory effects for UE supporting larger CBW

  - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3)

  - Calibration conditions

  - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc.

  - PA models for different device types, handheld UE is prioritized

  - Applicable requirements

    - 5G-A requirements as starting point

  - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

In TR38.803, multiple modelling methods have been captured already in 5G NR study, which covers mainstream modelling approach in our view. One noticeable difference is whether to consider memory effect of PA or only simple AM-AM/AM-PM effect. The memory effect is particular critical for wideband simulation (e.g., > 100 MHz in FR1) as and one of potential differences between 6GR and 5G NR is the need to support even wider bandwidth, especially in FR-1 where the relative bandwidth is large. Moreover, considering the various usages of PA modelling in RAN4 study, for example, to evaluate the gain on AI based DPD and etc., it is also necessary to consider the memory effect of PA modelling.

Observation 1: Memory effect in PA modelling would be useful to evaluate the signal with wide bandwidth (> 100 MHz in FR1) and to evaluate the gain of DPD during the AI study.

It is also proposed to consider the state of art PA implementations when determining the proper PA model to avoid over-pessimistic results from the evaluation. In particular, some pre-distortion mechanisms which may already be implemented in device today can also be considered. To facilitate the discussion, it is also proposed that RAN4 focus on the PA model for handheld devices around 7GHz as starting point to carry out the work.

Proposal 5: RAN4 to study PA model with memory effect, focus on around 7GHz for smartphone application as starting point, with consideration of state of art PA implementation with pre-distortion technics if feasible.

# Conclusion

In this contribution, we share our initial view on the waveform evaluation and PA modelling of 6GR from the RAN4 perspective, where the following observations and proposals are given:

Observation 1: Memory effect in PA modelling would be useful to evaluate the signal with wide bandwidth (> 100 MHz in FR1) and to evaluate the gain of DPD during the AI study.

Proposal 1: Adopt net gain can be used as the primary metric on the evaluation of waveform, but do not preclude other parameters like cubic metric (CM) at this stage, where net gain defined as:

Net gain = MPR reduction + link loss (SINR) relative to the reference @x% BLER

Proposal 2: For the MPR reduction evaluation, the relevant RF requirements, e.g., SEM, ACLR, EVM needs to be considered where 5G NR requirements can be taken as a starting point for existing modulations.

Proposal 3: Considering the spectral efficiency or spectral utilization as a side condition, e.g., the spectral utilization must be above y%, when evaluating the feasibility of waveform to reduce the PAPR.

Proposal 4: RAN4 should consider different PA models for different device types.

Proposal 5: RAN4 to study PA model with memory effect, focus on around 7GHz for smartphone application as starting point, with consideration of state of art PA implementation with pre-distortion technics if feasible.

# References

- R4-2514508		Topic Summary for [116bis][101] 6G system parameter	Moderator (Huawei)

- R4-2514618		WF on [116bis][101] 6G system parameter	Huawei