3GPP TSG-RAN WG4 Meeting #117				                           R4-2522045

Dallas, USA, Nov. 17-21, 2025

Source:	Sony

Title:	views on modulation study of 6GR

Agenda item:	8.2.2

Document for:	Approval

# Background

In RAN4#116bis, initial discussion on system parameter for 6GR from RAN4 side was carried out [1] and a WF was agreed [2].  In this contribution, we share our further views on the modulation study of 6GR based on the scope that was agreed in the WF.

# Discussion

In RAN4#116bis, the following agreements have been made for modulation evaluation from RAN4 side:

- RAN4 evaluation work focus on the feasibility study and RF requirements impact

- To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1

  - Identify the main affected requirements for modulation evaluations

    - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions.

  - Align on the evaluation assumptions.

  - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation.

  - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed.

  - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1.

- Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations

  - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies.

  - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

Comparing to 5G NR, one of the new modulation orders is on 1K QAM in UL direction. Such a high order modulation scheme requires extraordinary linearity from the PA, for example GaN based PA, with potential assistance from DPD technics. Therefore, RAN4 may consider already based on 5G NR system assumption to investigate from PA and transmitter perspective, what could be the achievable EVM for 1K QAM in UL.

Proposal 1: RAN4 to study the performance and feasibility to support 1K QAM in UL based on 5G NR system parameters as starting point, with consideration of state of art PA implementation and DPD technic.

In addition, modulation is also one of the key parameters that needs to be considered under a scalable design for diverse device types.

Proposal 2: RAN4 to study the modulation as part of scalable design, to determine the proper order of modulation to be supported by each device types.

- For massive IoT devices, considering the device complexity and the demand spectral efficiency of IoT deployment, the modulation order needs to be carefully selected to meet the target data rate without adding additional complexity to device implementations. Assuming a target data rate for massive IoT in 6GR would be in the range between 5-10 Mbps, with a 3MHz UE CBW in the UL direction, it can support a peak data rate around above 5 Mbps with 64QAM (assuming 5G NR MCS). To support a higher modulation order, it may require improved linearity in the device transmitter and a more advanced receiver in the DL direction, which may add additional complexity for IoT devices. Thus, it is proposed that 6G massive IoT supports up to 64 QAM.

- For broadband devices with limited form factor, e.g., a smartphone or glass type XR devices, it is proposed to adopt a similar modulation order as in 5G NR, with 256 QAM UL/1024 QAM DL in all frequency ranges. A higher order can be considered if a clear demand for a higher data rate and hardware feasibility can be identified.

- For ultra-broadband devices, e.g., FWA/CPE devices, due to the demand for higher data rates and the possibility of supporting higher-performing transceiver designs with less constrained power consumption and form factors, it is proposed to consider 1024 QAM UL/1024 QAM DL in FR1 and “FR3”. While, in FR2, we see 256QAM in UL and 1024QAM in DL should be feasible.

Proposal 3: RAN4 considering adopt the adopt the following assumption for modulation as baseline for further study in 6GR: 6G massive IoT can support up to 64 QAM. For broadband (e.g., smartphone, glass type XR devices) and ultra broadband devices (e.g., FWA/CPE), 256 QAM UL/1024 QAM DL and 1024 QAM UL/1024 QAM can be considered in 6GR, respectively.

# Conclusion

In this contribution, we share our initial view on the modulation of 6GR from the RAN4 perspective, where the following observations and proposals are given:

Proposal 1: RAN4 to study the performance and feasibility to support 1K QAM in UL based on 5G NR system parameters as starting point, with consideration of state of art PA implementation and DPD technic.

Proposal 2: RAN4 to study the modulation as part of scalable design, to determine the proper order of modulation to be supported by each device types.

Proposal 3: RAN4 considering adopt the adopt the following assumption for modulation as baseline for further study in 6GR: 6G massive IoT can support up to 64 QAM. For broadband (e.g., smartphone, glass type XR devices) and ultra broadband devices (e.g., FWA/CPE), 256 QAM UL/1024 QAM DL and 1024 QAM UL/1024 QAM can be considered in 6GR, respectively.

# References

- R4-2514508		Topic Summary for [116bis][101] 6G system parameter	Moderator (Huawei)

- R4-2514618		WF on [116bis][101] 6G system parameter	Huawei