3GPP TSG-RAN WG4 Meeting #117				                              R4-2522050

Dallas, USA, Nov. 17-21, 2025

Source:	Sony

Title:	Further RRM consideration on 6G Massive IoT

Agenda item:	8.6

Document for:	Approval

# Background

In RAN4#116bis the 6GR study on RRM was carried out [1] and the WF was agreed [2]. In this contribution, we share our further views on the RRM study.

# RRM framework

For 6GR design, it targets a scalable and forward-compatible design for diverse device types [3]. In particular, the massive IoT device is among the devices that will be supported in 6GR. It is likely that 6G massive IoT will become the device type with the lowest capability among all devices, which can further determine a common but minimum set of features for 6GR.

The 6G massive IoT aims to achieve reduced complexity and improved energy efficiency compared to legacy IoT devices, thereby enhancing the economies of scale and sustainability of the 6G system. To achieve this goal, RRM is a critical part that needs to be considered in the 6G era for massive IoT devices. Based on lessons learned from legacy IoT devices, a significant portion of the memory footprint of a cellular IoT chipset is occupied by storing measurement results from RRM, and frequent searching/measurement/reporting consumes a significant amount of power for an IoT device. Therefore, a proper RRM framework considering different device type implementation, especially the limited UE capabilities in 6G massive IoT, needs to be studied.

Observation 1: For legacy IoT devices, a significant part of the memory footprint is occupied by storing measurement results, and considerable power consumption is incurred during searching/measurement/reporting.

Proposal 1: RAN4 study the RRM framework, which can be scalable among different device types, with consideration of the different implementation assumptions.

# MG-related issues

Based on the agreed WF[2], reducing the number of measurement gap (MG) patterns is among the most popular options in the RRM study for 6GR. Currently, over 20 measurement gap patterns are defined in the RAN4 spec, but only a limited set of these patterns are commonly used in the field, as we understand it. Therefore, for massive IoT devices or all device types, a reduced number of measurement gap patterns could provide further benefit in reducing device complexity.

Proposal 2: RAN4 can also study methods to reduce the number of measurement gap patterns and eliminate possible redundant measurement results if identified.

In addition, gapless measurements were proposed as one of the directions to be studied in 6GR. In our understanding, a gapless measurement implies that the UE has an available RF chain to perform the measurement simultaneously with data transmission/reception. Although this could be feasible for mobile broadband devices where multiple RF chains are needed, it is questionable whether such a feature would be feasible for IoT devices. Massive IoT devices are likely to have a single RF chain for each sub-frequency range/superband and do not support carrier aggregation. In addition, it is preferred to always have a measurement gap configured for IoT devices to reduce the memory dedicated to measurement. Therefore, these aspects need to be investigated as well.

Observation 2: It is preferred to always have a measurement gap configured for IoT devices, to reduce the required memory that is needed dedicated for measurement.

Proposal 3: For the gap-less measurement study, different UE implementations for different device types, e.g., number of RF chains, need to be taken into account.

Other potential enhancements to MG-related techniques include unified MG across different features to reduce the overall number of MG configurations, and unified MG configuration and scheduling to minimize signaling for MG configuration.

Proposal 4: RAN4 to also study unified MG with different features as well as cross different configurations/scheduling in the 6GR RRM study.

# Mobility-related issues

In our view, mobility is also an area that can be simplified in 6GR RRM design, with consideration of different device types. For example, many massive IoT devices operate in stationary cases, such as surveillance cameras and smart meters. Therefore, considering measurements (e.g., cell searching) and reporting reductions for such types of devices could be studied. It is worth mentioning that the detection and measurement of neighboring cells place a heavy burden on IoT device implementations.

Observation 3: Detection and measurement of neighbouring cells place a heavy burden on IoT device implementations.

In addition, even in a mobile scenario, some use cases may not even require seamless handover. In those cases, connected mode mobility may not even be required, and Idle mode mobility can be taken as a baseline. Similar design considerations were taken into account for the early releases of NB-IoT.

Observation 4: For stationary or mobile use cases that do not require seamless handover, measurements and reporting relaxations need to be studied, and Idle mode mobility can serve as a baseline.

Proposal 5: RAN4 should study RRM relaxation and simplification for 6G massive IoT, comparing them with legacy IoT devices to reduce device complexity and improve network/device energy efficiency.

# Energy efficiency related issues

From an energy-saving perspective, we see the benefit of supporting legacy power-saving mechanisms in 6GR as well, for example, PSM/MICO, eDRX, LP-WUS, and others. From the RRM perspective, the measurement relaxation defined for 5G NR could also be considered in the 6G RRM design, especially with a scalable framework, where the measurement occasion (MO) can be configured in a scalable manner depending on the device type. Some other possible directions here could include, for example, allowing more RRM measurements to be offloaded to a low-power receiver (LR), to enable more energy savings.

Proposal 6: RAN4 shall study the power-saving/energy-efficiency improvement mechanism from the RRM perspective, including legacy techniques, e.g., MO relaxation, and shall further investigate whether more RRM measurements can be offloaded to LR.

# Conclusion

In this contribution, we share some further thoughts and directions for RRM design in 6GR, particularly concerning the 6G massive IoT device type. The following observations and proposals are given:

Observation 1: For legacy IoT devices, a significant part of the memory footprint is occupied by storing measurement results, and considerable power consumption is incurred during searching/measurement/reporting.

Observation 2: It is preferred to always have a measurement gap configured for IoT devices, to reduce the required memory that is needed dedicated for measurement.

Observation 3: Detection and measurement of neighbouring cells place a heavy burden on IoT device implementations.

Observation 4: For stationary or mobile use cases that do not require seamless handover, measurements and reporting relaxations need to be studied, and Idle mode mobility can serve as a baseline.

Proposal 1: RAN4 study the RRM framework, which can be scalable among different device types, with consideration of the different implementation assumptions.

Proposal 2: RAN4 can also study methods to reduce the number of measurement gap patterns and eliminate possible redundant measurement results if identified.

Proposal 3: For the gap-less measurement study, different UE implementations for different device types, e.g., number of RF chains, need to be taken into account.

Proposal 4: RAN4 to also study unified MG with different features as well as cross different configurations/scheduling in the 6GR RRM study.

Proposal 5: RAN4 should study RRM relaxation and simplification for 6G massive IoT, comparing them with legacy IoT devices to reduce device complexity and improve network/device energy efficiency.

Proposal 6: RAN4 shall study the power-saving/energy-efficiency improvement mechanism from the RRM perspective, including legacy techniques, e.g., MO relaxation, and shall further investigate whether more RRM measurements can be offloaded to LR.

# References

- R4-2514512		Topic Summary for [116bis][105] 6G RRM	Moderator (Apple)

- R4-2514644		WF on [116bis][105] 6G RRM	Apple

- RP-252912		Revised SID: Study on 6G Radio		NTT DOCOMO, CMCC, AT&T, Vodafone.