3GPP TSG-RAN WG4 Meeting #117				                              R4-2522051

Dallas, USA, Nov. 17-21, 2025

Source:	Sony

Title:	Considerations on 6GR spectrum sharing

Agenda item:	8.9

Document for:	Approval

# Background

In RAN4#116bis the 6GR study on spectrum sharing was carried out [1] and the WF was agreed [2]. In this contribution, we share our further views on the spectrum sharing based on the agreed WF.

# Discussion

In the agreed WF from RAN4#116bis, the RAN4 spectrum-sharing study focuses on the MRSS between 5 G and 6 G. However, there was no mention of the issue between 6G and 4G. In our understanding, the MRSS, which offers a more dynamic spectrum-sharing mechanism, is limited to 5G-6G to support an efficient/low-overhead scheme.

Observation 1: For the MRSS design, it is only about considering between the 5G and 6G systems.

However, this does not mean that spectrum sharing between 4G and 6 G is not needed. Many 4G services, especially IoT services that operate in low bands, are expected to last for decades. It can be expected that many 4G services would still operate when 6G roll out. Therefore, coexistence between 4G and 6G, especially between 4G IoT (cat-M1/NB-IoT) and 6G, needs to be addressed.

Observation 2: Although there is no need to consider the MRSS between 4G and 6G, basic coexistence between 4G (especially 4G IoT) and 6G must be guaranteed in the 6G design.

Although in the latest version of  6GR SID, it states that the” Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary “, RAN plenary already agreed the text proposal below and captured in the TR38.914 that “the 6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” as showed below:

Observation 3: Although the current version of 6G SID states that inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from the RAN plenary, the RAN plenary has agreed that 6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration, as captured in TR38.914.

Therefore, with the above clarification, we propose that RAN4 include coexistence between 6G and 4G IoT (NB-IoT and eMTC) via semi-static configuration in the spectrum-sharing study.

Proposal 1: RAN4 shall study and support the coexistence between 6G and 4G IoT (NB-IoT and eMTC) via semi-static configuration as per the RAN agreement.

Although it is still too early to assume the overall PHY design of 6GR, we assume it will be compatible or similar with 5G NR in terms of numerology and waveform. Therefore, we expect a similar coexistence mechanism between 5G-NR and 4G IoT can be applied to the semi-static coexistence between 6GR and 4G IoT. One noticeable impact from the network perspective is that a 7.5 kHz shift is needed for 6GR, due to the avoidance of a DC location for sub carrier placement in the LTE design. This would be the same as used in the coexistence between 5G NR and 4G.

Proposal 2: A 7.5 kHz shift is needed for 6GR coexistence with 4G IoTs.

On the other hand, such a shift is not needed between 5GNR and 6G MRSS design, assuming 6G would allocate the center of the subcarrier to the DC location as in 5G NR.

Proposal 3: A 7.5 kHz shift is not needed for 6GR MRSS with 5G NR

# Conclusion

In this contribution, we share some further thoughts on spectrum sharing, especially the importance between 6GR and 4G IoT, where the following observations and proposals are given:

Observation 1: For the MRSS design, it is only about considering between the 5G and 6G systems.

Observation 2: Although there is no need to consider the MRSS between 4G and 6G, basic coexistence between 4G (especially 4G IoT) and 6G must be guaranteed in the 6G design.

Observation 3: Although the current version of 6G SID states that inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from the RAN plenary, the RAN plenary has agreed that 6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration, as captured in TR38.914.

Proposal 1: RAN4 shall study and support the coexistence between 6G and 4G IoT (NB-IoT and eMTC) via semi-static configuration as per the RAN agreement.

Proposal 2: A 7.5 kHz shift is needed for 6GR coexistence with 4G IoTs.

Proposal 3: A 7.5 kHz shift is not needed for 6GR MRSS with 5G NR

# References

- R4-2514515		Topic Summary for [116bis][108] 6G spectrum sharing	Moderator (CMCC)

- R4-2514646		WF on [116bis][108] 6G spectrum sharing	CMCC