3GPP TSG-RAN WG4 Meeting #117 	R4-2522062

Dallas, US, Nov 17th – 21st, 2025

Agenda Item: 6.3.2

Source: MediaTek inc.

Title: Discussion on performance requirements for low-band CA switching

Document for: Discussion

# Discussion

The main objectives of the work item are to introduce the physical procedures and requirements to enable low band CA via switching, detailed below [1]:

### Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4]

RAN4 has already completed RRM aspects related to the low band CA operation and the agreements have been captured in the WF [2].

## Discussion on test cases

Issue 2-3-1: detailed TC list Agreement: TC Index; Requirement; RS used for test; Target Cell for testing; Support this TC; Leading Company. 1; Intra-frequency L3 measurement for activated SDL SCell or PCell; SSB; Both PCell and SCell in the same TC PCell and SCell can be configured with different events, details can be FFS. PCell is in partially overlapped case. SCell is in fully overlapped case.; Ericsson. 2; Deactivated SDL SCell measurement and interruption requirement; SSB; Verify measurement requirement on SDL SCell. And in the same TC verify interruption requirement to the PCell.; MTK. 3; SDL SCell activation/deactivation; SSB(for sync and L1-RSRP) and CSI-RS for CQI; SDL SCell. Only MAC CE based SCell activation is verified.; Nokia. 4; BFD/CBD for SDL SCell; Option 1: SSB (Apple) Option 2: CSI-RS Option 3: both; Option 1: SCell only (Apple) Option 2: Both PCell and SCell (OPPO, HW, ZTE, Ericsson) [Ericsson]: If option 2 is adopted, test PCell/SCell in the different test.; FFS whether this TC shall be introduced.. 6; PL-RS switch delay for PCell; Option 1: SSB (Apple) Option 2: CSI-RS Option 3: both; FDD PCell (Apple, OPPO, ZTE, Ericsson); FFS whether this TC shall be introduced.. 7; L1-RSRP for SDL SCell; SSB; SCell only; Huawei. 8; RLM requirement for PCell; SSB; FDD PCell; OPPO.

On TC 4 and TC 6: the WI impacts the following RRM aspects:

| Impacted sections | Reason for update | Whether TC is needed |
| --- | --- | --- |
| 8.1.2.2 | Update the P scaling factor. | Yes |
| 8.1.3.2 | Update the P scaling factor. | No, redundant |
| 8.2.2.2.3 | Adding interruption ratio requirements | Yes |
| 8.3.2 | Adding the UE processing delay for activating the LBCA-SwitchingPattern | Yes |
| 8.3.3 | Adding the UE processing delay for deactivating the LBCA-SwitchingPattern | No, redundant |
| 8.3.4 | Adding the UE processing delay for activating the LBCA-SwitchingPattern | No, redundant |
| 8.5.2.2 | Update the P scaling factor. | No, redundant |
| 8.5.3.2 | Update the P scaling factor. | No, redundant |
| 8.5.5.2 | Update the P scaling factor. | No, redundant |
| 8.5.6.2 | Update the P scaling factor. | No, redundant |
| 8.10.3 | Update the note for first SSB for TCI state | No need |
| 8.10.5 | Update the note for first SSB for TCI state | No need |
| 8.14.3 | Adding Kp scaling factor for Path Loss requirements | Yes |
| 8.15.3 | Update the note for first SSB for TCI state | No need |
| 9.2.5 | Updating the Kp scaling factor for Intra-f | No, redundant |
| 9.5.4 | Updating the P scaling factor for L1-RSRP |  |
| 9.8.4 | Updating the P scaling factor for L1-SINR |  |

Based on the changes to the RRM requirements, a single test case can be defined for each of the following:

- A test case to cover the impact on P scaling factor.

- A test case to cover the measurement on deactivated SCell (SDL).

- A test case to cover the processing delay for activation/deactivation.

- A test case to cover the impact on the updated Kp scaling factor.

Therefore, we don’t see a need to define test cases to cover BFD/CBD for SDL SCell and for PL-RS switch delay for PCell.

- RRM not to define test cases in LB-CA for the following RRM aspects: (i) a test case to cover BFD/CBD for SDL SCell, (ii) a test case to cover PL-RS switch delay for PCell.

Furthermore, on TC 2 the testing requirements to cover deactivated SCell (SDL) measurement and interruption requirements. However, based on legacy tests that cover the item of deactivated SCell measurements, there are the following legacy test cases:

| TC number | Title and purpose |
| --- | --- |
| A.6.5.2.1 | Interruptions during measurements on deactivated NR SCC in FR1 |
| A.6.6.19.4 | Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX |

It should be noted that the TC A.6.5.2.1 covers interruption requirements only, while TC A.6.6.19.4 covers deactivated SCell measurement delay and interruption requirements, which is part of NCSG pattern (i.e. VIL). Therefore, we would like to discuss with other companies on whether there is critical need to include deactivated SCell measurement in this TC 2? If companies are interested in including the deactivated SCell measurements, then companies to discuss what are the configuration parameters to define for this TC.

- RRM to discuss whether it is critical to verify deactivated SCell measurement requirements in TC 2?

# Summary

In summary:

Proposal 1: RRM not to define test cases in LB-CA for the following RRM aspects: (i) a test case to cover BFD/CBD for SDL SCell, (ii) a test case to cover PL-RS switch delay for PCell.

Proposal 2: RRM to discuss whether it is critical to verify deactivated SCell measurement requirements in TC 2?

# References

- RP-243317, “New WID on low band carrier aggregation via switching”, Madrid, Spain, Dec 2024.

- R4-2514794, “WF for [116bis][202] NR_LBCA_Sw_RRM”, Apple, Prague, CZ, Oct 2025.