# 3GPP TSG-RAN WG4 Meeting #117	R4-2522173

# Dallas, USA, Nov. 17-21, 2025

Title: 	Further discussion on parameters for AAS BS operating in bands below 1 GHz

Agenda Item:	11.4

Source: 	Huawei, HiSilicon, Vodafone, Telefónica, Deutsche Telekom, BT plc, Orange

Document for:	Discussion

# 1	Introduction

ECC PT1 has submitted an LS in R4-2513058 [1] on the topic of parameters for AAS BS operating in bands below 1 GHz. The related co-existence work for AAS BS below 1 GHz will be carried out in ECC PT1 and captured in a new ECC Reports that would form basis for the update of the regulatory technical conditions in ECC Decision (15)01 [2], ECC Decision (09)03 [3], and ECC Decision (06)13 [4].

ECC PT1 has informed that it has started work towards authorisation of AAS BS in the following frequency bands:

- 694-791 MHz (700 MHz),

- 791-862 MHz (800 MHz),

- 880-960 MHz (900 MHz).

ECC PT1 expects that AAS BSs in the three bands under study would be deployed in existing sites and hence the deployment parameters would be similar to non-AAS. However, ECC PT1 does not have information about the technical characteristics of AAS BSs in these frequency bands, and asks 3GPP for guidance on the following items:

- Unwanted emissions characteristics of AAS BSs operating in the 700 MHz, 800 MHz and 900 MHz bands.

- Whether 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration.

- A model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz, both in-band and out of band for frequency bands under consideration.

ECC PT1 asks 3GPP to provide a response for the next ECC PT1 meeting scheduled for 12-16 January 2026. The LS and feedback requested by ECC PT1 was initially discussed during RAN4#116-bis meeting (Prague) under [324] thread (topic summary in R4-2514447 [5]), which resulted in a Way Forward in R4-2515071 [6]. WF agreements were extracted below:

# Plan for replying to CEPT on parameters for AAS BS operating in bands below 1 GHz Agreement: The current work on replying to CEPT is to be conducted according to the following plan: - Send the first reply LS from RAN#110 to ECC PT1 with answers to question(s) as agreed in RAN4 and scheduled second reply LS for remaining question(s). - Seek guidance at RAN#110 how to leverage 6G studies and their TU’s, or ask for dedicated TU allocation for remaining question(s). # Answers to Questions from CEPT # Question #1: Unwanted emissions characteristics of AAS BSs operating in the 700 MHz, 800 MHz and 900 MHz bands Agreement: FFS on the following answer is considered to ECC PT1 Question #1: Unwanted emissions characteristics of AAS BS operating in the 700 MHz, 800 MHz and 900 MHz bands, are already specified since Release 15 in AAS BS specifications (i.e., TS 37.105, TS 37.145-1, TS 37.145-2), and NR BS specifications (i.e., TS 38.104, TS 38.141-1, TS 38.141-2). However, updates to these specifications may still be considered in the future if relevant spatial unwanted emission data or measurement results for AAS BS operating below 1 GHz reveal any issues. # Question #2: Whether 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration Agreement: FFS on the following answer is considered to ECC PT1 Question #2: 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration. It should be noted, however, that these requirement levels were originally derived based on legacy non-AAS base station specifications and may be revisited.

In this contribution we provide further discussion on the questions received in LS from CEPT ECC PT1 on AAS BS operation in low frequency bands below 1 GHz.

# 2	Discussion

# 2.1	Unwanted emissions characteristics of AAS BSs operating in the 700 MHz, 800 MHz and 900 MHz bands

The first question in ECC PT1 LS [1] was related to the unwanted emissions characteristics of AAS BSs operating in the 700 MHz, 800 MHz and 900 MHz bands.

RAN4 has discussed in Q4 2024 whether a low frequency boundary should be specified for the BS RF requirements in AAS BS and NR BS specifications. Specifically, RAN4 considered introduction of a clarification note for AAS BS lower operating band frequency limit in the following form:

“Operating bands below [AAS_low_limit] MHz are not considered as supported by this specification. Operating bands below [AAS_low_limit] MHz are still applicable for protection requirements (i.e., Tx spurious co-location and co-existence, Rx blocking co-location).”

That topic was discussed and closed at RAN4#113. There was no agreement on a lower bound for applicability of the specifications of RF requirements for AAS BS and NR BS. Therefore, the RF requirements – in particular the OTA ACLR limits, the OTA operating band unwanted emissions limits, and the OTA spurious emissions limits – in TS 37.105, TS 37.145-2, TS 38.104, TS 38.141-2 are also valid for operating bands below 1 GHz.

Since Rel-15, there are no specific frequency limits nor restrictions for unwanted emissions characteristics applicability of AAS BS and NR BS operating in the 700 MHz, 800 MHz and 900 MHz bands.

AAS BS unwanted emissions limits are specified as TRP based on the analysis at 2 GHz, which showed for an AAS BS network where wanted UE location (and potentially victim system location) are random and varying, the average interference level is directly proportional to the average spatial power distribution, which will take the shape of the cell. The same is true for non-AAS systems. Whatever the correlation factor of the emission, the interference is comparable to that of the static non-AAS system and hence the OTA TRP limits are equivalent to the conducted limits.

Conducted limits for sub-1 GHz are stable and have been used for many years. Hance, those can be regarded as also suitable for AAS BS.

Low band AAS BS antennas due to the restrictions in size, tend to have fewer narrow beams and lower beam steering capabilities. As such, the spatial power distribution, if anything, will approach the average state more rapidly than higher frequency systems so the assumption for unwanted emissions is valid below 1 GHz.

Proposal 1: Clarify in the LS to RAN and to CEPT ECC PT1, that the unwanted emissions characteristics of AAS BS and NR BS operating in bands below 1 GHz (including 700 MHz, 800 MHz and 900 MHz bands) are captured since Rel-15 in AAS BS specifications (TS 37.105, TS 37.145-1, TS 37.145-2), and NR BS specifications (TS 38.104, TS 38.141-1, TS 38.141-2).

# 2.2	BS RF requirements in TS 37.105 and TS 38.104

The second question in ECC PT1 LS [1] was asking whether 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration. Similar to the discussion in clause 2.1 above, both the 3GPP TS 37.105 and TS 38.104 BS RF specifications were structured in a way that most BS RF requirements are defined in a band agnostic manner, and are equally applicable for AAS BS operation in bands below 1 GHz.

AAS BS requirements can be separated into TRP and directional requirements.

As previously discussed, TRP requirements rely on the average spatial power distribution of the AAS BS and for systems under consideration this power distribution tends towards covering the cell irrespective of the AAS antenna capabilities. As such, the non-AAS conducted limits have been found to be suitable for AAS TRP limits. As sub-1GHz AAS BS systems are if anything less capable than those used for the initial study at 2 GHz, this assumption is still valid and hence AAS TRP requirements are valid below 1 GHz.

Directional requirements are generally based on in-band wanted signal performance. As AAS BS system antenna gain is dependent on implementation (and implementation is limited by frequency) there is no general assumption for the antenna gain and hence the power level of the wanted signals. This is managed by vendor declaration of the direction output power and sensitivity and the coverage range over which they are valid. These declarations will vary based on frequency but the current structure is valid for systems operating below 1 GHz. OTA directional limits are equivalent to the conducted limits plus antenna gain.

Proposal 2: Clarify in the LS to RAN and to CEPT ECC PT1, that 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz, including operating bands in the 700 MHz, 800 MHz and 900 MHz.

# Antenna model and parameters

The third question in ECC PT1 LS [1] was related to the model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz, both in-band and out of band for frequency bands under consideration. In this clause we focus on the first part of the question, i.e., antenna model and related parameters for in-band.

3GPP has produced technical reports with AAS antenna models and parameters for different bands, for own use and on request from ITU WP5D:

- 3GPP TR 38.803, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects.

This TR contains in section 5.2.3.2.4 a model developed to support sub-array antenna geometries. The model was created to model AAS base station operating within the frequency range 1710 to 4990 MHz. Section 5.2.3.2.4 of this TR also contains a parameter set, to be used with the model, applicable to BSs operating in the 1710 to 4990 MHz range.

- 3GPP TR 38.922 NR; Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz.

This TR was created in March 2025 in response to a request from WP5D and is the most recent report in 3GPP with an AAS model and parameters. The model equations (section 7.1.3), and the parameters for the 1710 to 4990 MHz range (section 4.4), are the same as in TR 38.803. TR38.922 also includes guidance on how to model the array antenna pattern outside the wanted channel.

It is expected that the architecture of AAS BS in operating bands below 1 GHz will be similar to the sub-array antenna geometry considered in the TRs above. Therefore, the model in section 7.1 of TR 38.922 could be used by ECC PT1 in their compatibility studies. Section 7.1.2 in TR 38.922 lists the input parameters required to utilize the model.

It is proposed to consider an antenna array architecture as shown in the figure 1 below and the associated parameters in the table 1 below:

Figure 1: Proposed antenna array model in 700/800/900 MHz operating bands

Table 1: Proposed set of parameters for an array model in 700/800/900 MHz operating bands

| Parameter | Value |
| --- | --- |
| Antenna pattern model | Extended AAS subarray model |
| Element gain (dBi) | 5 (with 2dB Ohmic loss) |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) | 2×4 |
| Horizontal/Vertical radiating sub-array or element spacing | 0.5 of wavelength for H, 2.8 of wavelength for V |
| Number of element rows in sub-array | 4 |
| Vertical element separation in sub-array | 0.7 of wavelength for V |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Mechanical down-tilt (degrees) | 6 for urban/suburban, 3 for rural |
| Base station horizontal coverage range (degrees) | ±60 |
| Vertical steering range (degree) | 90-100 |

The model should take into account the maximum likely dimensions for a practical antenna model in the frequency range in question. The large wavelength below 1 GHz is one reason why AAS BS systems have been less popular as to obtain any significant beam forming capability the antenna array is very large. It should be considered that there are practical upper limits based on these physical restrictions (size on tower, wind loading, weight, etc.). Particularly width is seen as more of an issue than height when mounting large antennas. The antenna model described is regarded as reasonable maximum practical implementation for such products.

Proposal 3: Clarify in the LS to RAN and to CEPT ECC PT1, that the antenna model in section 7.1 of TR 38.922 can be used in 700 MHz, 800 MHz and 900 MHz operating bands as baseline for further discussion in RAN4, with the following set of parameters proposed to be used in the model:

| Parameter | Value |
| --- | --- |
| Antenna pattern model | Extended AAS subarray model |
| Element gain (dBi) | 5 (with 2dB Ohmic loss) |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) | 2×4 |
| Horizontal/Vertical radiating sub-array or element spacing | 0.5 of wavelength for H, 2.8 of wavelength for V |
| Number of element rows in sub-array | 4 |
| Vertical element separation in sub-array | 0.7 of wavelength for V |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Mechanical down-tilt (degrees) | 6 for urban/suburban, 3 for rural |
| Base station horizontal coverage range (degrees) | ±60 |
| Vertical steering range (degree) | 90-100 |

# 2.4	Out of band characterisation

The third question in ECC PT1 LS [1] was related to the model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz, both in-band and out of band for frequency bands under consideration. In this clause we focus on the second part of the question, i.e., out of band aspects.

Section 7 of TR 38.922 provides analysis for modelling the antenna pattern that has support to model the array response outside the wanted carrier bandwidth, within adjacent channels using the correlation factor, . The radiation pattern includes a correlation factor ρ that, for frequencies offset from the wanted signal, is in the interval 0≤ρ<1. In particular, clause 7.3.2.1.2 of TR 38.922 includes the discussion on the parameterized piece-wise linear roll-off model for the correlation factor.

Figure 2: Array correlation factor roll-off model [TR 38.922]

The characterisation of the factor as a function of the frequency offset has been discussed in RAN4, as well as other technical bodies on multiple occasions in the past without conclusion. The out of band antenna gain modelling remains an open issue and is FFS. Furter technical work is necessary in RAN4 to derive correlation factor (possibly with the parameterized linear roll-off model) for the specific bands under study in CEPT ECC PT1.

Proposal 4: Clarify in the LS to RAN and to CEPT ECC PT1, that AAS BS model and associated parameters for out-of-band region (of sub-1GHz operation and others) requires more technical work in RAN4, including but not limited to the following aspects:

- Out of band array response for operation in the 700 MHz, 800 MHz and 900 MHz bands,

- Potential technical solution(s) to address the out of band antenna gain modelling for 700/800/900 MHz operating bands, including at least consideration of parameterized linear roll-off model as in TR 38.922.

- Band agnostic solution are not precluded.

# 3	Conclusions

Based on the above discussion, the following proposals were formulated:

Proposal 1: Clarify in the LS to RAN and to CEPT ECC PT1, that the unwanted emissions characteristics of AAS BS and NR BS operating in bands below 1 GHz (including 700 MHz, 800 MHz and 900 MHz bands) are captured since Rel-15 in AAS BS specifications (TS 37.105, TS 37.145-1, TS 37.145-2), and NR BS specifications (TS 38.104, TS 38.141-1, TS 38.141-2).

Proposal 2: Clarify in the LS to RAN and to CEPT ECC PT1, that 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz, including operating bands in the 700 MHz, 800 MHz and 900 MHz.

Proposal 3: Clarify in the LS to RAN and to CEPT ECC PT1, that the antenna model in section 7.1 of TR 38.922 can be used in 700 MHz, 800 MHz and 900 MHz operating bands as baseline for further discussion in RAN4, with the following set of parameters proposed to be used in the model:

| Parameter | Value |
| --- | --- |
| Antenna pattern model | Extended AAS subarray model |
| Element gain (dBi) | 5 (with 2dB Ohmic loss) |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) | 2×4 |
| Horizontal/Vertical radiating sub-array or element spacing | 0.5 of wavelength for H, 2.8 of wavelength for V |
| Number of element rows in sub-array | 4 |
| Vertical element separation in sub-array | 0.7 of wavelength for V |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Mechanical down-tilt (degrees) | 6 for urban/suburban, 3 for rural |
| Base station horizontal coverage range (degrees) | ±60 |
| Vertical steering range (degree) | 90-100 |

Proposal 4: Clarify in the LS to RAN and to CEPT ECC PT1, that AAS BS model and associated parameters for out-of-band region (of sub-1GHz operation and others) requires more technical work in RAN4, including but not limited to the following aspects:

- Out of band array response for operation in the 700 MHz, 800 MHz and 900 MHz bands,

- Potential technical solution(s) to address the out of band antenna gain modelling for 700/800/900 MHz operating bands, including at least consideration of parameterized linear roll-off model as in TR 38.922.

- Band agnostic solution are not precluded.

According to the WF in [6], it was agreed to send reply LS from RAN#110 to ECC PT1 with first set of replies to questions as agreed in RAN4. It shall be noted, that the LS [1] was also send from PT1 to RAN. In order to proceed efficiently, RAN4 shall provide related technical background and recommended handling to RAN beforehand. Therefore, related Draft LS to RAN was provided in [7].

Proposal 5: Agree on the draft LS to RAN, capturing at least the following information:

- Technical RAN4 background of the ECC PT1 LS,

- Recommended handling of ECC PT1 questions on parameters for AAS BS operating in bands below 1 GHz,

- Possible handling of technical work in RAN4, including recommended approach, if any concluded in RAN4,

- Envisioned timeline required.

Proposal 6: Agree on the draft LS content to be sent from RAN#110 to ECC PT1, with the elements in proposals 1 to 5.

Related draft LS was submitted in [7].

# 4	References

[1]	R4-2513058			Parameters for AAS BS operating in bands below 1 GHz, CEPT ECC PT1, RAN4#116bis

[2]	ECC Decision (15)01 	Harmonised technical conditions for mobile/fixed communications networks (MFCN) in the band 694-790 MHz including a paired frequency arrangement (Frequency Division Duplex 2x30 MHz) and an optional unpaired frequency arrangement (Supplemental Downlink)

[3]	ECC Decision (09)03 	ECC/DEC/(09)03 of 30 October 2009 on harmonised conditions for Mobile/Fixed Communications Networks (MFCN) operating in the band 790-862 MHz

[4]	ECC Decision (06)13 	Harmonised technical conditions for mobile/fixed communications networks (MFCN) including terrestrial IMT systems, other than GSM and EC-GSM IoT, in the bands 880-915/925-960 MHz and 1710-1785/1805-1880 MHz

[5]	R4-2514447			Topic summary for [116bis][324] LS_BDaT, RAN4#116bis

[6]	R4-2515071			Way Forward for [116bis][324] LS_BDaT reply to CEPT, RAN4#116bis

[7]	R4-2522174			Draft LS reply to CEPT ECC PT1 on AAS BS operation in low frequency bands below 1 GHz