3GPP TSG-RAN WG4 Meeting #117	R4-2522198

Dallas, US, Nov 17th – 21st, 2025

Agenda item:	7.4.3.2

Source:	Qualcomm Incorporated

Title: 		RRM enhancements for RedCap UEs

Document for:	Discussion

# Introduction

3GPP has agreed to specify RRM requirements for (e)RedCap UEs as a part of the new WI on NR Radio Resource Management (RRM) Phase 6 [1], where the scope of discussion is limited to:

- RLM/BFD RRM requirements

  - Use R17 RLM/BFD relaxation for legacy UE as a baseline

- Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB)

  - RF requirements, considering example bands: n100, n106, n26, n28 and n85

  - RRM requirements including relaxed RLM/BFD RRM requirements

  - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline

- Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

During the last meeting, RAN4 made quite some progress and achieved many agreements [2].

In this contribution paper, we provide our thoughts on some remaining RRM aspects to be considered in this WI.

# RLM/BFD measurement relaxations

RAN4 already agreed to reuse the existing R17 RLM/BFD relaxation requirements for 2 Rx RedCap UEs, where a relaxation factor, K1 = 4 for Max(TDRX,TSSB) ≤40 ms and K1 = 2 for 40 ms<Max(TDRX,TSSB) ≤80 ms was introduced. These relaxations help reduce power consumption while maintaining link reliability.

For 1Rx RedCap UEs, however, the evaluation periods are already doubled due to the single receiver chain. Introducing further relaxation using the same scaling factors may result in excessive delays, potentially impacting link reliability. We think RAN4 should evaluate whether additional RLM/BFD relaxation is necessary for 1Rx RedCap UEs. If deemed beneficial, a smaller scaling factor should be considered to avoid excessive delays. For example:

- K1 = 2 for Max(TDRX,TSSB) ≤ 40 ms

- K1 = 1.5 for 40 ms < Max(TDRX,TSSB) ≤ 80 ms

Observation 1: RLM/BFD evaluation periods for 1Rx RedCap UEs are inherently longer (double) than those for 2Rx RedCap UEs due to reduced measurement capability.

Proposal 1: RAN4 should evaluate whether RLM/BFD measurement relaxation is necessary for 1Rx RedCap UEs, If yes, a smaller scaling factor should be considered to avoid excessive delays.

R17 introduced the UE feature rlm-Relaxation-r17and network signalling goodServingCellEvaluationRLM and goodServingCellEvaluationBFD. We think the UE feature and the network signallings from R17 can be re-used for RedCap UEs and no need to define new signaling.

Proposal 2: Re-purpose the UE feature rlm-Relaxation-r17and network signalling goodServingCellEvaluationRLM and goodServingCellEvaluationBFD for RedCap UEs. No new signaling is needed.

# Support of less than 5MHz Channel BW

During the last RAN4 meeting, the feasibility of achieving coverage equivalence between 1Rx and 2Rx RedCap UEs was discussed, particularly in the context of hypothetical PDCCH parameters for RLM/BFD in channel bandwidths less than 5 MHz.

In Release 17, coverage gaps between 1Rx and 2Rx RedCap UEs were partially mitigated by allowing higher aggregation levels for 1Rx UEs. However, this approach is not feasible in narrowband configurations—such as those with 15 PRB CORESET#0—due to resource limitations. Specifically, AL16 cannot be supported in such configurations.

Given these constraints, we propose using the same hypothetical PDCCH parameters (e.g., number of control OFDM symbols, aggregation level, bandwidth, REG-to-CCE mapping, etc.) from Release 18 for both 1Rx and 2Rx RedCap UEs. This approach simplifies implementation but may result in 1Rx UEs indicating out-of-sync (OOS) or beam failure detection (BFD) earlier than 2Rx UEs due to their reduced reception capability.

Observation 2: Higher AL of 16 cannot be supported by RedCap UEs with 15PRB CORESE#0

Observation 3: 1Rx RedCap UEs may indicate OOS/BFD earlier than a 2Rx UE

Proposal 3: Use the hypothetical PDCCH parameters (e.g., Number of control OFDM symbols, AL, Bandwidth, REG to CCE mapping etc.) for 15 PRB BW from R18 for both 1Rx and 2Rx RedCap UEs.

# RLM/BFD measurement relaxations with less than 5MHz Channel BW

Both RLM/BFD measurement relaxation and operation in channel bandwidths below 5 MHz independently contribute to long evaluation delays. When combined, these features may result in excessively long delays, which could negatively impact link reliability and responsiveness.

Given this, we believe that the combination of RLM/BFD relaxation with <5 MHz channel bandwidth operation should be deprioritized at the beginning of the work item.

Observation 4: Combining RLM/BFD measurement relaxation with <5 MHz operation may lead to excessively long evaluation delays.

Proposal 4: RAN4 should deprioritize the specification of relaxed RLM/BFD measurements for RedCap UEs operating with 15 PRB channel bandwidth.

# Conclusion

Observation 1: RLM/BFD evaluation periods for 1Rx RedCap UEs are inherently longer (double) than those for 2Rx RedCap UEs due to reduced measurement capability.

Proposal 1: RAN4 should evaluate whether RLM/BFD measurement relaxation is necessary for 1Rx RedCap UEs, If yes, a smaller scaling factor should be considered to avoid excessive delays.

Proposal 2: Re-purpose the UE feature rlm-Relaxation-r17and network signalling goodServingCellEvaluationRLM and goodServingCellEvaluationBFD for RedCap UEs. No new signaling is needed.

Observation 2: Higher AL of 16 cannot be supported by RedCap UEs with 15PRB CORESE#0

Observation 3: 1Rx RedCap UEs may indicate OOS/BFD earlier than a 2Rx UE

Proposal 3: Use the hypothetical PDCCH parameters (e.g., Number of control OFDM symbols, AL, Bandwidth, REG to CCE mapping etc.) for 15 PRB BW from R18 for both 1Rx and 2Rx RedCap UEs.

Observation 4: Combining RLM/BFD measurement relaxation with <5 MHz operation may lead to excessively long evaluation delays.

Proposal 4: RAN4 should deprioritize the specification of relaxed RLM/BFD measurements for RedCap UEs operating with 15 PRB channel bandwidth.

# References

[1] R4-252955, “WID: NR Radio Resource Management (RRM) Phase 6”, Apple, RAN#109

[2] R4-2514821, “WF on NR_RRM_Ph6”, Ericsson, RAN4#116-bis