3GPP TSG-RAN WG4 Meeting #117	R4-2522201

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.13.3

Source: 	Qualcomm Incorporated

Title: 		RRM performance requirements for L1 CLI measurements

Document for:	Discussion

# Introduction

In Release 19, one of the main objectives of the 3GPP work item on the Evolution of NR Duplex Operation: Sub-band Full Duplex (SBFD) [1] is to specify SBFD operation in TDD bands. RAN4 already completed the core part and is now specifying the performance requirements. In this paper, we discuss some of the performance aspects of the work-item.

# Measurement accuracy requirements

RAN4 has been discussing the CSI-RS based L1-RSRP measurement accuracy requirements under DUD configuration for SBFD operation [2]. As shown in the figure below, a CSI-RS resource can be distributed between the two DL subbands. Since RAN4 measurement accuracy requirements for CSI-RS based L1-RSRP are specified based on 48 RB CSI-RS bandwidth, the following three cases were identified:

- Case 1: X1 >= 48 RBs and X2 >= 48 RBs  Both DL subbands individually satisfy the existing min BW req of 48RBs

- Case 2: X1 >= 48 RBs and X2 < 48 RBs  Atleast one DL subband individually satisfy the existing min BW req 48RBs

- Case 3: X1 < 48 RBs and X2 < 48 RBs, (X1 + X2) >= 48 RBs  Both DL subbands combined satisfy the existing min BW req of 48RBs but not individually

During RAN4#114bis, it was agreed that for case 1 and 2, the existing measurement accuracy requirements (i.e., based on 48 RB CSI-RS) apply but for Case 3, the accuracy requirements are relaxed provided that X1 > 24 PRBs or X2 > 24 PRBs. It was specifically noted that only delay requirements may be considered to update if the UE indicates CSI processing time scaling [3].

- Agreement from RAN4#114bis

  - Note: The following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD

  - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2

  - For Case 3

    - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs

    - In the test case, not model the interfering UE.

  - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements

    - Option 1: Add a lower bound of 8ms on top of existing delay requirements.

    - Option 2: other

RAN1 has now finalized this UE capability as a part of the Component 11 of FG 60-1

Component 11 of FG 60-1

Supported CSI timeline, active CSI-RS ports counting and active CSI-RS resource counting for a single CSI-RS resource across two DL subbands

Component 11 candidate values:

- “with factor-of-two relaxation for CSI timeline”,

- “with factor-of-two relaxation for active CSI-RS resource and port counting”,

- “no relaxation”

In our opinion, this should be relevant only for measurement delay determination and not for measurement accuracy determination of CSI-RS based L1-RSRP. This is because there is obvious performance degradation due to loss of processing gain in disjoint CSI-RS RBs as compared to contiguous CSI-RS RBs.

Contiguous CSI-RS RBs (e.g., 48 PRBs in a single subband) allow coherent averaging across adjacent REs, benefiting from better channel estimation due to consistent frequency-domain correlation and resulting in higher measurement accuracy.

Disjoint CSI-RS RBs (e.g., 24 PRBs in each of two subbands), on the other hand, lead to lower measurement accuracy, even if the total number of PRBs is the same because the RBs are spread across separate frequency regions causing a loss of processing gain due to reduced frequency coherence, increased estimation noise and less effective averaging across REs.

This is why, despite receiving CSI-RS over 48 PRBs from two subbands, the measurement accuracy is not equivalent to that of a contiguous 48 PRB configuration. The processing gain is inherently lower due to the fragmented nature of the resource allocation.

So, we support applying accuracy requirements based on 24 PRBs for Case 3, regardless of CSI processing time indication, for the following reasons:

- This approach aligns with the agreement from RAN4#114bis.

- Even if the UE can receive CSI-RS over 48 PRBs from two DL subbands simultaneously, the measurement accuracy is not equivalent to contiguous 48 PRBs due to the loss of processing gain from disjoint CSI-RS resource blocks (e.g., degraded channel estimation performance).

Observation 1: RAN4 has already agreed to specify CSI-RS based L1-RSRP (and L1-SINR) measurement accuracy requirements based on 24 RBs for Case 3.

Observation 2: Even if the UE can receive CSI-RS over 48 PRBs from two DL subbands simultaneously, the measurement accuracy is not equivalent to contiguous 48 PRBs due to the loss of processing gain from disjoint CSI-RS resource blocks (e.g., degraded channel estimation performance).

Proposal 1: Maintain the agreement from RAN4#114bis and specify CSI-RS-based L1-RSRP and L1-SINR measurement accuracy requirements based on 24 RBs for Case 3, irrespective of CSI processing time indication.

# Conclusion

Observation 1: RAN4 has already agreed to specify CSI-RS based L1-RSRP (and L1-SINR) measurement accuracy requirements based on 24 RBs for Case 3.

Observation 2: Even if the UE can receive CSI-RS over 48 PRBs from two DL subbands simultaneously, the measurement accuracy is not equivalent to contiguous 48 PRBs due to the loss of processing gain from disjoint CSI-RS resource blocks (e.g., degraded channel estimation performance).

Proposal 1: Maintain the agreement from RAN4#114bis and specify CSI-RS-based L1-RSRP and L1-SINR measurement accuracy requirements based on 24 RBs for Case 3, irrespective of CSI processing time indication.

# References

[1] 	RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#104

[2] 	R4-2515166, “WF for [116bis][309] Rel-19 RRM performance_Part1”, Qualcomm, RAN4#116-bis

[3] R4-2504904, “WF on RRM requirements for NR_duplex_evo”, Huawei, RAN4#114-bis