3GPP TSG-RAN4 Meeting #117						           R4-2522242

Dallas, Texas US, 17 – 21 November 2025

Agenda Item:	   8.2.1

Source:	Thales

Title:	NTN PA model discussion for 6GR air interface

WI code: 	FS_6G_Radio

Release:	Rel-20

Document for:	Discussion

# Introduction

The 6G Radio Study Item approved at RAN#108 (RP-251881) [1] and revised at RAN#109 in RP-252912 [2] highlights the importance of Non-Terrestrial Network (NTN) support in 6G from the outset.

The non-ideal behavior of Radio Frequency (RF) hardware imposes boundaries and limitations on system performance. The envisioned use of high carrier frequencies and wide signal bandwidths introduces additional challenges for RF design.

RF non-idealities that reduces the signal quality are quantization noise of the converters, IQ-imbalance, phase noise, thermal noise, and PA nonlinearity. In this first version of the deliverable, we focused on the phase noise and the PA nonlinearity.

In this contribution, we provide PA inputs for NTN support in 6G.

# PA model discussion for satellite

Power amplifiers are non-linear. As the carrier power applied to the input of an amplifier is increased, there is a region of quasi-linear operation at low level after which the output power no longer increases in proportion to the input power. The maximum output power corresponds to saturation. When operating with several sub-carriers, intermodulation products appear at frequencies corresponding to linear combinations of the input carrier frequencies and when the carriers are modulated, the intermodulation products which fall within the useful bandwidth of the amplifier behave as noise. This noise is characterized within the bandwidth of each carrier by the value of the intermodulation power spectral density (N0)IM. To limit intermodulation noise when several carriers are amplified simultaneously to a value compatible with the overall link budget requirement, it is advisable to operate the amplifier below the saturation region. The OBO, defined as the ratio of the output power delivered on one of the n carriers (Pon) to the saturation power, determines the position of the operating point. The power delivered at the amplifier output for the carrier concerned is thus equal to:

Pon= Psat∙OBO [W]

The value of the back-off depends on the minimum allowed value of the receiver’s contributions to the carrier power to intermodulation power spectral density ratio (CN0)IM of the link, the number of carriers, and the input-output characteristic of the amplifier.

In order to model the nonlinear behavior of the amplifier, a few simple models can be considered. A more rigorous model would include the Volterra series expansion which can model complex non-linearities such as memory effects. Among the simpler models, the Rapp model, Saleh model, and the Ghorbani model can be considered. Combinations of pure polynomial models and filter models are also often referred to as fairly simple models, among which the Hammerstein model could be mentioned.

The advantage of the simpler models is usually related to the need of very few parameters to model the non-linear behaviour. The drawback is that such a model only can be used in conjunction with simple architecture amplifiers such as the basic Class A, AB and C amplifiers. Amplifiers such as the high efficiency Doherty amplifier can in general not be modelled by one of these simple models.

In addition, to properly capture the PA behaviour for the envisaged large bandwidths, it is essential to use PA models capturing the memory effects. Such models would require an extensive set of empirical measurements for proper parameterization.

1/ Rapp Model

A widely accepted solid state amplifier model encompassing the amplitude clipping, i.e., the AM/AM model, is the Rapp model. The latter is designed based on the following equation and it produces a smooth transition for the envelope characteristics as the input amplitude approaches saturation. Amplitude distortion (AM/AM) and Phase distortion (AM/PM) are obtained as follow:

FAM-AM=Gxt1+GxtAsat2P12P

FAM-PM=Ax(t)q1+xtBq

where Asat is saturation output amplitude, xt is complex envelope of the input signal, G is the signal gain, while P controls the smoothness of the transition from a linear region to a saturation region.

If signal measurements are at hand of the input/output relationship, the parameters of the model may be readily found for a particular amplifier by for example regression techniques.

The strength of the Rapp model is lies in its simple and compact formulation, and that it gives an estimation of the saturation characteristics of an amplifier. The drawback of this simple model is of course that it cannot model higher order classes of amplifiers such as the Doherty amplifier. It also lacks the ability to model memory effects of an amplifier.

In conclusion, RAPP model similar to other memory less models would capture some aspects in relation to waveform design but could not serve as a complete and comprehensive PA model covering all the effect possibly affecting the waveform design.

2/ Saleh Model

Saleh’s nonlinear models are widely used in the behavioral modeling of modern RF nonlinear solid-state power amplifiers (SSPAs), particularly when the PA can be assumed to be memoryless—typically true only over a narrowband—or when an equivalent memoryless (EM) model is sufficient.

His original 4-parameter generic model, is given by:

FAM-AM= α xtη(1+βxt2)ν

with η=1,2, or 3 and ν=1 or 2. The variable xt represent the envelope of the mem-oryless PA’s input signal. The parameters η and ν are preselected. The model’s other two parameters, the approximating coefficients α and β, may be extracted usinga least squares (LS) approximation to minimise the relative error between the envelope measurements of the target and the values predicted by the model.

From this, the well-know ‘2-parameter’ AM-AM and AM-PM distortion relation are found to be as:

FAM-AM=αax(t)1+βaxt2

FAM-PM=αφxt21+βφxt2

Where x is the envelop of the complex signal fed into the amplifier, and / are real-valued parameters that can be used to tune the model to fit a particular amplifier.

3/ Modified Saleh model

The modifications to Saleh’s model include allowing η to take the value of zero and introducing a new ‘offset phase shift parameter ε as a means to cater for non-zero initial AM-PM. This latter leaves α ‘free’ to function as a scaling parameter of |x(t)|η attenuated by a monotonic denominator, and help eliminate the danger of the pole instabilities occurring. A second new parameter γ is also introduced to allow greater variability at the denominator. This improves the range of curves which can be matched and likely accuracy of such matching. The new generic model can be written as:

FAM-AM= α xtη(1+βxtγ)ν-ε

4/ Ghorbani Model

The Ghorbani model also gives expressions similar to the Saleh model, where AM-AM and AM-PM distortion is modeled. The AM/AM and AM/PM distortion functions are:

FAM-AM=x1r(t)x21+x3rtx2+x4r

FAM-PM=y1rty21+y3rty2+y4|rt|

In the expressions above, r is the amplitude of the input signal and the actual scalars x1-4 and y1-4 have to be extracted from measurements by curve fitting or some sort of regression analysis.

## C-band UE amplifier model

The proposed PA model at UE side corresponds to a modified Saleh model.

For an input signal s=|x(t)| ejθ and output Tx(t)ejθ+Φa, the AM-AM model is given by:

Tx=α xtη1+β x(t)γν

α=16, η =1, β = α /1.9, γ=2.2, ν=1/ γ

The phase distortion Φa may be neglected (as it can be precomputed and cancelled).

Figure 1 presents an example implementation of the AM-AM model following adjustment of the saturation power (Psat) and linear amplification gain to 30 dBm and 0 dB, respectively, via input and output scaling of the PA model above.

Figure 1:  Example implementation of the AM-AM model following adjustement of the saturation power

Importantly, as input and output scaling of the model corresponds simply to a translation of the power representation in the figure above, it is evidenced that the 1dB and 3 dB compression points of the suggested UE PA model correspond approximately to Psat – 5 dB and Psat of input power, respectively.

## Q/V band UE amplifier model

The UE amplifier model in Q/V band is based on advanced circuit simulation of a CMOS PA research prototype at 28 GHz and it has been proposed by ERICSSON in [R4-165901]. In particular, the model is based on the following Generalized Memory Polynomial (GMP) model:

yGMPn= k∈Kal∈Laaklxn-l|x(n-l)|2k+k∈Kbl∈Lbm∈Mbklmxn-l|x(n-l-m)|2k

Where yGMPn and xn represent the complex baseband equivalent output and input, respectively, of the model. The first term represents the double sum of so called diagonal terms where the input signal at time shift l,xn-l;l ∈La, is multiplied by different orders of the time aligned input signal envelope |x(n-l)|2k; k∈Ka. The triple sum represents cross terms, i.e. the input signal at each time shifts is multiplied by different orders of the input signal envelope at different time shifts. The GMP is linear in the coefficients, akl and bklm which caters for robust estimation based on input and output signal waveforms of the PAs to be characterized. As a complement to the above, also memoryless polynomial models have been derived based on:

yPn=k∈Kpakxn|x(n)|2

The memoryless model has -32.1 dB Normalized Mean Square Error (NMSE) and is defined by:

ak, k∈[0,1,..,7]: [0.491576+0.870835i; -1.26213+0.242689i; 7.11693+5.14105i; -30.7048-53.4924i; 73.8814+169.146i; -96.7955-253.635i; 65.0665+185.434i; -17.5838-53.1786i]

The corresponding GMP model has -41.7 dB NMSE and is defined by:

ak, k∈[0,1,..,7]:

l=1:

[-0.0109821+0.00313982i; -0.00397658-0.0427409i; -0.171194+0.151692i; 0.879844-0.0235651i; 
-1.97684-0.862044i; 2.32524+1.99694i; -1.34472-1.77602i; 0.289959+0.559338i]

l=0:

[0.473465+0.860276i; -0.953417+0.640666i; 1.9899-2.3847i; 7.5417+6.38381i; 
-64.8415-60.8762i; 159.01+189.579i; -167.466-225.579i; 65.4247+92.5967i]

l=-1:

[0.0164844+0.00671299i; -0.0198519+0.177212i; 0.669594-0.543745i; -2.98038-0.279477i; 
6.6717+4.50511i; -8.26935-9.04627i; 5.42365+7.52782i; -1.47259-2.32623i]

Figure 3 and Figure 4 show the gain and phase characteristics of the GMP and static model using the same OFDM signal that was used for model estimation.

Figure 3: Gain characteristics: GMP model (BLUE), static model (RED)

Figure 4: Phase characteristics: GMP model (BLUE), static model (RED)

## C-band satellite amplifier model

The AM/AM and AM/PM characteristics of the satellite amplifier model in C-band are obtained by means of measurements.

Table 1 and Figure 5 represent the amplifier model of a single radiating element in the C-band.

Table 1: AM/AM and AM/PM characteristics of the satellite amplifier in C-band

| Input power (dBm) | Output power (dBm) | Phase (°) |
| --- | --- | --- |
| -35,0 | 16,485 | -19,132 |
| -34,0 | 17,537 | -19,020 |
| -33,0 | 18,525 | -18,940 |
| -32,0 | 19,488 | -19,198 |
| -31,0 | 20,460 | -18,203 |
| -30,0 | 21,475 | -18,567 |
| -29,0 | 22,394 | -20,139 |
| -28,0 | 23,350 | -18,540 |
| -27,0 | 24,267 | -18,004 |
| -26,0 | 25,151 | -16,626 |
| -25,0 | 26,027 | -16,050 |
| -24,1 | 26,877 | -16,048 |
| -23,0 | 27,795 | -14,384 |
| -22,0 | 28,778 | -13,913 |
| -21,0 | 29,797 | -11,809 |
| -20,0 | 30,806 | -9,754 |
| -19,0 | 31,760 | -10,004 |
| -18,0 | 32,605 | -8,331 |
| -17,0 | 33,333 | -7,515 |
| -16,0 | 33,944 | -6,840 |
| -15,0 | 34,458 | -5,354 |
| -14,0 | 34,784 | -1,210 |
| -13,0 | 34,954 | 2,015 |

Figure 5: NON-LINEAR AMPLIFIER HPA CHARACTERISTICS IN C-BAND

## Q-band satellite amplifier model

The AM/AM and AM/PM characteristics of the satellite amplifier model in Q/V bands are obtained by means of measurements.

Table 2 and Figure 6 represent the amplifier model of a single radiating element in the Q-band.

Table 2: AM/AM AND AM/PM AMPLIFIER RESPONSE FOR Q-BAND MODEL

| Input power (dBm) | Output power (dBm) | Phase (°) |
| --- | --- | --- |
| -20,00 | 1,90 | 0,00 |
| -19,00 | 2,90 | -0,10 |
| -18,00 | 3,90 | -0,20 |
| -17,00 | 4,90 | -0,30 |
| -16,00 | 5,90 | -0,50 |
| -15,00 | 7,00 | -0,70 |
| -14,00 | 8,00 | -1,00 |
| -13,00 | 9,00 | -1,40 |
| -12,00 | 10,00 | -1,80 |
| -11,00 | 11,00 | -2,30 |
| -10,00 | 12,10 | -2,90 |
| -9,00 | 13,10 | -3,60 |
| -8,00 | 14,10 | -4,50 |
| -7,00 | 15,20 | -5,50 |
| -6,00 | 16,20 | -6,70 |
| -5,00 | 17,30 | -8,20 |
| -4,00 | 18,30 | -10,10 |
| -3,00 | 19,30 | -12,30 |
| -2,00 | 20,30 | -14,70 |
| -1,00 | 21,10 | -17,40 |
| 0,00 | 21,80 | -20,10 |
| 1,00 | 22,40 | -22,30 |
| 2,00 | 22,90 | -24,20 |
| 3,00 | 23,40 | -26,30 |
| 4,00 | 23,70 | -28,80 |
| 5,00 | 24,00 | -31,40 |
| 6,00 | 24,20 | -34,20 |
| 7,00 | 24,40 | -37,20 |
| 8,00 | 24,50 | -40,00 |
| 9,00 | 24,70 | -42,60 |
| 10,00 | 24,90 | -45,30 |

Figure 6: Payload Non-Linear Amplifier HPA characteristics in the Q-band

## Phase Noise

Phase noise is identified as one of the key non-idealities that limit the performance of wideband 6G systems operating in high carrier frequencies.

The selection of waveforms and the frame structure significantly impact the system's overall performance under phase noise. Specifically, the effect of phase noise on different waveforms depends on how phase synchronization is achieved in the receiver and how data and pilot sequences are distributed across the time domain signal relative to phase jitter characteristics. In multi-carrier waveforms, lower offset frequency components of phase noise cause common phase error (CPE) due to long symbol durations, which can be mitigated in the receiver using appropriate phase noise tracking reference signals. Higher offset frequencies and the flat portion of the phase noise contribute to inter-symbol interference, generally limiting the achievable SNR. For single-carrier waveforms, the symbols are short in the time domain, so potential phase drift over a single symbol is primarily dominated by flat phase noise, assuming a fast symbol rate. The impact of lower offset frequency components of phase noise also depends on the signal length.

The proposed phase noise profiles are the ones proposed in the MARINA project [ETSI TR 103 886] and they are representative of the total aggregated phase noise contribution generated by the main contributors (i.e. the satellite and the terminal) on the FWD, i.e., from the gNB (satellite) to the UE, and RTN links, i.e., from the UE to the gNB (satellite).

Table 3 and Table 4 present the phase noise profiles considered in this study.

Table 3: Single sideband aggregated FWD phase noise profile

| Offset from Carrier Frequency | Forward link (dBc/Hz) |
| --- | --- |
| 10 Hz | NA |
| 100 Hz | [-25] |
| 1 kHz | [-50] |
| 10 kHz | [-73] |
| 100 kHz | [-92] |
| 1 MHz | [-102] |
| 10 MHz | [-113] |
| 100 MHz | [-116] |

Table 4: Single sideband aggregated RTN phase noise profile

| Offset from Carrier Frequency | Return link (dBc/Hz) |
| --- | --- |
| 10 Hz | [-36] |
| 100 Hz | [-58] |
| 1 kHz | [-69] |
| 10 kHz | [-79] |
| 100 kHz | [-87] |
| 1 MHz | [-104] |
| 10 MHz | [-109] |
| 100 MHz | [-109] |

For more details please also check: Public Deliverables | 6G-NTN

# Conclusion

In this contribution, we made the following observations and proposals:

Proposal 1: C-band and Q/V-band PAs to be considered for the NTN part of 6G.

Proposal 2: Other PAs for other frequency bands such as S/L/Ku/Ka to be provided for NTN at RAN4#117 (as a revision paper) or at RAN4#118 (RAN4 February meeting in 2026).