# 3GPP TSG-RAN WG4 Meeting #117	R4-2522642

Dallas, US, November 17-21, 2025

Agenda item:			6.15.1

Source:	Ericsson

Title:	WF on Netw_Energy_NR_enh_RRM_Part1

Document for:	Approval

# Topic #1: On-demand SSB(OD-SSB) performance part

# Sub-topic 1-1: Test case

Issue 1-1-1: SCell activation of known SCell Test Case

Agreement:

No need to define the following test case

| Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- |
| SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation FR1 |

Issue 1-1-2: Directed SCell activation Test Case

Agreement:

Define the following test case

| Test category | Test scenario | Test configuration | Test purpose | Company |
| --- | --- | --- | --- | --- |
| Direct SCell activation at SCell addition based on OD-SSB | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a FR1 | ZTE |

Issue 1-1-3: Deactivated SCell measurement Test Case

Agreement:

Define the following test case:

| Test category | Test scenario | Test configuration | Test purpose | Company |
| --- | --- | --- | --- | --- |
| Intra-frequency measurement (deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW FR1 | Huawei |

Issue 1-1-4: Test case for Alt Time-C2

Agreement:

Not define TC for Alt Time-C2.

# Topic #2: Maintenance

Issue 2-1-1: OD-SSB interruption for Intra-band CA

Agreement

If the active serving cells are in the same band as any of the SCells on which OD-SSB is being activated or deactivated, the interruption length is RF retuning time plus the time of transmitted one OD-SSB burst.

FFS the following conditions

- The cell specific reference signals from the active serving cells and the SCells on which OD-SSB is being activated or deactivated are available in the same slot in deactivated OD-SSB SCell measurements

- Otherwise, the condition is not needed.