3GPP TSG-RAN WG4 Meeting #117	R4-2522647

Dallas, US, 17th – 21st November 2025

Agenda item:			6.17.1

Source:	Nokia

Title:	Way forward for [117][215] NR_Mob_Ph4_RRM_Part1

Document for:	Information

# Open issue summary

The following issues are open in RRM core or performance part and will be discussed in the next meeting.

RRM Core part for CLTM (AI 4.23)

Issue 1-1-2: How to capture the known / unknown condition in the specification

Performance part for event triggered reporting (AI 6.17.2.1):

Issue 2-1-1: Whether to send DCI in every slot after the event condition is met.

Moderator guidance:

- For SSB based test cases, check if “delta approach” can be used, and apply the delta approach if possible.

- Delta approach means that the part that is identical to the baseline test case is referred and only the difference is captured. An example of delta approach, you can check A.6.3.1.18. This should work for SSB based

Performance part for CSI-RS L1-RSRP (AI 6.17.2.2):

Issue 3-1-1: Whether to send DCI in every slot after the event condition is met

Issue 3-1-2: Test case design for event triggered CSI-RS based L1-RSRP when SSB L1-RSRP measurement is not performed

# Topic #1 RRM Core requirements

## Conditional Intra-CU LTM

Issue 1-1-1: Whether to include unknown cell case on SSB triggered CLTM

Issue 1-3-1: Whether to include unknown cell case on SSB triggered CLTM Recommended WF: Discuss the following: - For SSB based L1-RSRP measurement used with CLTM. Discuss and confirm which scenario is supported: - Option 1: Support only known case (specification change needed) - Option 2: Support both unknown and known case (specification change needed) - Option 3: Support only unknown case (no specification change).

<Agreement>:

- Define requirements for both known and unknown cases for CLTM when an SSB-based L1-RSRP measurement is used in the event.

- The requirements apply when CLTM is configured after L3 measurement report from the configured CLTM candidate cell(s)

  - Exact wording can be discussed in CR

Issue 1-1-2: How to capture the Known / Unknown condition in the specification

Proposals:

- Proposal 1 (Huawei): The definition of known cell in CLTM needs to be defined, since the known condition for LTM stated in section 6.3.1 TS38.133 requires UE to send a valid L1/L3 report, and it is not applicable for CLTM.

- Proposal 2 (ZTE): For SSB based L1-RSRP measurement used with CLTM, support both unknown and known case. And further discuss whether it is necessary to change existing specification.

  - Observation 1: For the definition of Tmeasure in CHO, both known cell and unknown cell are considered, and the requirements defined based on the worst case.

- Proposal 3 (Nokia): Both known and unknown target cell cases should be covered in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event.

  - CR R4-2521527:

    - For intra-frequency target cell is known according to the definition in clause 9.14.2

    - For inter-frequency target cell is known according to the definition in clause 9.15.2

<Way Forward>:

- FFS how capture known in the specification.

# Topic #2 Event triggered L1 measurement reporting performance part

Issue 2-1-1: Whether to send DCI in every slot after the event condition is met

Technical background from 	R4-2521920. Check also Issue 3-1-1 as it is more or less the same.

The event triggered measurement reporting delay is defined as the time between an event that will trigger an event triggered measurement report and the point when the UE starts to transmit the first UL transmission to report the measurement results over the air interface. This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

In the test configuration, using SR as the end point is not verifiable, since the UE can send the SR at any arbitrary time, and the test equipment (TE) cannot determine when the measurement period has actually ended. While using a special purpose SR could be an alternative, but not all UEs may support it. To ensure consistent testing across all UEs, we propose scheduling a DCI with UL grant in every slot once the event is triggered.

Proposals:

- Proposal 1 (Ericsson): 	RAN4 to agree that TE sends DCI in every slot after the event condition is changed in the test.

<Way Forward>:

- FFS for the design where TE sends DCI in every slot after the event condition is changed in the test.

# Topic #3 CSI-RS based L1 measurement Performance part

Issue 3-1-1: Whether to send DCI in every slot after the event condition is met

Technical background from 	R4-2521922

The event triggered measurement reporting delay is defined as the time between an event that will trigger an event triggered measurement report and the point when the UE starts to transmit the first UL transmission to report the measurement results over the air interface. This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

In the test configuration, using SR as the end point is not verifiable, since the UE can send the SR at any arbitrary time, and the test equipment (TE) cannot determine when the measurement period has actually ended. While using a special purpose SR could be an alternative, but not all UEs may support it. To ensure consistent testing across all UEs, we propose scheduling a DCI with UL grant in every slot once the event is triggered.

Proposals:

- Proposal 1 (Ericsson): 	RAN4 to agree that TE sends DCI in every slot after the event condition is changed in the test.

<Way Forward>:

- FFS for the design where TE sends DCI in every slot after the event condition is changed in the test.

Issue 3-1-2: Test case design for event triggered CSI-RS based L1-RSRP when SSB L1-RSRP measurement is not performed

Technical background from R4-2521922:

- UE may need time to detect the event, it is assumed that the measurement is performed after T2 begins

- Considering the beam sweeping factor, the UE may realize the event condition after completing a full sweep, which could take up to 8 × 20 ms = 160 ms

- Assuming DCI is scheduled every slot (Dependency to the previous issue)

- UE reports the event-triggered L1-RSRP within 160 ms from the start of T2

Proposals:

- Proposal 1 (Ericsson):	For the event triggered CSI-RS based L1-RSRP by skipping SSB based L1-RSRP, the UE is expected to report the event-triggered L1-RSRP within 160 ms (assuming CSI-RS periodicity of 20ms) from the start of T2.

<Way Forward>

- Further discuss about the TC design where with 20ms CSI-RS periodicity, the UE transmits the event triggered L1-RSRP measurement report 160ms from the start of T2.