3GPP TSG-RAN4 Meeting #117	R4-2522779

Dallas, US, 17 – 21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR TC for Intra-cell SSB-based UEIBM reporting in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of test cases for NR_MIMO_Ph5 WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the following test cases: - Intra-cell SSB-based event triggered reporting for UE initiated beam management without eventDetectionTimeWindowLength-r19 - Intra-cell SSB-based event triggered reporting for UE initiated beam management with eventDetectionTimeWindowLength-r19 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature is not tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A.6.6.1.X, A.6.6.1.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Change #1 >

#### A.6.6.1.X	Event Triggered Reporting for UE initiated beam management without eventDetectionTimeWindowLength-r19

##### A.6.6.1. X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes the correct reporting of event triggered L1-RSRP measurement for the UE initiated beam management. This test will partly verify the L1-RSRP reporting requirements in clause 9.5.3.4, with the testing configurations in table A.6.6.1.X.1-1.

Table A.6.6.1.X.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

##### A.6.6.1.X.2 	Test parameters

There is one cell in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.1.X.2-1 and table A.6.6.1.X.2-2 below.

In CSI resource configuration, UE is indicated to perform L1-RSRP measurement on the resourcesForChannelMeasurement and the resourcesForChannelMeasurement consists of SSB. In the CSI report configuration, UE is configured with event-triggered reporting, with the event 2 and Mode-A, and without eventDetectionTimeWindowLength-r19 configuration. The test consists of two successive time periods, with time duration of T1 and T2, respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

During test period T1, the UE receives from SSB#0 which is the current beam. During T1, SSB#1 is not detectable. At the beginning of T2, SSB#1 becomes stronger than the SSB#0 and the event entering condition is met.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.1.X.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | Off |
| resourcesForChannelMeasurement | 1~3 |  | SSB#1 |
| eventThreshold-r19 | 1,2,3 | dB | 4 |
| reportTransmissionMode-r19 | 1,2,3 |  | ModeA |
| nrofReportedRS-UE-IBR | 1~3 |  | 1 |
| currentBeamReport | 1~3 |  | disabled |
| pucch-Resource-r19: periodicityAndOffset | 1,2 | slot | sl4, 3 |
|  | 3 |  | sl8, 4 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 2 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.1.X.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.64 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 15 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -79.65 |
|  | 3 |  | -91.64 | -91.64 | -Infinity | -76.64 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -51.56 |
|  | 3 | dBm/38.16 MHz | -57.58 | -57.58 | -60.59 | -45.46 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 15 |
| NOTE 1: The resources for first uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.X.3	Test Requirements

In the test, the UE shall send a first PUCCH message using the new UCI type, no later than 20 ms + X slots from the beginning of T2, where:

- Config 1 and 2: X = 5

- Config 3: X = 8

The UE shall not send event triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

< End of change1  >

< Change #2 >

#### A.6.6.1.Y	Event Triggered Reporting for UE initiated beam management with eventDetectionTimeWindowLength-r19

##### A.6.6.1.Y.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes the correct reporting of event triggered L1-RSRP measurement for the UE initiated beam management, when eventDetectionTimeWindowLength-r19 is configured. This test will partly verify the L1-RSRP reporting requirements in clause 9.5.3.4, with the testing configurations in table A.6.6.1.Y.1-1.

Table A.6.6.1.Y.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

##### A.6.6.1.Y.2 	Test parameters

There is one cell in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.1.Y.2-1 and table A.6.6.1.Y.2-2 below.

In CSI resource configuration, UE is indicated to perform L1-RSRP measurement on the resourcesForChannelMeasurement and the resourcesForChannelMeasurement consists of SSB. In the CSI report configuration, UE is configured with event-triggered reporting, with the event 2 and Mode-A, and with eventDetectionTimeWindowLength-r19 configuration. The test consists of two successive time periods, with time duration of T1 and T2, respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

During test period T1, the UE receives from SSB#0 which is the current beam. During T1, SSB#1 is not detectable. At the beginning of T2, SSB#1 becomes stronger than the SSB#0 and the event entering condition is met.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.1.Y.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | Off |
| eventThreshold-r19 | 1,2,3 | dB | 4 |
| reportTransmissionMode | 1,2,3 |  | ModeA |
| eventDetectionTimeWindowLength-r19 | 1,2,3 | ms | 80 |
| eventInstanceCount-r19 | 1,2,3 |  | 3 |
| nrofReportedRS-UE-IBR | 1~3 |  | 1 |
| currentBeamReport | 1~3 |  | enabled |
| pucch-Resource-r19: periodicityAndOffset | 1,2 | slot | sl4, 3 |
|  | 3 |  | sl8, 4 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 2 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.1.Y.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.64 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 15 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -79.65 |
|  | 3 |  | -91.64 | -91.64 | -Infinity | -76.64 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -51.56 |
|  | 3 | dBm/38.16 MHz | -57.58 | -57.58 | -60.59 | -45.46 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 15 |
| NOTE 1: The resources for first uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.Y.3	Test Requirements

In the test, the UE shall send a first PUCCH message using the new UCI type, no later than 60 ms + X slots from the beginning of T2, where:

- Config 1 and 2: X = 5

- Config 3: X = 8

The UE shall not send event triggered measurement reports as long as the reporting criteria with eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 according to the configuration in Table A.6.6.1.Y.2-1 is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

< End of changes >