3GPP TSG-WG4 Meeting #117	R4-2522781

# Dallas, USA, Nov. 17 – Nov. 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft CR | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Event-2, FR2, Intra-cell SSB based, no window time based, mode-A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Core requirements are agreed to be supported for the event 2. The TC should be defined to test the feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC on Event-2 for FR2, Intra-cell SSB based, no window time based, mode-A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec. is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2521917 |  |  |  |  |  |  |  |  |

---------------------- Start of changes ---------------------------

#### A.7.6.3.X	Event Triggered Reporting for the UE initiated beam management

##### A.7.6.3.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes the correct reporting of event triggered L1-RSRP measurement for the UE initiated beam management. This test will partly verify the L1-RSRP reporting requirements in clause 9.5.3.4, with the testing configurations for NR cells in table A.7.6.3.X.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15.

Config 1 is used in this test, and it is given by, Config 1: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode.

##### A.7.6.3.X.2	Test parameters

There is one cell in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.7.6.3.X.2-1 and table A.7.6.3.X.2-2 below.

In CSI resource configuration, UE is indicated to perform L1-RSRP measurement on the resourcesForChannelMeasurement and the resourcesForChannelMeasurement consists of SSB. In the CSI report configuration, UE is configured with event-triggered reporting, with the event 2 and Mode-A. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

During the T1, SSB 1 is not detectable. At the beginning of T2, SSB 1 becomes stronger than the SSB 0 by offset amount and the event entering condition is met.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.7.6.3.X.2-1: General test parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| SSB ARFCN |  | freq1 |
| Duplex mode |  | TDD |
| TDD Configuration |  | TDDConf.3.1 |
| BWchannel | MHz | 100: NPRB,c = 66 |
| Data PRBs allocated |  | 66 |
| PDSCH Reference measurement channel |  | SR.3.2 TDD |
|  |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD |
|  |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel |  | CCR.3.1 TDD |
|  |  | CCR.3.7 TDD |
| SSB configuration |  | SSB.1 FR2 |
| eventThreshold-r19 | dB | 4 |
| OCNG Patterns |  | OP.1 |
| Initial BWP Configuration |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration |  | SMTC.1 |
| TRS Configuration |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration |  | TCI.State.2 |
| DRX configuration |  | Off |
| reportConfigType |  | periodic |
| reportQuantity |  | ssb-Index-RSRP |
| Number of reported RS |  | 2 |
| L1-RSRP reporting period | slot | 320 |
| PUCCH-Cell |  | Cell 1 |
| periodicityAndOffset | slots | Sl5, 4 |
| T1 | s | 5 |
| T2 | s | 2 |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |
| Propagation condition |  | AWGN |

Table A.7.6.3.X.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam AssumptionNote 4 | 1 |  | Rough |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -96 |  |  |  |
|  | 1 | dB | [-32.03] | [-32,03] | -Infinity | [-34.53] |
| SSB_RP Note3 | 1 | dBm/SSB SCS | -96 | -96 | -Infinity | [-92] |
| Io Note3 | 1 | dBm/95.04 MHz | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 1 | dB | 0 | 0 | -Infinity | 4 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.6.3.X.3	Test Requirements

In the test, the UE shall send a first PUCCH message using the new UCI type, within 160ms plus 5 slots from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

--------------------END OF CHANGES--------------------------