3GPP TSG-RAN4 Meeting #117	R4-2522840

# Dallas, Texas, USA, November 17th – November 21st, 2025

Agenda item:			6.9.1

Source:	vivo

Title:	WF on TRP_TRS_MIMO_OTA_Ph3

Document for:	Approval

Introduction

This is WF of Rel-19 OTA WI.

Topic #1: Rel-19 OTA test methodologies

Sub-topic 1-1 R19 OTA test methods

Issue 1-1-3: Discussions on NTN OTA configurations suggested by CTIA for testing time reduction

Agreements:

- Adopt the CTIA outcome on NB-IoT NTN TRP TRS test time reduction to replace previous configuration or as one alternative. This can be further discussed.

- If this is alternative, this can be selected based on OEM declaration.

Issue 1-1-4: Discussions on other alternative NTN OTA configurations for testing time reduction

- Proposals

  - Proposal 1: Consider the ephemeris consistency check introduced and adopted in RAN5 specs already as alternate test time reduction approach for Rel-17 NB-IoT NTN OTA testing. (Keysight)

  - Proposal 2: Consider disabling of the HARQ feedback as alternate test time reduction approach for Rel-18 and beyond NB-IoT NTN OTA testing. (Keysight)

  - Proposal 3: Study whether the ephemeris consistency check for NR NTN OTA testing is needed for GSO and/or whether the test time without this consistency check is acceptable. (Keysight)

  - Proposal 4: For devices that support both NGSO and GSO, the test equipment in the OTA test environment for the NR NTN OTA TC shall emulate the signal with constant doppler and constant delay (per RAN5 agreeements) according to ephemeris for NGSO (which LEO is FFS). (Keysight)

Way Forward:

  - RAN4 could further discuss above proposals.

Topic #2: Rel-19 OTA requirements

Sub-topic 2-1 1Tx FR1 TRP TRS requirements

Issue 2-1-1: LS from RAN5 on Rel-19

Agreements:

- No need to update the TT value, it has been concluded for all FR1 bands. This information can be shared to RAN5 next meeting potentially with concluded requirements.

Issue 2-1-2: Analysis of Rel-19 measurement campaign

Way forward：

- The Rel-19 data pool is concluded (The Thresholds value in Issue 3-1-6 agreed last meeting WF are met and no issue identified). No new devices measurement results are allowed for Rel-19 measurement campaign.

- The analysis of Rel-19 measurement campaign is presented in R4-2520728.

Issue 2-1-3: Device information collection update

Moderator: The following analysis provided by MCC in RAN4 reflector:

Agreements:

- With the analysis of UE information statistics by MCC, the Rel-19 data pool number of UEs meets all the required threshold values.

Issue 2-1-4: Processing of measurement data pool for Rel-19 requirements discussion

Agreements:

- RAN4 decide not to remove any measurement results out of data pool. The Rel-19 data pool is stable and solid. The requipments discussion will be decide based on the full set of data pool results.

Issue 2-1-5: Discussion of potential requirements and TT for Rel-19 TRP/TRS

Agreement:

RAN4 aims to complete the performance requirements in the next meeting.