3GPP TSG RAN WG4 Meeting #117	R4-2522988

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on TC for signalling characteristics for RedCap NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19, the RedCap NTN feature was introduced. For, relevant RRM requirements on RedCap NTN are defined. Correaspondinly, the test cases need to be defined in RAN4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce following test cases for RedCap NTN: - Active BWP switch delay for RedCap UE with satellite access - UE-specific CBW change for RedCap UE with satellite access - Pathloss reference signal switching delay for RedCap UE with satellite access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No correasponding test case design for signalling characteristics for redcap NTN in TS38.133 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1X.4.3, A.1X.4.4, A.1X.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2521446 |  |  |  |  |  |  |  |  |

### <Start of Change 1>

A.1X.4.3	Active BWP switch for RedCap UE with Satellite Access

A.1X.4.3.1	DCI-based and Timer-based Active BWP Switch

A.1X.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA

A.1X.4.3.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6E.

The supported test configurations are shown in table A.1X.4.3.1.1.1-1 below.

The test procedure and environment in clause A.14.4.3.1.1.1 shall apply.

Table A.14.4.3.1.1.1-1: DL BWP switch supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

A.1X.4.3.1.1.2	Test Requirements

The test requirements in clause A.14.4.3.1.1.2 shall apply.

A.1X.4.3.2	RRC-based Active BWP Switch

A.1X.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA

A.1X.4.3.2.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in clause 8.6E.

The supported test configurations are shown in table A.1X.4.3.2.1.1-1 below.

The test procedure and environment in clause A.14.4.3.2.1.1 shall apply.

Table A.1X.4.3.2.1.1-1: DL BWP switch supported test configurations in SA scenario

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

A.1X.4.3.2.1.2	Test Requirements

The test requirements in clause A.14.4.3.1.1.2 shall apply.

### <End of Change 1>

### <Start of Change 2>

A.1X.4.4	UE specific CBW change for RedCap UE with Satellite Access

A.1X.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX

A.1X.4.4.1.1	Test Purpose and Environment

The purpose of this test is to verify the UE specific CBW change delay requirement defined in clause 8.13E.

The supported test configurations are shown in table A.1X.4.4.1.1-1. The test scenario comprises of one Cell (Cell 1), which is PCell as given in table A.1X.4.4.1.1-2. Cell-specific parameters are specified in table A.1X.4.4.1.1-3.

The test procedure in clause A.14.4.4.1.1.1 shall apply.

Table A.1X.4.4.1.1-1: Supported test configurations for UE specific CBW change in SA scenario

| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| --- | --- |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Table A.14.4.4.1.1-2: General test parameters for UE specific CBW change in SA scenario

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

Table A.1X.4.4.1.1-3: NR Cell specific test parameters for UE specific CBW change in SA scenario

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1, 2 |  | FDD |
|  |  | Config 3, 4 |  | HD-FDD |
| BWchannel |  | Config 1, 2, 3, 4 |  | 10 MHz: NPRB,c = 52 |
| Satellite information |  | Config 1, 3 |  | SSC.1 |
|  |  | Config 2, 4 |  | SSC.2 |
| Active DL BWP ID |  | Config 1, 2, 3, 4 |  | 1 |
| Initial DL BWP Configuration (BWP-1) |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| Initial UL BWP Configuration |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| Initial Condition | Active DLCBW-1 Configureation | Config 1, 2, 3, 4 |  | DLCBW.1.1 |
|  | Active UL CBW-1 Configuration | Config 1, 2, 3, 4 |  | ULCBW.1.1 |
| Final Condition | Active DLCBW-1 Configureation | Config 1, 2, 3, 4 |  | DLCBW.1.2 |
|  | Active UL CBW-1 Configuration | Config 1, 2, 3, 4 |  | ULCBW.1.2 |
| PDSCH Reference measurement channel |  | Config 1, 2, 3, 4 |  | SR.1.1 FDD |
| RMSI CORESET parameters |  | Config 1, 2, 3, 4 |  | CR.1.1 FDD |
| Dedicated CORESET parameters |  | Config 1, 2, 3, 4 |  | CCR.1.1 FDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | Config 1, 2, 3, 4 |  | SSB.1 FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| TRS Configuration |  | Config 1, 2, 3, 4 |  | TRS.1.1 FDD |
| Antenna Configuration |  |  |  | 1x1 |
| Propagation Condition |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |
| NocNote 2 |  | Config 1, 2, 3, 4 | dBm/SCS | -104 |
| SS-RSRP Note 3 |  | Config 1, 2, 3, 4 | dBm/SCS | -87 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | Config 1, 2, 3, 4 | dBm/ 9.36 MHz | -58.96 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

A.1X.4.4.1.2	Test Requirements

The test requirements in clause A.14.4.4.1.2 shall apply.

### <End of Change 2>

### <Start of Change 3>

A.1X.4.5	Pathloss reference signal switching delay for RedCap UE with Satellite Access

A.1X.4.5.1	MAC-CE based pathloss reference signal switch delay

A.1X.4.5.1.1	Test Purpose and Environment

The purpose of this test is to verify the MAC-CE based pathloss reference signal switch delay requirement defined in clause 8.14E.

The supported test configurations are shown in table A.1X.4.5.1.1-1 below.

The test procedure and environment in clause A.14.4.5.1.1 shall apply.

Table A.1X.4.5.1.1-1: MAC-CE based pathloss reference signal switch supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

A.1X.4.5.1.2	Test Requirements

The test requirements in clause A.14.4.5.1.2 shall apply.

### <End of Change 3>