3GPP TSG RAN WG4 Meeting #117	R4-2523108

# Dallas, USA, 17 November 2025 – 21 November 2025

Agenda Item:	6.8.1

Source:	Ericsson, Huawei, ZTE, Nokia

Title:	On Transmitter Spurious Emission Co-existence discussion

Document for:	Information

# 1	Introduction

This document collects results of work performed by RAN4 based on NR_BS_RF_req_evo-Core WI in Rel-19, with the following objective:

- Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES

- Improve transmitter co-existence spurious emission requirements if needed.

  - Note: start the work after March 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz.

This document collects the co-existence simulation assumptions and results from different sources for Informaiton to the readers. This work was not concluded by RAN4 in Rel-19 in notrmative work due to lack of consensus.

# 2	BS Transmitter spurious emissions: co-existence requirements limits re-evaluation for high frequency bands

### 2.1	General

#### 2.1.1	Scope

This clause presents the outcome based on the request from ETSI MSG TFES [1] to re-evaluate Transmitter spurious emission co-existence requirement levels. The focus of this work is on bands in the upper region of FR1 with priority above 5.9 GHz.

##### 2.1.1.1	General

The spurious emission requirements in current RAN4 specifications for lower bands in FR1 region were originally evaluated considering passive sector antenna having a fixed main beam with peak gain of 17 dBi, where they are two networks in the same geographical area. The aggressor BS has interfering link towards the wanted UE in the same network and victim BS of the other network. The reference simulation assumptions can be found in TR 25.942 [2].

Based on the duplexing method FDD and TDD, the requirement levels apply as listed in Table 2.1.1.1-1 below.

Table 2.1.1.1-1: Requirement levels

| Duplex method | BS-to-BS (dBm/MHz) | BS-to-UE (dBm/MHz) |
| --- | --- | --- |
| FDD | -49 | -52 |
| TDD | -52 | -52 |

##### 2.1.1.2	Protection of UE receiver (BS-to-UE)

The transmitter co-existence spurious emission requirement for BS is defined to protect the downlink of another network in the same geographical area. According to TR 25.942 [2], subclause 5.1.4.1, the coupling loss between BS and UE was set to -

- Minimum Coupling Loss (MCL) equal to 70 dB for the Macro-cellular environment,

- Minimum Coupling Loss (MCL) equal to 53 dB for the Microcell environment.

However, the MCL for Microcell environment was adopted for requirement level derivation. The Microcell MCL was selected since it was seen as the worst-case MCL.

The assumed values are listed in Table 2.1.1.2-1.

Table 2.1.1.2-1: Original assumptions for BS-to-UE coupling

| Parameter | Variable | Value |
| --- | --- | --- |
| UE receiver noise figure | F | 9 dB |
| BS-to-UE coupling loss | L | 53 dB |

The current requirement level is derived based on a link budget calculation on a logarithmical scale:

Pem=kT+B+F+L

where kT is equal to -174 dBm/Hz and B is equal to 1 MHz. Assuming the following values for the UE noise figure, F equal to 9 dB and L equal to 53 dB, the maximum allowed spurious emission level, Pem at the BS transmitter can be calculated to -52 dBm/MHz.

##### 2.1.1.3	Protection of BS receiver (BS-to-BS)

Traditionally the requirement level was determined by a static link budget calculation based on assumptions of BS noise figure and Macrocell BS-to-BS coupling loss. The assumed values are listed in Table 2.1.2.3-1 below -

Table 2.1.1.3.-1.: Original assumptions for BS-to-BS coupling

| Parameter | Variable | Value | Note |
| --- | --- | --- | --- |
| BS receiver noise figure | F | 5 dB |  |
| Macrocell BS-to-BS coupling loss | L | 67 dB | The current requirement level is derived based on an assumption of MCL between two base stations operating in different networks in the same geographical area of 67 dB (see TR 25.942 [2], subclause 10.2.1). |
| Protection criteria | M | 9 dB | Assumed value corresponds to 0.8 dB degradation of victim receiver sensitivity. |

The current requirement level is derived based on logarithmical scale as:

Pem=kT+B+F+L-M

where kT is equal to -174 dBm/Hz, B is equal to 1 MHz and parameter values in Table 2.1.1.3-1. With the assumption the maximum allowed spurious emission level in another band is calculated to -49 dBm/MHz.

#### 2.1.2	Modelling of out-of-band antenna gain

Clasue 7.3 in TR 38.922 [3] provides IMT base station and IMT user equipment technical parameters for analyzing adjacent band compatibility of terrestrial IMT systems with other co-primary services in the ITU Radio Regulations covering aspects related to modelling array antenna gain outside the wanted carrier relevant for modelling spatial characteristics for unwanted emissions.

As no agreement has been made, results have been considered with correlation factor ρ = 0, ρ = 0.5 and ρ = 1.

### 2.2	Co-existence study

#### 2.2.1 	Co-existence simulation scenarios

Table 2.2.1-1 summarizes the simulation scenarios which have been investigated during Rel-19.

Table 2.2.1-1: Summary of simulation scenarios

| No. | Aggressor frequency | Victim frequency | Direction |  |
| --- | --- | --- | --- | --- |
| 1 | 2 GHz | 6.4 GHz | DL – BS Micro to UE |  |
| 2 | 2 GHz | 6.4 GHz | UL – BS Macro to BS Macro |  |
| Note 1: It is agreed that for DL, the most critical scenario is Micro BS to UE interference similar to Rel-99 TR 25.942 assumption. Note 2: It is agreed that for UL, the most critical scenario is Macro BS to Macro BS interference similar to Rel-99 TR 25.942 assumption |  |  |  |  |

#### 2.2.2 	Co-existence simulation assumption

##### 2.2.2.1	Network layout model

###### 2.2.2.1.1 	Urban Macro

Except those listed in Table 2.2.2.1.1-1, the simulation assumptions in clause 4.2.1.1 from TR 38.921 [4] are resued.

Table 2.2.2.1.1-1: Additional simulation assumptions

| Parameters | Values | Note |
| --- | --- | --- |
| Inter-site distance | 0.5 km (urban) |  |
| BS antenna height | 25 m (urban) |  |
| Multi operators layout | Un-coordinated operation (100%/ 50% Grid Shift) | 100% grid shift has been the baseline assumption based on cell arrangements as depicted typical deployments in TR 25.942. Some have also considered 50% grid shift as a more realistic deployment. |

###### 2.2.2.1.2 		Dense Urban

Details on dense urban network layout model are listed in Table 2.2.2.1.2 -1 and 2.2.2.1.2 -2.

Table 2.2.2.1.2-1: Single operator layout for dense urban

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell |  | < 72.17 m | 3÷4×ISD÷3 |
| BS antenna height |  | 10 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi |  |
| Shadowing correlation |  | Between Micro site: 0.5 |  |
| Note: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD = 500 m is assumed. |  |  |  |

Figure 2.2.2.1.2-1: Network layout for dense urban

Table 2.2.2.1.2-2: Multi operators layout for dense urban

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Cluster circle is coordinated | Note |
| Minimum distance between micro BSs in different operator | 10 m |  |
| Note: Macro cell is collocated. Micro BS itself is randomly dropped. |  |  |

##### 2.2.2.2 	Propagation model

The path-loss model is described in Clause 4.2.2. from TR 38.921 [4] and can be used all propagations unless stated.

For modelling BS-to-BS propagation, the free space path loss (FSPL) in dB should be used, where the separation distance d between BSs is in meter and frequency fc in GHz is given by

##### 2.2.2.3 	Antenna characteristics

###### 1.2.2.3.1	BS antenna characteristics

The BS antenna is modelled as described in Clause 8.1.1. However, the antenna parameters for 6.4 GHz can be taken from Clause 8.1.2 TR 38.921 [3] and for 2 GHz for different BS deployments are listed in Table 2.2.2.3.1-1 below –

Table 2.2.2.3.1-1: IMT parameters relevant for 1710 to 4990 MHz [3]

| Parameter | Macro Rural | Macro suburban | Macro urban | Micro urban |
| --- | --- | --- | --- | --- |
| Am | 30 dB | 30 dB | 30 dB | 30 dB |
| SLAv | 30 dB | 30 dB | 30 dB | 30 dB |
| 3dB | 90 deg. | 90 deg. | 90 deg. | 90 deg. |
| 3dB | 65 deg. | 65 deg. | 65 deg. | 65 deg. |
| GE,max | 6.4 dBi | 6.4 dBi | 6.4 dBi | 6.4 dBi |
| Msub | 3 | 3 | 3 | N/A |
| dv,sub | 0.7l m | 0.7l m | 0.7l m | N/A |
| subtilt | 3 deg. | 3 deg. | 3 deg. | N/A |
| M | 4 | 4 | 4 | 8 |
| N | 8 | 8 | 8 | 8 |
| dh | 0.5l m | 0.5l m | 0.5l m | 0.5l m |
| dv | 2.1l m | 2.1l m | 2.1l m | 0.7l m |
| etilt | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤120 deg. |
| escan | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. |
| r | 1 | 1 | 1 | 1 |
| Ptx | 46 dBm | 46 dBm | 46 dBm | 37 dBm |
| mech | 3 deg. | 6 deg. | 6 deg. | N/A |

###### 2.2.2.3.2	UE antenna characteristics

The UE is expected to have a conducted interface assuming an isotropic radiation pattern antenna and without beamforming.

##### 2.2.2.4 	Other simulation parameters

Table 2.2.2.4-1: Other simulation parameters

| Parameters | BS-to-BS coupling: Urban macro | BS-to-UE coupling: Dense urban |
| --- | --- | --- |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | 1 UE per slot | 1 UE per slot |
| Additional Polarization gain | 0 dB (total BS output power of 46 dBm for two polarizations) | 0 dB (total BS output power of 46 dBm for two polarizations) |
| Gain at Rx | Full array again |  |
| BS Noise figure in dB | 6 dB for 6.4 GHz |  |
| UE Noise figure in dB |  | 12 dB for 6.4 GHz |

##### 2.2.2.5 	Evaluation metrics

The Release-99 study in TR 25.942 [2] to evaluate the minimum coupling loss refers to 100%tile on the coupling loss CDF curve which is sensitive to simulation and can fluctuate across different simulation runs.

The metric presented in the simlation results identifies the coupling loss from each interferering BS to each victim BS or UE as such can be used to identify a worst case at a certain probability of that parameter. The dynamic nature of the network set up means the CDF curves have a wide variation leading to the issue of selecting a suitable probability with which to extract the requirement. The results presented are summarized for the following CDF %tiles on the coupling loss curve based on Rel-99 interpretation.

- 99.9%tile

- 99.95%tile

- 99.99%tile

- 100%tile

However, it was argued that in the derivation of the requirement there may be need to consider the level of the interferer as well as the level of the wanted signal. It was assumed a 0.8 dB desensitidation of the wanted signal at the noise floor is acceptable. As the spatial parameters are static using a fixed desenistisation level is reasonable however in the case of  two dynamic AAS networks looking at the interferer level only gives an overly pessismistic result.

If we identify the worst case coupling loss cases as in figure 2.2.2.5-1, it is observed that when the worst case coupling occurs for the interferer the wanted UE’s are close to the victims BS. As such they are not receiving signals at the noise floor (and will not suffer 0.8 dB desenistisation).

Figure 2.2.2.5-1: Example of the 8 worst case UE placements (BS-BS, fundamental antenna model, rho=1)

It follows that UE’s near the cell edge, which are receiving low level signals close to the noise floor may not suffer large worst case inteferer levels.

The inteferer level and the wanted signal level both depend on the location of the UE as such both can be considered when selecting a metric or if only one is considered this effect should be represented in the selected CDF percentile.

### 2.3	Co-existence simulation results

#### 2.3.1	General

Simulation calibration results and early results focused on the CDF of the coupling loss, these results were presented below. Since there has been no consensus between the participating companies, it has not been possible to agree on the following:

- If the metric assumed in Release-99 methodology is appropriate for AAS BS evaluations.

- What is the correct metric and what is the correct percentile to extract the requirement from.

- If the antenna model for out of band antenna gain captured in Clause 7.3 in TR 38.922 is sufficient and what is the appropriate antenna correlation level.

#### 2.3.2	Simulation results - MCL

##### 2.3.2.1	BS-to-BS coupling: Urban Macro

Table 2.3.2.1-1: BS to BS CL (Nokia)

| %-tile | CL values with 0 correlation | CL values with 1 correlation | CL values with 0.5 correlation | CL values with scaled 0 correlation | CL values with scaled 1 correlation | CL values with scaled 0.5 correlation |
| --- | --- | --- | --- | --- | --- | --- |
| 99 | -86.7938 | -91.3974 | -87.8284 | -96.022 | -101.9742 | -97.2973 |
| 99.9 | -74.0343 | -76.2064 | -74.8746 | -83.4406 | -86.6269 | -84.3963 |
| 99.99 | -65.8391 | -64.9049 | -65.6347 | -76.6021 | -74.3620 | -75.6208 |
| 100 | -59.7523 | -48.1203 | -50.9161 | -70.5869 | -59.5737 | -62.3714 |

Table 2.3.2.1-2: BS to BS CL (Ericsson) with 100% grid shift

| %-tile | CL values with 1 correlation | CL values with 0.5 correlation |
| --- | --- | --- |
| 99.9 | -79.4 | -80 |
| 99.95 | -79 | -74.8 |
| 100 | -72.3 | -70.8 |

Table 2.3.2.1-3: BS to BS CL (Ericsson) with 50% grid shift

| %-tile | CL values with 1 correlation | CL values with 0.5 correlation |
| --- | --- | --- |
| 99.9 | -71.6 | -71.6 |
| 99.95 | -70.2 | -71.2 |
| 100 | -68.7 | -70.5 |

Table 2.3.2.1-4: BS to BS CL (ZTE)

| Signal type | Correlation level between sub-array | Percentile of BS2BS coupling loss CDF |  |  |
| --- | --- | --- | --- | --- |
|  |  | 99% | 99.9% | 99.99% |
|  |  | CL [dB] | CL [dB] | CL [dB] |
| 3rd harmonic signal | 50% | -94.8596 | -81.5196 | -73.5077 |
|  | 100% | -99.5072 | -83.5311 | -71.7873 |
|  | 0% | -93.7391 | -80.4932 | -72.8547 |
| 2nd harmonic signal | 50% | -88.3497 | -75.4640 | -66.5155 |
|  | 100% | -92.5781 | -76.9807 | -64.6391 |
|  | 0% | -87.3982 | -75.1114 | -66.0110 |

##### 2.3.2.2	BS-to-UE coupling: Dense Urban

Table 2.3.2.2-1: BS to UE CL (Nokia)

| %-tile | CL values with 0 correlation | CL values with 1 correlation |
| --- | --- | --- |
| 99 | -111.3788 | -110.0181 |
| 100 | -58.4692 | -48.7397 |

Table 2.3.2.2-2: BS to UE CL (Ericsson)

| %-tile | CL values with 0.5 correlation |
| --- | --- |
| 99.9 | -85.8 |
| 99.95 | -84.5 |
| 99.99 | -81.7 |

Table 1.3.2.2-3: BS to UE CL (ZTE)

| Signal type | Correlation level between sub-array | Percentile of BS2UE coupling loss CDF |  |  |
| --- | --- | --- | --- | --- |
|  |  | 99% | 99.9% | 99.99% |
|  |  | CL [dB] | CL [dB] | CL [dB] |
| 3rd harmonic signal | 50% | -117.0993 | -93.9449 | -77.9506 |
|  | 100% | -116.8091 | -94.3013 | -77.7408 |
|  | 0% | -116.4873 | -95.5220 | -78.6758 |
| 2nd harmonic signal | 50% | -117.1788 | -95.8344 | -81.7833 |
|  | 100% | -116.7631 | -94.4136 | -76.6655 |
|  | 0% | -117.2344 | -93.3777 | -78.7230 |

#### 2.3.3	Simulation results - Throughput loss

##### 2.3.2.1	BS-to-BS throughput loss: Urban Macro

Looking at the results for the 2 GHz (with 3rd harmonic) and 3 GHz (with 2nd harmonic), the throughput loss and MCL values were collected table 2.3.2.1-1:

Table 2.3.2.1-1: Throughput loss (and MCL) results (2k drops) (Huawei)

| Aggressor Freq (GHz) |  | 2 |  |  | 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| antenna model |  | Harmonic |  |  | Harmonic |  |  |
| Metric |  | correlation |  |  | correlation |  |  |
| type | Percentile | 1 | 0.5 | 0 | 1 | 0.5 | 0 |
| 5% TP Loss | Average | -15 | -21 | -21 | -22 | -21 | -21 |
|  | 5% | -30 | -25 | -25 | -31 | -30 | -30 |
|  | 1% | -34 | -37 | -36 | -40 | -37 | -37 |
|  | 0.10% | -47 | -40 | -40 | -47 | -44 | -43 |
| MCL (dB) | 99 | -98.1 | -91.4 | -91.4 | -91.4 | -86.6 | -86.6 |
|  | 99.9 | -81.6 | -76.8 | -76.8 | -75.2 | -76.3 | -76.3 |
|  | 100 | -52.1 | -64.1 | -64.2 | -54.3 | -66.6 | -67 |

If we consider the existing emission level is -52 dBm and the current MCL assumption is 67 dB, it is easier to compare the results by looking at the margin over the current specification.

Table 2.3.2.1-2: Throughput loss (and MCL) margin (2k drops) (Huawei)

| Aggressor Freq (GHz) |  | 2 |  |  | 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| antenna model |  | Harmonic |  |  | Harmonic |  |  |
| Metric |  | correlation |  |  | correlation |  |  |
| type | Percentile | 1 | 0.5 | 0 | 1 | 0.5 | 0 |
| 5% TP Loss | Average | 37 | 31 | 31 | 30 | 31 | 31 |
|  | 5% | 22 | 27 | 27 | 21 | 22 | 22 |
|  | 1% | 18 | 15 | 16 | 12 | 15 | 15 |
|  | 0.10% | 5 | 12 | 12 | 5 | 8 | 9 |
| MCL (dB) | 99 | 31.1 | 24.4 | 24.4 | 24.4 | 19.6 | 19.6 |
|  | 99.9 | 14.6 | 9.8 | 9.8 | 8.2 | 9.3 | 9.3 |
|  | 100 | -14.9 | -2.9 | -2.8 | -12.7 | -0.4 | 0 |

The worst case (100%) MCL results are not realistic as they cannot occur at the same time as a low power level wanted signal.

The usual metric used for outlier cases is the 5%-tile however in this case it can be seen there are huge margins even with this metric, the 1%-tile and even 0.1%-tile also show significant margin.

It can be noted that a similar level of margin is achieved if the 99.9% MCL metric is used. It seems that when the actual performance of the dynamic AAS BS networks are considered there is significant margin in the existing spurious emissions levels.

Using the simulation to effectively search for the worst-case coupling highlights a worst-case interference case which indicates a pessimistic result as it does not consider the level of the wanted signal. The wanted signal level is directly related to the interferer level as they are both related to the victim UE location. Whilst this pessimistic approach can be regarded as safe it does result in an over specified parameter compared to the original static network approach. When both the wanted signal and the interferer signal are considered the impact on SNR or throughput can be identified and provides a more realistic metric for such a system, however there is no agreement on the exact metric of the level at which the requirement can be extracted.

Table 2.3.2.1-3: Required coexistence spurious emission based on throughput loss metric (ZTE)

| Signal type | Correlation level between sub-array | ACIR offset in dB [NOTE 1] | Coexistence spurious emission dBm/MHz | NOTE |
| --- | --- | --- | --- | --- |
| 3rd harmonic signal | 50% | 21 | -34.4600 | 49-10*log10(100)-42.46-21 |
|  | 100% | 22 | -35.4600 | 49-10*log10(100)-42.46-22 |
|  | 0% | 19 | -32.4600 | 49-10*log10(100)-42.46-19 |
| 2nd harmonic signal | 50% | 26 | -39.4600 | 49-10*log10(100)-42.46-26 |
|  | 100% | 26 | -39.4600 | 49-10*log10(100)-42.46-26 |
|  | 0% | 24 | -37.4600 | 49-10*log10(100)-42.46-24 |
| NOTE 1: the baseline ACIR 42.46dB is based assumption of ACLR as 45dB and ACS as 46dB. |  |  |  |  |

##### 2.3.2.2	BS-to-UE throughput loss: Dense Urban

Table 2.3.2.2-1: Required coexistence spurious emission based on throughput loss metric (ZTE)

| Signal type | Correlation level between sub-array | ACIR offset in dB | Coexistence spurious emission dBm/MHz | NOTE |
| --- | --- | --- | --- | --- |
| 3rd harmonic signal | 50% | 0 | -15.73 | 37dBm-10*log10(100)-32.73=-15.73dBm/MHz |
|  | 100% | 0 | -15.73 | 37dBm-10*log10(100)-32.73=-15.73dBm/MHz |
|  | 0% | 0 | -15.73 | 37dBm-10*log10(100)-32.73=-15.73dBm/MHz |
| 2nd harmonic signal | 50% | 0 | -15.73 | 37dBm-10*log10(100)-32.73=-15.73dBm/MHz |
|  | 100% | 0 | -15.73 | 37dBm-10*log10(100)-32.73=-15.73dBm/MHz |
|  | 0% | 0 | -15.73 | 37dBm-10*log10(100)-32.73=-15.73dBm/MHz |

### 2.4 	Summary

The objective was to re-evaluate the co-existence transmitter spurious emission limits, considering potential improvements to the transmitter co-existence spurious emission requirement limits for NR operating bands in the upper region of FR1, with priority above 5.9 GHz. The task was to investigate whether the higher pathloss found at 6.4 GHz had any effect on the co-existence in the same geographical area spurious emissions requirements. More specifically, the evaluation scenario was focused on 2 GHz aggressor carrier, and 6.4 GHz victim.

In order to answer that question, it became necessary to consider methodology used to derive the original requirement and apply that to the higher frequency use case, where to maintain the same BS deployment grid to overcome the increased path loss due to frequency high gain AAS BS systems are used. These AAS BS systes are dynamic in nature compared to the static sceanrios considered in the original Rel-99 MCL-based analysis.

Since there has been no consensus between the participating companies, it has not been possible to agree on the following:

- If the performance metric assumed in Release-99 methodology is appropriate for AAS BS evaluations.

- If the antenna model for out of band antenna gain captured in Clause 7.3 in TR 38.922 [3] is sufficient and what should be the antenna correlation level.

Based on the above, 3GPP RAN WG4 re-evaluation did not conclude to modify the existing TX spurious emissions co-existence requirements limits for upper region of FR1. Results presented by some companies indicated that the existing requirement emission levels are safe, but potentially are over-restrictive (margin presented varied across companies due lack of conclusion on the requirement derivation metric, and its levels).

Further analysis of the dynamic AAS BS networks may be required, to investigate their operation using a simulation of both the interferer and the wanted signal, which could potentially lead to a different Tx spurious co-ex limit requirement.

# 3	References

[1]	RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, LS, ETSI TC MSG/TFES

[2]	3GPP TR 25.942, “Radio Frequency (RF) system scenarios”

[3]	3GPP TR 38.922, “Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz and 14800-15350 MHz”

[4]	3GPP TR 38.921, “Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz”

[5]	RP-252578, NR base station (BS) RF requirement evolution for FR1/FR2 and testing t, WID, NR_BS_RF_req_evo-Core