# 3GPP TSG-RAN WG4 Meeting #117                           R4-2523112

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.108 | CR | draftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big draft CR on 36.108 for Rel-19 IoT-NTN Phase 3 demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel -19, a new NPUSCH demodulation requirement for IoT-NTN with OCC was introduced. Therefore, new demodulation requirements shall be added in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new demodulation requirements for Rel-19 IoT-NTN Phase 3. This draft big CR contains the following draft CRs endorsed in RAN4#117. R4-2523073; draft CR: NPUSCH format 1 requirements for NTN. R4-2521473; Draft CR on FRC for 15KHz for IoT NTN phase 3 in TS 36.108. |
| Consequences if not approved: |  | The new demodulation reuqirements will be missing in 36.108. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TSTR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.181 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < START OF CHANGE>

8.5	Performance requirements for Narrowband IoT

8.5.1	Requirements for NPUSCH format 1

8.5.1.1	Requirements

The performance requirement of NPUSCH format 1 is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A.7. The performance requirements assume HARQ retransmissions.

An NB-IoT Base Station supports 15 kHz subcarrier spacing requirements, or 3.75 kHz subcarrier spacing requirements, or both.

For 15kHz subcarrier spacing multi-subcarrier, the demodulation requirements apply for the supported number of subcarriers.

For 15kHz subcarrier spacing with symbol-level length-2 OCC, the demonstration requirements apply for the base stations supporting 15kHz subcarrier spacing with symbol-level length-2 OCC.

For3.75kHz subcarrier spacing with slot-level length-2 OCC, the demodulation requirements apply for the base stations supporting 3.75kHz subcarrier spacing with slot-level length-2 OCC.

Table 8.5.1.1-1: Test parameters

| Parameter | Value |
| --- | --- |
| Maximum number of HARQ transmissions | 4 |
| RV sequence | RV0, RV2 |

8.5.1.1.1	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput stated in table 8.5.1.1.1-1 for the single-subcarrier of 3.75KHz subcarrier spacing, and in table 8.5.1.1.1-2 for multi-subcarrier of 15KHz subcarrier spacing at the given SNR for 1Tx.

Table 8.5.1.1.1-1: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN-TDLA100-1 | A7-1 | 128 Hz | 256 | 4 | 70% | -2.0 |
| 1 | 1 | 3.75KHz | 1 | NTN-TDLC5-1 | A7-1 | 128 Hz | 256 | 4 | 70% | -3.6 |
| 1 | 2 | 3.75KHz | 1 | NTN-TDLA100-1 | A7-1 | 128 Hz | 256 | 4 | 70% | -5.4 |
| 1 | 2 | 3.75KHz | 1 | NTN-TDLC5-1 | A7-1 | 128 Hz | 256 | 4 | 70% | -6.5 |

Table 8.5.1.1.1-2: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -3.2 |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -4.0 |
| 1 | 2 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -7.1 |
| 1 | 2 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -7.8 |

Table 8.5.1.2.1-1: Minimum requirements for NPUSCH format 1 with symbol-level length-2 OCC, 200 kHz Channel Bandwidth, 3.75 kHz subcarrier spacing, single subcarrier, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | OCC sequence index | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN-TDLA100-1 | A7-1 | 128 Hz | 256 | 2 | 0 | 70% | TBD |
|  |  |  |  |  |  |  |  |  | 1 | 70% | TBD |
| 1 | 2 | 3.75KHz | 1 | NTN-TDLA100-1 | A7-1 | 128 Hz | 256 | 2 | 0 | 70% | TBD |
|  |  |  |  |  |  |  |  |  | 1 | 70% | TBD |
| Note 1: The total number of slots in NPUSCH transmission after transmission after OCC is applied Repetition number* OCC length *NslotsULNRU, where NslotsUL is number of UL slot and NRU is the number of RU. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.5.1.2.1-2: Minimum requirements for NPUSCH format 1 with slot-level length-2 OCC, 200 kHz Channel Bandwidth, 15 kHz subcarrier spacing, single subcarrier, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | OCC sequence index | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 1 | NTN-TDLA100-1 | A7-3 | 128 Hz | 256 | 8 | 0 | 70% | TBD |
|  |  |  |  |  |  |  |  |  | 1 | 70% | TBD |
| 1 | 2 | 15KHz | 1 | NTN-TDLA100-1 | A7-3 | 128 Hz | 256 | 8 | 0 | 70% | TBD |
|  |  |  |  |  |  |  |  |  | 1 | 70% | TBD |
| Note 1: The total number of slots in NPUSCH transmission after transmission after OCC is applied Repetition number* OCC length *NslotsULNRU, where NslotsUL is number of UL slot and NRU is the number of RU. |  |  |  |  |  |  |  |  |  |  |  |

## < NEXT OF CHANGE>

8.5.2.2	ACK missed detection requirements

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent per NPUSCH format 2 transmission.

8.5.2.2.1	Minimum requirements

The ACK missed detection probability shall not exceed 1% at the SNR given in table 8.5.2.2.1-1 and table 8.5.2.2.1-2 for 1Tx case.

Table 8.5.2.2.1-1: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Propagation conditions (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 3.75KHz | 64 Hz | 256 | 4 | 1.8 |
| 1 | 2 | NTN-TDLA100-1 | 1 | 3.75KHz | 64 Hz | 256 | 4 | -3.9 |

Table 8.5.2.2.1-2: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Propagation conditions (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 15KHz | 16 Hz | 32 | 16 | 5.1 |
| 1 | 2 | NTN-TDLA100-1 | 1 | 15KHz | 16 Hz | 32 | 16 | -3.3 |

## < NEXT OF CHANGE>

8.5.2.2	ACK missed detection requirements

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent per NPUSCH format 2 transmission.

8.5.2.2.1	Minimum requirements

The ACK missed detection probability shall not exceed 1% at the SNR given in table 8.5.2.2.1-1 and table 8.5.2.2.1-2 for 1Tx case.

Table 8.5.2.2.1-1: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Propagation conditions (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 3.75KHz | 64 Hz | 256 | 4 | 1.8 |
| 1 | 2 | NTN-TDLA100-1 | 1 | 3.75KHz | 64 Hz | 256 | 4 | -3.9 |

Table 8.5.2.2.1-2: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Propagation conditions (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 15KHz | 16 Hz | 32 | 16 | 5.1 |
| 1 | 2 | NTN-TDLA100-1 | 1 | 15KHz | 16 Hz | 32 | 16 | -3.3 |

## < NEXT OF CHANGE>

A.7	Fixed Reference Channels for NB-IoT NPUSCH format 1

A.7.1	One PRB

Table A.7.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A7-1 | A7-2 | A7-3 |
| --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 | 15 |
| Number of allocated subcarriers | 1 | 12 | 1 |
| Diversity | No | No | No |
| Modulation | BPSK | QPSK | BPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 | 0/1 |
| Payload size (bits) | 32 | 136 | 32 |
| Allocated resource unit | 2 | 1 | 2 |
| Code rate (target) | 1/3 | 2/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.56 | 0.29 |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 | 96 |
| Total symbols per resource unit | 96 | 144 | 96 |
| Channel estimation length (ms) Note 1 | 16 | 2 (when repetition = 2) 4 (when repetition > 2) | 4 |

## < End OF CHANGE>