3GPP TSG-RAN WG4 Meeting #117	R4-2522402

Dallas, US, November 17th – 21st, 2025

Source: 	KDDI, Murata, Samsung

Title:	TP to TR 38.746 for adding HPUE feature to CA_n18-n40-n77

Agenda item:	5.7.3

Document for:	Approval

- Introduction

This paper is a Text Proposal to TR 38.746 for adding HPUE UL notes to CA_ n18A-n40A-n77A with BCS4 and 5.

The TP for the PC3 case for this 3-band combination was submitted to this meeting in R4-2522354. All the PC2 fallbacks for this band combination have been approved except CA_n18-n40, which was submitted to this meeting in R4-2521657.

- Text proposal

< Start of Text Proposal>

# 6	High Power UE CA of 3 bands DL and 1 band UL and 2 bands UL

## 6.x	CA_ n18-n40-n77

### 6.x.1	Configurations

Table 6.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n18A-n40A-n77A | n407,9 n777,9 CA_n18A-n40A7,9 CA_n18A-n77A7,9 CA_n40A-n77A7,9 | n18 | n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.x.2	Maximum output power

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18-n40 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n18-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n40-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

Table 6.X.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 1.5 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18-n40 | Case b | 29dBm | 23dBm | 29dBm |
| CA_n18-n77 | Case b | 29dBm | 23dBm | 29dBm |
| CA_n40-n77 | Case a | 29dBm | 26dBm | 26dBm |
|  | Case b | 29dBm | 23dBm | 29dBm |
|  | Case c | 29dBm | 29dBm | 23dBm |
|  | Case d | 29dBm | 26dBm | 29dBm |
|  | Case e | 29dBm | 29dBm | 26dBm |

### 6.x.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink. ,

For single UL n40 and n77, there is neither IMD issue nor harmonic mixing issue, hence there is no MSD for the either UL n40 nor UL n77 for both PC2 and PC1.5.

#### 6.x.3.1	Power class 2 case a, b, c, d

Based on analysis, for the three 2-bands interband UL cases, IMD3 etc. exist.

- IMD3 and IMD5 due to UL CA_n40A-n77A falls into n18 DL. The MSD value corresponding to this IMD3 is calculated by adding 9dB to the PC3 MSD value, resulting in 24.2dB.

- IMD3 due to UL CA_n18A-n40A falls into n77 DL. The MSD value corresponding to this IMD3 is calculated by adding 9dB to the PC3 MSD value, resulting in 25.1dB.

- IMD3 due to UL CA_n18A-n77A falls into n40 DL. The MSD value corresponding to this IMD3 is calculated by adding 9dB to the PC3 MSD value, resulting in 22.2dB.

The test points and the MSD values are shown in Table 6.x.3-1.

Table 6.x.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n40-n77 | n18 | N/A | 5 | N/A | 862.5 | 24.2 | FDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n77 | 3742.5 | 10 | 50 | 3742.5 | N/A | TDD | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | N/A | FDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3882.5 | 25.1 | TDD | IMD3 |
|  | n18 | 817.5 | 5 | 25 | 862.5 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2367.5 | 22.2 | TDD | IMD3 |
|  | n77 | 4002.5 | 10 | 50 | 4002.5 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 6.x.3.2	Power class 1.5 case a, b, c, d, e

Based on calculation, for the three 2-band interband UL cases, IMD3 etc. exist. The test points and the MSD values are shown in Table 6.x.3-2.

- IMD3 and IMD5 due to UL CA_n40A-n77A falls into n18 DL. The MSD value corresponding to this IMD3 is calculated by adding 18dB to the PC3 MSD value, resulting in 33.2dB.

- IMD3 due to UL CA_n18A-n40A falls into n77 DL. The MSD value corresponding to this IMD3 is calculated by adding 18dB to the PC3 MSD value, resulting in 34.1dB.

- IMD3 due to UL CA_n18A-n77A falls into n40 DL. The MSD value corresponding to this IMD3 is calculated by  adding 18dB to the PC3 MSD value, resulting in 31.2dB.

Table 6.x.3-2: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n40-n77 | n18 | N/A | 5 | N/A | 862.5 | 33.2 | FDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n77 | 3742.5 | 10 | 50 | 3742.5 | N/A | TDD | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | N/A | FDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3882.5 | 34.1 | TDD | IMD3 |
|  | n18 | 817.5 | 5 | 25 | 862.5 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2367.5 | 31.2 | TDD | IMD3 |
|  | n77 | 4002.5 | 10 | 50 | 4002.5 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

< End of Text Proposal>

- References

[1] 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[2] 	RP-241679_New WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR SUL (supplementary uplink)

[3] R4-2522354, TP to TR 38.719-03-01 for adding CA_n18A-n40A-n77A

[4] R4-2505143, WF on MSD simplification for HPUE

[5] R4-2514599, Draft CR for 38.101-1 on enabling HPUE support for band combinations without HPUE applicability notes in clause 5

[6] R4-2521657, Draft CR to TS38.101-1 Rel-19 Adding HPUE notes to CA_n18-n40 with BCS4 and 5