3GPP TSG-RAN4 Meeting #117	R4-2522992

# Dallas, USA, 17 – 21 Novermber, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on intra-frequency measurement delay test cases for RedCap NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on WF R4-2515056, test cases for intra-frequency measurement delay need to be introduced for RedCap NTN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce test cases for intra-frequency measurement delay for RedCap NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intra-frequency measurement delay for RedCap NTN are not verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.X.5.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## A.X.5	Measurement procedure

### A.X.5.1	Intra-frequency Measurements

#### A.X.5.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE

A.X.5.1.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2E.5.1 and 9.2E.5.2.

A.X.5.1.1.2	Test parameters

The test parameters and procedure in clause A.14.5.1.1.2 apply, except that the supported test configurtions are defined in table A.X.5.1.1.2-1, and NR Cell specific test parameters in Table A.X.5.1.1.2-2 replaces the corresponding parameters in Table A.14.5.1.1.2-3.

Table A.X.5.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.5.1.1.2-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1, 3 | SSC.1 |  | NSC.1 |  |
|  |  | 2, 4 | SSC.2 |  | NSC.2 |  |

A.X.5.1.1.3	Test Requirements

The test requirements in clause A.14.5.1.1.3 apply for this test.

#### A.X.5.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE

A.X.5.1.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2E.5.1 and 9.2E.5.2.

A.X.5.1.2.2	Test parameters

The test parameters and procedure in clause A.14.5.1.1.2 apply, except that the supported test configurtions are defined in table A.X.5.1.2.2-1, and NR Cell specific test parameters in Table A.X.5.1.2.2-2 replaces the corresponding parameters in Table A.14.5.1.1.2-3.

Table A.X.5.1.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.5.1.2.2-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1, 3 | SSC.1 |  | NSC.1 |  |
|  |  | 2, 4 | SSC.2 |  | NSC.2 |  |

A.X.5.1.2.3	Test Requirements

The test requirements in clause A.14.5.1.1.3 apply for this test.

#### A.X.5.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE

A.X.5.1.3.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2E.5.1 and 9.2E.5.2.

A.X.5.1.3.2	Test parameters

The test parameters and procedure in clause A.14.5.1.2.2 apply, except that the supported test configurtions are defined in table A.X.5.1.3.2-1, and NR Cell specific test parameters in Table A.X.5.1.3.2-2 replaces the corresponding parameters in Table A.14.5.1.2.2-3.

Table A.X.5.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.5.1.3.2-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1, 3 | SSC.1 |  | NSC.1 |  |
|  |  | 2, 4 | SSC.2 |  | NSC.2 |  |

A.X.5.1.2.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test. X=1520 for test configuration 2 and if UE indicates ‘n1’ for maxNumber-NGSO-SatellitesWithinOneSMTC, otherwise X=920.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Y ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test. Y=15360 for test configuration 2 and if UE indicates ‘n1’ for maxNumber-NGSO-SatellitesWithinOneSMTC, otherwise Y=7680.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.X.5.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE

A.X.5.1.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2E.5.1 and 9.2E.5.2.

A.X.5.1.2.2	Test parameters

The test parameters and procedure in clause A.14.5.1.2.2 apply, except that the supported test configurtions are defined in table A.X.5.1.4.2-1, and NR Cell specific test parameters in Table A.X.5.1.4.2-2 replaces the corresponding parameters in Table A.14.5.1.2.2-3.

Table A.X.5.1.4.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.5.1.4.2-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1, 3 | SSC.1 |  | NSC.1 |  |
|  |  | 2, 4 | SSC.2 |  | NSC.2 |  |

A.X.5.1.2.4	Test Requirements

The test requirements in clause A.14.5.1.2.3 apply for this test.

#### A.X.5.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE

A.X.5.1.5.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2E.5.1 and 9.2E.5.2.

A.X.5.1.5.2	Test parameters

The test parameters and procedure in clause A.14.5.1.3.2 apply, except that the supported test configurtions are defined in table A.X.5.1.5.2-1, and NR Cell specific test parameters in Table A.X.5.1.5.2-2 replaces the corresponding parameters in Table A.14.5.1.3.2-3.

Table A.X.5.1.5.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.5.1.5.2-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1, 3 | SSC.1 |  | NSC.1 |  |
|  |  | 2, 4 | SSC.2 |  | NSC.2 |  |

A.X.5.1.5.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X = 1040 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.X.5.1.6	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE

A.X.5.1.6.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2E.5.1 and 9.2E.5.2.

A.X.5.1.6.2	Test parameters

The test parameters and procedure in clause A.14.5.1.3.2 apply, except that the supported test configurtions are defined in table A.X.5.1.6.2-1, and NR Cell specific test parameters in Table A.X.5.1.6.2-2 replaces the corresponding parameters in Table A.14.5.1.3.2-3.

Table A.X.5.1.6.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.5.1.6.2-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1, 3 | SSC.1 |  | NSC.1 |  |
|  |  | 2, 4 | SSC.2 |  | NSC.2 |  |

A.X.5.1.6.3	Test Requirements

The test requirements in clause A.14.5.1.3.3 apply for this test.

#### A.X.5.1.7	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access for 1 Rx UE

A.X.5.1.7.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2E.6.1 and 9.2E.6.2.

A.X.5.1.7.2	Test parameters

The test parameters and procedure in clause A.14.5.1.4.2 apply, except that the supported test configurtions are defined in table A.X.5.1.7.2-1, and NR Cell specific test parameters in Table A.X.5.1.7.2-2 replaces the corresponding parameters in Table A.14.5.1.4.2-3..

Table A.X.5.1.7.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.5.1.7.2-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1, 3 | SSC.1 |  | NSC.1 |  |
|  |  | 2, 4 | SSC.2 |  | NSC.2 |  |

A.X.5.1.7.3	Test Requirements

The test requirements in clause A.14.5.1.4.3 apply for this test.

#### A.X.5.1.8	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access for 2 Rx UE

A.X.5.1.8.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2E.6.1 and 9.2E.6.2.

A.X.5.1.8.2	Test parameters

The test parameters and procedure in clause A.14.5.1.4.2 apply, except that the supported test configurtions are defined in table A.X.5.1.8.2-1, and NR Cell specific test parameters in Table A.X.5.1.8.2-2 replaces the corresponding parameters in Table A.14.5.1.4.2-3.

Table A.X.5.1.8.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.5.1.8.2-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1, 3 | SSC.1 |  | NSC.1 |  |
|  |  | 2, 4 | SSC.2 |  | NSC.2 |  |

A.X.5.1.8.3	Test Requirements

The test requirements in clause A.14.5.1.4.3 apply for this test.

#### A.X.5.1.9	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access for 1 Rx UE

A.X.5.1.9.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2E.6.1 and 9.2E.6.2.

A.X.5.1.9.2	Test parameters

The test parameters and procedure in clause A.14.5.1.5.2 apply, except that the supported test configurtions are defined in table A.X.5.1.9.2-1, and NR Cell specific test parameters in Table A.X.5.1.9.2-2 replaces the corresponding parameters in Table A.14.5.1.5.2-3.

Table A.X.5.1.9.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.5.1.9.2-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1, 3 | SSC.1 |  | NSC.1 |  |
|  |  | 2, 4 | SSC.2 |  | NSC.2 |  |

A.X.5.1.9.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 7680 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.X.5.1.10	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access for 2 Rx UE

A.X.5.1.10.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2E.6.1 and 9.2E.6.2.

A.X.5.1.10.2	Test parameters

The test parameters and procedure in clause A.14.5.1.5.2 apply, except that the supported test configurtions are defined in table A.X.5.1.10.2-1, and NR Cell specific test parameters in Table A.X.5.1.10.2-2 replaces the corresponding parameters in Table A.14.5.1.5.2-3.

Table A.X.5.1.10.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.5.1.10.2-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1, 3 | SSC.1 |  | NSC.1 |  |
|  |  | 2, 4 | SSC.2 |  | NSC.2 |  |

A.X.5.1.10.3	Test Requirements

The test requirements in clause A.14.5.1.5.3 apply for this test.

#### A.X.5.1.11	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access for 1 Rx UE

A.X.5.1.11.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2E.6.1 and 9.2E.6.2.

A.X.5.1.11.2	Test parameters

The test parameters and procedure in clause A.14.5.1.6.2 apply, except that the supported test configurtions are defined in table A.X.5.1.11.2-1, and NR Cell specific test parameters in Table A.X.5.1.11.2-2 replaces the corresponding parameters in Table A.14.5.1.6.2-3.

Table A.X.5.1.11.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.5.1.11.2-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1, 3 | SSC.1 |  | NSC.1 |  |
|  |  | 2, 4 | SSC.2 |  | NSC.2 |  |

A.X.5.1.11.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1480 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.X.5.1.12	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access for 2 Rx UE

A.X.5.1.12.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2E.6.1 and 9.2E.6.2.

A.X.5.1.12.2	Test parameters

The test parameters and procedure in clause A.14.5.1.6.2 apply, except that the supported test configurtions are defined in table A.X.5.1.12.2-1, and NR Cell specific test parameters in Table A.X.5.1.12.2-2 replaces the corresponding parameters in Table A.14.5.1.6.2-3.

Table A.X.5.1.12.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.X.5.1.12.2-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1, 3 | SSC.1 |  | NSC.1 |  |
|  |  | 2, 4 | SSC.2 |  | NSC.2 |  |

A.X.5.1.12.3	Test Requirements

The test requirements in clause A.14.5.1.5.3 apply for this test.

<End of Change 1>