3GPP TSG-RAN WG4 Meeting # 118	R4-2600366

Gothenburg, SE., 9th – 13th, Feb. 2026

Source: 	Samsung

Title: 	Discussion on Tx and Rx requirements for 6GR

Agenda Item:	8.3.2

Document for:	Approval

- Introduction

This paper presents our views on following two aspects, for the issues identified and captured in WF [1]

- Tx requirements

2)  Rx requirements

- 2. Discussion

2.1 Tx requirements

It was agreed that 6G PA modelling will be discussed in the system parameter thread, which is mainly for RAN1 waveform evaluation purposes, and early feedback to RAN1 is required. RAN4 may further evolve this Power Amplifier (PA) model for RAN4-centric studies such as MPR/A-MPR evaluation. In addition to PA models, Tx impairment assumptions, including carrier leakage and image rejection, are also critical factors that should be determined early. As noted by companies in [2][3], the -28 dB value for both parameters have been in use for over 20 years, and it is time to revisit this, especially to support the higher-order modulation scheme 1024QAM. To accommodate different UE types (in particular low-tier and low-cost UEs), we propose adopting -35 dB for both carrier leakage and image rejection for frequencies up to around 7GHz (i.e., up to 8.4GHz).

Proposal #2.1-1: For frequencies up to around 7GHz (i.e., up to 8.4GHz), adopt -35 dB for carrier leakage and -35 dB for image rejection.

There have been proposals [10][11] regarding “fast LO retuning”. By setting the LO at the center of allocated RB, it avoids the situations where the IMD product of signal and its image or the CIM3/5 falling in certain protected frequency range which requires high MPR/A-MPR. This approach offers the advantage of reduced MPR/A-MPR hence enhancing UL coverage. Additionally, we have observed that this would also enable some power saving benefits by reducing the clock rate of baseband and D/A conversion.

However, it is important to note that the longer the LO switching takes, the long the interruption. Excessive interruption should be avoided to ensure optimal performance. Therefore, we recommend a detailed discussion on this concept, including aspects such as LO retuning time and interruption time.

Observation #2.1-1: In terms of the benefits of “fast LO retuning”, apart from the reduced MPR/A-MPR, there is also a reduction in power consumption achieved by reducing the clock rate of the baseband and D/A conversion.

Proposal #2.1-2: We recommend a detailed discussion on the concept of “fast LO retuning”, including aspects such as LO retuning time and interruption time.

To date, there are over 20 MPR tables in TS 38.101-1, considering the following factors.

- Different power classes (PC3, PC2, PC1.5, PC1, [PC1bis])

- Different UE architectures (TxD, dualPA, 1LO1PA),

- Different modes (single carrier, UL MIMO, UL NC CA, UL C CA with contiguous RB, UL C CA with non-contiguous RB),

- Different Tx Chain numbers (1~4 Tx)

- Different form factor (For PC1.5 and above, there are separate MPR requirements assuming different antenna isolation for handheld and FWA)

- Different use case (e.g.,V2X)

MPR evaluation is widely recognized as time-consuming for both measurement and simulation approaches, especially in case of multiple Tx chains. Given that reverse-IMD and counter-IMD cannot be accurately modeled, measurements are considered a more reliable method than simulations, albeit more time consuming.

Regarding different RF architectures for MPR evaluation, for instance, for PC2 UL contiguous CA with contiguous RB allocation, MPRs were evaluated across 3 different RF architectures (not just the widely implemented one), with MPR differences of 0.5–1 dB among these architectures. While for PC1.5 UL contiguous CA with contiguous RB allocation, only one architecture was studied as the referenced architecture due to time constraints; requirements were defined based on this reference architecture, while other implementations are not precluded provided they can meet the specified requirements.

Regarding the different Tx Chain number for single carrier operation with the same power class, during discussion on 3Tx UL MIMO, a brief evaluation was conducted. Ultimately, it was agreed to reuse the PC2 2Tx MPR requirements for PC2 3Tx. For FWA PC1.5 2Tx and 4Tx, the MPR difference is approximately 1–1.5 dB for edge RB allocation, and 0.5–1 dB for outer and inner RB allocations.

Above is kind of lesson and learn from 5G. We therefore propose streamlining MPR requirements when the performance differences are not significant across various RF architectures, Tx chain counts, and other relevant factors.

Proposal #2.1-3: Strive to streamline MPR requirements when the performance differences are not significant across various RF architectures, Tx chain counts, and other relevant factors.

- For instance, for one device type, with same power class and mode (i.e., single carrier, or C/NC CA), strive to specify only one set of MPR requirements. This doesnot preclude other architecture implementations (i.e., those not selected as reference architecture for deriving requirements), provided the specified requirements are met.

2.2 Rx requirements

2.2.1 Other Rx requirements

It was agreed in [4] that RAN4 shall reuse the prefix concept from NR for 6G. It is expected that many 5G bands will be refarmed for 6G. It was agreed in [5] to study two aspects. First, whether the Rx requirements remain relevant for respective 6G bands up to 100 MHz. Second, how to define relevant Rx requirements for CBWs greater than 100 MHz.

It was proposed in [6] to evaluate if the narrowband blocking requirements, introduced to protect 5G/4G from GSM, is still needed. From our knowledge, GSM is still in use in some European countries, and it remains unclear whether all operators will completely decommission GSM before the first 6G specifications are finalized. Therefore, narrowband blocking requirements should be retained.

Observation #2.2-1: It remains unclear whether the GSM network will be fully decommissioned prior to the finalization of the first 6G specification.

In our view, other Rx requirements remain relevant for the respective bands with a CBW of up to 100 MHz. The same requirements framework can be extended to 200 MHz for the existing FR1, as well as to the band around 7 GHz (i.e., up to 8.4GHz) with a CBW of up to 200 MHz.

Proposal #2.2-1: All existing Rx requirements remain relevant for respective bands with a CBW of up to 100MHz. Same requirements framework/concept can be applicable to frequencies up to around 7GHz (i.e., up to 8.4GHz) with a CBW of up to 200MHz.

The most essential requirement for Rx is Reference Sensitivity(REFSENS), as many other Rx requirements are built on top of it. For 5G, REFSENS is calculated by the formula: REFSENS = -174 + 10lgCBW + NF + SNR + IM - Gdiv. For 6G, we think this equation should still be used for REFSENS calculation, at least for frequencies up to around 7GHz (i.e., up to 8.4GHz) with a CBW of up to 200MHz. This is because for at least up to around 7GHz(i.e., up to 8.4GHz), UE is still expected to have a conducted interface, assuming an isotropic radiation pattern antenna and without beamforming.. The Signal-to-Noise Ratio(SNR) very likely would remain at -1dB, but this will need to be confirmed through further evaluation by RAN1. Parameters such as Noise figure (NF) and Implementation margin (IM) need to be discussed further. We understand there is technology advancement leading to better NF and IM, while it is probably too optimistic to tighten NF as 6G UE are expected to support more bands and CA hence more complex duplexer as a contributor for worst NF.

Additionally, as well summarised in [7], “UE’s are nowadays quite well optimised for REFSENS, so in general it can be assumed that the real REFSENS would not be materially improved even if REFSENS is tightened” and “If REFSENS is tightened, many requirements needs to be re-evaluated for both single carrier and CA”. We are unsure if it is desirable to essentially redefine all requirements, as this is unlikely to significantly improve the implementation in the field.

As [8] well summarized, and quoted here, “ΔTIB/ΔRIB are rather tedious CA requirements which have not been practically leveraged by most UE implementations nor precisely accounted in conformance tests”. In RAN4 permanent reference document (PRD) clause 5.4 developed in the late Rel-19 stage, much more simplified rules have been captured in an effort to replace the case-by-case study/specification for ΔTIB/ΔRIB requirements or at least to provide aligned rules. However, these simplified rules are currently only documented in PRD, while the extensive pages for ΔTIB/ΔRIB requirements for CA are still remained in specification at least in Rel-19 and most likely also in Rel-20. For 6G, it is right time to review whether the justification of ΔTIB/ΔRIB still stands for CA. They were initially motivated by the consideration of the needed additional RF components, such as multi-plexer, to support CA. However, it is important to note that single carrier actually passes through the same RF components as a carrier within CA. As such, the original motivation no longer holds. In our view, ΔRIB can be removed from CA requirements, consequently the equivalent effect as ΔRIB should be taken into account in single carrier REFSENS. As such, IM will be increased compared to 5G if other factors doesnot change. However, to avoid the re-evaluation of the REFSENS and all relevant excessive requirements, we propose IM keep unchanged, for simplicity.

Proposal #2.2-2: For the derivation of REFSENS in the frequency range up to around 7GHz (i.e., up to 8.4GHz) with a CBP of up to 200MHz, the formula "REFSENS = -174 + 10lgCBW + NF + SNR + IM - Gdiv" shall still be adopted.

- NF and IM remains unchanged compared to 5G

Proposal #2.2-3: Remove ΔRIB from CA requirement

Regarding how to specify 6G REFSENS in a scalable manner in terms of different Rx Chain count, in our view, the 5G approach, where REFSENS is defined for all bands for 2Rx, with delta values added for 4Rx, 6Rx,8Rx, and 1Rx based on 2Rx, is already scalable. The values are not per band but per frequency range. Thus, the key discussion point here is likely whether 2Rx should remain the "baseline" for REFSENS requirements, with delta values added for other Rx counts. We suggest retaining 2Rx REFSENS as the baseline, given that it is not practical to mandate all kinds of UEs across all bands to support 4Rx or higher.

Proposal #2.2-4: The 2Rx REFSENS shall remain the baseline, with delta values applied to it to derive REFSENS for other Rx count such as 4Rx, 6Rx, 8Rx and 1Rx.

Regarding different power classes, they have no impact on TDD REFSENS but FDD REFSENS. In 5G, the HPUE FDD reference value is derived from PC3 by adding delta values, with consideration of different numbers of UL Tx chains (1Tx or 2Tx). For 6G, we still believe PC3 should serve as the basis for FDD REFSENS, as it is expected to be the default power class for FDD bands. For simplicity, it is preferable to have a single set of requirements regardless of the number of Tx chains.

Proposal #2.2-5: For FDD REFSENS, PC3 is the basis, and delta values to higher power class can be added to derive HPUE REFSENS.

- For the same power class, only one set of requirements is defined, regardless of the number of Tx chains.

2.2.2 MSD

The comprehensive WF for MSD is copied below.

According to the WF guidance, the study on Maximum sensitivity degradation (MSD) will proceed step-by-step, with the first priority being to determine whether MSD will still be introduced in 6G. In our opinion, MSD remains meaningful, even though it is typically applied to specific configuration, often the worst-case scenarios. It can still provide valuable insights into the degradation trend of the victim band to a certain extent, it is helpful to give a picture on which combos have less degradation in general.

Proposal #2.2-6: MSD requirements should be introduced in 6G.

Several potential framework for 6G MSD were identified and captured in our paper in last meeting, copied as following.

According to the WF, the step 2 is to discuss in UE RF thread whether OTA testing should be considered, in our understanding OTA testing here refers to the real OTA testing rather than emulating the OTA attribute by antenna coupling.

More comprehensive analysis are presented in our OTA companion paper [9] of this meeting, the observations and proposal is replicated as following.

Observation #2.2-2: Radiated MSD testing at single direction e.g. based on EIS is not feasible because the EIS degradation not only contains MSD de-sense but also antenna gain variation between single carrier operation and multi-carrier operation.

Observation #2.2-3: Radiated MSD testing based on TRS is also questionable because the TRS degradation not only contains MSD de-sense but also antenna efficiency variation between single carrier operation and multi-carrier operation.

Observation #2.2-4: The reference point for MSD requirement is the specified REFSENS requirement rather than the UE’s real sensitivity performance, so even if deltaEIS > MSD requirement or deltaTRS > MSD requirement, it does not mean to fail the MSD test.

Observation #2.2-5: High measurement uncertainty of radiated testing restricts the applicability of radiated MSD testing, e.g., for a measured small MSD result, it is difficult to judge it is real MSD or from MU.

Observation #2.2-6: One MSD test corresponds to two OTA test (before de-sense and after de-sense) which doubles the test time and cost compared with legacy OTA testing.

Observation #2.2-7: Radiated MSD testing is not affordable for the industry in terms of test time and cost.

Proposal #2.2-7: It is proposed to conclude that radiated MSD testing is not feasible from testability perspective.

With the above proposals, only two alternatives about remain in terms of MSD framework, and we can further evaluate their pros and cons further in greater detail. The simplification of MSD requirements can be considered in future meetings after the framework has been established.

Proposal #2.2-8: Further consider the following two alternatives to establish the 6G MSD framework

- Alt1: The MSD calculation and specification manner remain unchanged, i.e., accounting for both the conductive part and OTA part, which are then combined

- Alt2: Conductive-only MSD calculation and specification, i.e., the MSD calculation and specification only consider the conductive part. The terminology must be changed (e.g., MSDConductive) to differentiate with 5G.

For MSD reporting, it has been clarified in last meeting that the intention behind MSD reporting is to complement the MSD minimum requirements. The potential benefits of this mechanism are well understood. However, as expressed by all NW vendors during last meeting, MSD values have never been taken into account in any their scheduling to date, and it is very less likely to be incorporated in the scheduling in future. Therefore, we suggest not to introduce any MSD reporting scheme regardless of dynamic reporting (e.g., UE to report the actual MSD for any configurations requested by NW) or semi-static reporting (e.g., UE can report the actual MSD for certain specific configurations for each combo if NW requests such configurations), at least in 6G day 1.

Proposal #2.2-9: Not to consider any kind of MSD reporting scheme, at least in 6G day1.

- 3. Conclusion

Tx requirements

Proposal #2.1-1: For frequencies up to around 7GHz (i.e., up to 8.4GHz), adopt -35 dB for carrier leakage and -35 dB for image rejection.

Observation #2.1-1: In terms of the benefits of “fast LO retuning”, apart from the reduced MPR/A-MPR, there is also a reduction in power consumption achieved by reducing the clock rate of the baseband and D/A conversion.

Proposal #2.1-2: We recommend a detailed discussion on the concept of “fast LO retuning”, including aspects such as LO retuning time and interruption time.

Proposal #2.1-3: Strive to streamline MPR requirements when the performance differences are not significant across various RF architectures, Tx chain counts, and other relevant factors.

- For instance, for one device type, with same power class and mode (i.e., single carrier, or C/NC CA), strive to specify only one set of MPR requirements. This doesnot preclude other architecture implementations (i.e., those not selected as reference architecture for deriving requirements), provided the specified requirements are met.

Rx requirements

Other Rx requirements

Observation #2.2-1: It remains unclear whether the GSM network will be fully decommissioned prior to the finalization of the first 6G specification.

Proposal #2.2-1: All existing Rx requirements remain relevant for respective bands with a CBW of up to 100MHz. Same requirements framework/concept can be applicable to frequencies up to around 7GHz (i.e., up to 8.4GHz) with a CBW of up to 200MHz.

Proposal #2.2-2: For the derivation of REFSENS in the frequency range up to around 7GHz (i.e., up to 8.4GHz) with a CBP of up to 200MHz, the formula "REFSENS = -174 + 10lgCBW + NF + SNR + IM - Gdiv" shall still be adopted.

- NF and IM remains unchanged compared to 5G

Proposal #2.2-3: Remove ΔRIB from CA requirement

Proposal #2.2-4: The 2Rx REFSENS shall remain the baseline, with delta values applied to it to derive REFSENS for other Rx count such as 4Rx, 6Rx, 8Rx and 1Rx.

Proposal #2.2-5: For FDD REFSENS, PC3 is the basis, and delta values to higher power class can be added to derive HPUE REFSENS.

- For the same power class, only one set of requirements is defined, regardless of the number of Tx chains.

MSD

Proposal #2.2-6: MSD requirements should be introduced in 6G.

Observation #2.2-2: Radiated MSD testing at single direction e.g. based on EIS is not feasible because the EIS degradation not only contains MSD de-sense but also antenna gain variation between single carrier operation and multi-carrier operation.

Observation #2.2-3: Radiated MSD testing based on TRS is also questionable because the TRS degradation not only contains MSD de-sense but also antenna efficiency variation between single carrier operation and multi-carrier operation.

Observation #2.2-4: The reference point for MSD requirement is the specified REFSENS requirement rather than the UE’s real sensitivity performance, so even if deltaEIS > MSD requirement or deltaTRS > MSD requirement, it does not mean to fail the MSD test.

Observation #2.2-5: High measurement uncertainty of radiated testing restricts the applicability of radiated MSD testing, e.g., for a measured small MSD result, it is difficult to judge it is real MSD or from MU.

Observation #2.2-6: One MSD test corresponds to two OTA test (before de-sense and after de-sense) which doubles the test time and cost compared with legacy OTA testing.

Observation #2.2-7: Radiated MSD testing is not affordable for the industry in terms of test time and cost.

Proposal #2.2-7: It is proposed to conclude that radiated MSD testing is not feasible from testability perspective.

Proposal #2.2-8: Further consider the following two alternatives to establish the 6G MSD framework

- Alt1: The MSD calculation and specification manner remain unchanged, i.e., accounting for both the conductive part and OTA part, which are then combined

- Alt2: Conductive-only MSD calculation and specification, i.e., the MSD calculation and specification only consider the conductive part. The terminology must be changed (e.g., MSDConductive) to differentiate with 5G.

Proposal #2.2-9: Not to consider any kind of MSD reporting scheme, at least in 6G day1.

- 4. Reference

[1] R4-2522451, WF_6G_UE_RF, Feature lead (Qualcomm), RAN4#117

[2] R4-2513145, 6G General RF and UE RF, Nokia, RAN4#116bis

[3] R4-2513117, General and UE RF considerations for 6G, Ericsson, RAN4#116bis

[4] R4-2514626, WF on 6G Spectrum, Feature lead (Nokia), RAN4#116bis

[5]R4-2514642, WF for [116bis][102] 6G general RF and UE RF, Feature lead (Qualcomm), RAN4#116bis

[6] R4-2513021, Qualcomm views on 6G General and UE RF, Qualcomm, RAN4#116bis

[7] R4-2520825 Qualcomm views on 6G general and UE RF, Qualcomm, RAN4#117

[8] R4-2513349, Initial views on 6G UE RF requirements framework, Apple, RAN4#116bis

[9] R4-2601146, Discussion on UE RF testability for 6GR, Samsung, RAN4#118

[10] R4-2520554, 6GR General RF and UE RF, Nokia, RAN4#117

[11] R4-2520342, Views on 6G general RF and UE RF, MediaTek Inc., RAN4#117