# 3GPP TSG-RAN WG4 Meeting #118	                                                        R4-2600528

# Gothenburg, Sweden, Feb. 09-13, 2026

Source:	Apple

Title:	Discussion on AI-based SRS power imbalance compensation

Agenda item:	8.8.3

Document for:	Discussion

Introduction

SRS-assisted channel reconstruction is being explored as a scalable mechanism for acquiring downlink channel knowledge in future wireless systems, enabling significant reduction of CSI-RS and feedback overhead in large-scale MIMO deployments. In practical scenarios, uplink SRS observations are affected by UE-dependent transmit-chain distortions that break ideal reciprocity assumptions and can limit the robustness of AI/ML-based inference. We are discussing SRS-assisted reconstruction by incorporating compact, latent conditioning at the network side, allowing reliable channel inference across heterogeneous devices without requiring UEs to report their true hardware imbalance state.

Discussion On AI- based SRS power imbalance compensation

In 3GPP RAN4 discussions up to and including meeting RAN4#116bis, SRS-assisted AI/ML channel reconstruction was recognized as a potential enabler for reducing CSI-RS and feedback overhead in large-antenna 6G scenarios, while also exposing key robustness challenges. In particular, RAN4 acknowledged that uplink SRS measurements may exhibit UE-dependent distortions that impact reciprocity-based inference, and that classical non-AI compensation approaches have not reached consensus, especially under low-SNR or power-limited conditions.

It was therefore agreed that RAN4 should focus on defining evaluation assumptions and performance metrics for AI-assisted SRS interpretation, while avoiding specification of UE RF behavior or mandatory compensation mechanisms, and leaving detailed signal modeling and reconstruction aspects to RAN1-led studies.

SRS-assisted channel reconstruction is a scalable approach for acquiring downlink channel knowledge in future wireless systems, particularly as antenna counts and bandwidth continue to increase. By inferring downlink channel characteristics from uplink SRS observations, the network can reduce reliance on dense CSI-RS transmission and explicit feedback, which otherwise scale poorly in large MIMO deployments. The effectiveness of this approach depends on how accurately the uplink SRS observed at the gNB reflects the propagation channel relevant for downlink transmission.

In practice, uplink SRS measurements are influenced not only by the propagation environment but also by UE transmit-chain characteristics, such as antenna switching losses and port-dependent amplitude and phase offsets. These effects introduce structured, UE-dependent distortions that deviate from ideal reciprocity and vary across device types, frequency bands, and operating conditions. If not accounted for, such distortions can degrade the accuracy and generalization of AI/ML-based channel reconstruction, especially when a single model is expected to operate across heterogeneous UE populations.

To improve robustness under these conditions, SRS-assisted channel reconstruction can be extended to incorporate compact contextual representations of UE-dependent distortion directly into the network-side inference process. Rather than relying on explicit reporting of physical imbalance parameters, the network conditions its AI/ML model on low-dimensional latent descriptors that capture the dominant structure of the distortion in an abstract form. This allows the reconstruction process to adapt to device-specific characteristics while avoiding exposure of fine-grained RF states or continuous calibration values.

These latent descriptors may be realized through learned embeddings associated with broad hardware categories or through quantized indices selected from a shared set of representative imbalance patterns. In both cases, the uplink SRS observations are interpreted jointly with a compact context indicator, enabling reciprocity-consistent channel inference without modifying SRS signaling or requiring the UE to disclose its true imbalance state. This formulation could advance SRS-assisted channel reconstruction toward a hardware-aware inference framework, improving resilience to device diversity.

- Continue RAN4 evaluation of AI/ML-assisted SRS channel interpretation under UE-dependent distortion only if significant system-level gains are demonstrated. Focus on network-side conditioning techniques that enable such gains while allowing proprietary UE compensation to remain internal. Further consideration should remain contingent on demonstrable system-level gains while preserving UE implementation confidentiality and established RAN1/RAN4 boundaries.

Conclusions

In this paper, we discussed the key challenge for SRS power imbalance compensation. We made the following proposal:

- Continue RAN4 evaluation of AI/ML-assisted SRS channel interpretation under UE-dependent distortion only if significant system-level gains are demonstrated. Focus on network-side conditioning techniques that enable such gains while allowing proprietary UE compensation to remain internal. Further consideration should remain contingent on demonstrable system-level gains while preserving UE implementation confidentiality and established RAN1/RAN4 boundaries.

References

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR, Draft Technical Report, RAN #109.

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Technical Report