# 3GPP TSG-RAN WG4 Meeting #118	                                                          R4-2600529

# Gothenburg, Sweden, Feb. 09-13, 2026

Source:	Apple

Title:	Discussion on AI/ML use cases for RRM

Agenda item:	8.8.4

Document for:	Discussion

Introduction

This paper presents candidate AI/ML-based use cases for 6G RRM aimed at reducing measurement gaps, lowering UE power consumption, boosting throughput, and strengthening mobility robustness. Building on 3GPP studies in TR 38.744 [1] and TR 38.843 [2], it extends prediction from cell-level to beam- and multi-domain L3 RSRP across time, frequency, and spatial dimensions, and further advances spatial-domain prediction in beam management across Rx spatial and temporal domains.

Discussion On AI/ML use case for RRM : Track-1

During RAN4#117  the following agreements were reached:

Agreement:

1.	Postpone L1-beam level prediction for Tx beam

2.	Further discussion on the following identified the sub use cases under AI for RRM use case in RAN4#118 as the first priority

a.	For L3-beam level prediction for Tx beam

i.	FR1/FR2-1 spatial domain (intra-cell)

ii.	FR1/FR2-1 spatial domain (inter-cell, non-collocated)

iii.	FR1/FR2-1 frequency domain (inter-cell, non-collocated)

b.	For L3-cell level prediction

i.	FR1/FR2-1 freq. domain (inter-cell, non-collocated)

c.	Dynamic Adaptation of Measurement Procedure (single domain prediction based, no RSRP prediction is involved)

d.	Spatial domain RX beam sweeping reduction (L1/L3)

For the discussion that follows we discuss Topic 1(= 2aii with merged 2d), Topic 2(= 2bi with/without variant 2d), Topic 3 (= 2ai with/without variant 2d, Topic 4 (=2aii + frequency domain with/without variant 2d ),  Topic 5 (=2aii + frequency domain +time domain with/without variant 2d )

Topic 1: Spatial Domain Prediction (Inter-Site Geometry) considers an intra-frequency, inter-site (non-collocated) scenario where the core logic maps a Measured Set (B) to a Predicted Set (A). The input (Set B) consists of measurements from any set of currently visible cells, forming a flexible cluster fingerprint that does not strictly require the Serving Cell and can rely on strong neighbor-to-neighbor prediction. The output (Set A) is the beam-level quality for other target neighbor cells (Target Set). Variant 1A (Target Tx Beam Prediction) aims to predict the best Tx beam IDs for the Target Set (A) based on inputs from Set B across non-collocated cells. The benefit in terms of measurement overhead and scheduling comes from the reduction logic whereby the UE measures only the subset (Set B) instead of the full list (Set A). Multiple neighbors can be measured in parallel since SSBs are time-aligned and CDM across neighbor cell per carrier. By only measuring set B of beams, scheduling restrictions are reduced, leading to fewer silence periods where data cannot be scheduled and increased PDSCH/PUSCH availability.

Variant 1B (Target Tx + Rx Beam Prediction – Full Link Alignment) extends Variant 1A by predicting not only the Tx beam ID but also the specific UE Rx spatial filter (Rx beam) required to measure it. The goal is full link alignment, and the benefit is Rx beam optimization. On top of the gains from Variant 1A, this enhancement reduces the duration of the measurement window. Instead of sweeping all N Rx beams during a measurement occasion, the UE measures only a small number of Rx beams. The result is a further reduction in scheduling restrictions, as the busy time per measurement event is minimized, freeing up additional resources for data throughput.

In 5G networks, intra-frequency SSBs are time-aligned and often code-division multiplexed (CDM). When a UE opens its receive window for an SMTC occasion, it captures the composite signal containing multiple cells simultaneously. The "cost" of this measurement is primarily the duration of the window itself, not necessarily the number of cell IDs processed within that window. Consequently, the "scheduling restrictions" claimed to be reduced by Topic 1 are largely fixed by the SMTC duration. Predicting a subset of cells (Set A) from another subset (Set B) offers negligible scheduling gain because the UE has already paid the penalty of the measurement.

We contend that Topic 1 (Inter-Site Spatial Prediction) should be viewed as a distributed variant of Topic 3 (Intra-Cell Low SNR) rather than a distinct architectural modality. Both use cases rely on identical spatial prediction logic to map a Measured Set of beams to a Predicted Set. The primary distinction is merely geometrical: Topic 3 operates within a controlled, collocated environment, whereas Topic 1 attempts to resolve the same spatial correlations across non-collocated sites, introducing high geometric complexity.

Cluster-based approach inherent to Topic 1 could offer a specific technical advantage regarding noise reduction. By aggregating measurements from multiple neighbor cells simultaneously, the AI input (Set B) benefits from macro-diversity, which can effectively smooth out independent fading and measurement noise across the cluster.

However, despite this noise reduction benefit, the core objective of Topic 1, reducing intra-frequency measurement overhead, yields diminishing returns since the UE can already measure multiple intra-frequency cells simultaneously within a single SMTC window. Conceptually, Topic 1 and Topic 3 should be merged because they utilize the exact same "cluster-based" architectural logic to solve a spatial mapping problem. While Topic 3 is categorized as "Intra-Cell," its definition explicitly permits the input (Set B) to include measurements from "neighboring cells" to leverage spatial correlation across the site. This is mechanically identical to the definition of Topic 1, which uses measurements from "currently visible cells" to form a "flexible cluster fingerprint". Since Topic 3 already validates the capability to ingest multi-cell neighbor data to infer beam quality, it technically encompasses the "inter-site" mechanism defined in Topic 1.

Therefore, Topic 1 is best viewed simply as a specific configuration of Topic 3—a "cluster-based input" variant—rather than a distinct use case. In both scenarios, the AI performs the same core function: mapping a Measured Set (B) derived from the surrounding spatial geometry to a Predicted Set (A). Whether the target beam is on the same tower (Topic 3) or a non-collocated tower (Topic 1) is a secondary variable; the fundamental engine is a spatial predictor driven by a cluster of inputs. Merging them into a single "Spatial Domain Prediction" topic acknowledges that solving the cluster-based mapping in Topic 3 inherently solves the geometric challenge of Topic 1.

Topic 2: Frequency Domain Prediction (Inter-Site Cell Level) addresses a non-collocated inter-site scenario where the UE uses low-band (FR1) measurements to predict coverage on high-band (FR2) neighbor cells. Topic 2A (Cluster-Based Cell Level) focuses on predicting inter-frequency L3-RSRP for the target FR2 cell. The main benefit is measurement gap (MG) avoidance, achieved by skipping the measurement of frequency layers that are enabled through AI/ML-based cell-level prediction, thereby reducing the need for inter-frequency measurement gaps.

Topic 2B (Cluster-Based Frequency–Spatial (Rx)) extends this approach by predicting both inter-frequency L3-RSRP and the UE Rx beam. The mechanism uses inputs from spatial clusters across cells and Rx beams in FR1 to predict the FR2 cell-level RSRP. The main benefit is further measurement gap (MG) skipping, as MGs are further reduced by skipping Rx beam sweeps. The measurement gaps are significantly reduced by measuring only a targeted subset of Rx beams, which minimizes overhead and maximizes serving cell throughput.

While the primary classification of Topic 2 is 'Frequency Domain,' the definition explicitly scopes it as an inter-site, non-collocated scenario. This introduces a fundamental geometrical non-linearity that the AI must solve:

- The Spectral Challenge (Explicit): The AI must map propagation characteristics from a robust low band (FR1) to a volatile high band (FR2), accounting for frequency-dependent diffraction and blockage.

- The Geometrical Challenge (Implicit): Because the FR1 source and FR2 target are non-collocated, their spatial domains are decoupled. The AI cannot simply rely on a shared Angle of Arrival (AoA) or co-sited beam correspondence. Instead, it must infer the channel quality of a physically distant FR2 neighbor using only the local FR1 spatial cluster fingerprint.

Therefore, Topic 2 is effectively a dual-domain prediction problem (Spatial Geometry + Spectral Extrapolation). This makes it a rigorous stress test for the AI architecture. Validating this 'disjointed' prediction capability in Step 1 is essential because it proves the AI can handle non-linear geometric mappings before we introduce the additional complexity of time-domain forecasting in later steps

Topic 3: Spatial Domain Prediction (Intra-Cell Low SNR) focuses on an intra-site, collocated scenario targeting beam prediction under low-SNR (cell-edge) conditions, where both the input measurements and the prediction targets belong to the same physical site or cell. The core logic maps a Measured Set (B) to a Predicted Set (A) within the same site or cell. The flexible input (Set B) consists of measurements from the target cell itself, using a subset of strong beams, and optionally from collocated or neighboring cells such as the serving cell or other sectors. This forms a cluster-based input that leverages spatial correlation across the entire site. The output (Set A) is the full set of beam-level quality metrics (L1/L3-RSRP) for the target cell. Topic 3A (Intra-Site Tx Beam Recovery at the Cell Edge) aims to predict Tx beam IDs for the target cell (Set A) using only a sparse subset of measurements (Set B), where the AI reconstructs the full beam power spectrum from partial and noisy inputs to identify optimal beams even when they are buried in noise under low-SNR conditions.

Topic 3B (Intra-Site Spatial Alignment – Rx Beam) extends the framework by predicting both the Tx beam ID and the optimal UE Rx beam index. The main benefit is Rx efficiency, as the UE can switch directly to the predicted Rx beam without performing a full Rx beam sweep. This yields a clear scheduling gain by eliminating the silence periods required for Rx beam training, thereby maximizing throughput even in challenging cell-edge conditions, while also preserving the robustness and overhead reduction benefits achieved through low-SNR recovery and reduced exhaustive beam sweeping.

Topic 4: Spatio-Spectral Prediction (Inter-Site Beam Level) represents the superset of Topic 1 and Topic 2 by combining inter-site spatial geometry with inter-frequency extrapolation. In this context, the UE uses low-band (FR1) measurements from the serving cell and neighbor cells to predict high-band (FR2) beam-level quality on non-collocated neighbor sites. Topic 4A (Cross-Band Tx Beam Prediction – Tx Only) targets the prediction of specific FR2 Tx beam IDs (Set A) using FR1 inputs (Set B). The benefits are measurement gap (MG) avoidance and reduction. MG elimination is enabled through frequency-domain AI/ML prediction following the Topic 2 logic, while measurement gap length (MGL) reduction is enabled through spatial prediction following the Topic 1 logic. When an MG is still required, the UE measures only the predicted subset of Tx beams (Set B) instead of performing a full blind sweep over Set A, thereby reducing the required MGL.

Topic 4B (Cross-Band Tx + Rx Beam Prediction – Full Link) extends Topic 4A by predicting both the FR2 Tx beam ID and the optimal UE Rx beam index from FR1 inputs. The benefit is the maximization of MG reduction through cumulative gains, which include all advantages of Topic 4A such as gap avoidance and reduced Tx beam search. In addition, Rx beam sweep avoidance further reduces the MGL by eliminating the need for the UE to sweep its Rx beams. Instead, the network opens minimal micro-gaps solely to measure the predicted Tx beam over a small number of Rx beams, maximizing availability for serving-cell data transmission.

Topic 5: Spatio-Temporal-Spectral Prediction (Inter-Site Beam Level) is the superset of Topic 4 augmented with time-domain prediction, extending the spatio-spectral logic by incorporating a temporal dimension to forecast future channel states. In this context, the UE uses historical and current low-band (FR1) measurements to predict future high-band (FR2) beam-level quality at time t+Δt. Topic 5A (Spatio-Temporal Tx Prediction) focuses on predicting future FR2 Tx beam IDs based on user trajectory and mobility trends. The benefits include time-domain efficiency and enhanced mobility. It inherits all measurement gap (MG) reduction benefits of Topic 4A, while additionally achieving time-domain MG reduction by skipping measurement patterns in time. Periodic physical measurements are replaced by AI/ML forecasts, reducing the total number of required gaps. This also enables enhanced mobility support, such as conditional handover (CHO) and reduced handover latency, by allowing the network to prepare target-cell resources in advance based on forecasted signal trends.

Topic 5B (Spatio-Temporal Tx + Rx Prediction) extends Topic 5A by predicting both the future FR2 Tx beam and the optimal future UE Rx beam index. The benefits are proactive alignment and maximized overhead reduction. It inherits all benefits of Topic 4B, including gap avoidance and Rx sweep removal, and further maximizes efficiency by combining skipped time instances (time-domain) with skipped Rx sweeps (spatial-domain). This proactive beam alignment improves beam reliability during high-speed mobility by reducing beam failure risk. The UE can proactively rotate its Rx beam to match the predicted future angle of arrival, enabling zero-interruption maintenance of the link.

To ensure credible standard impact and manageable technical risk, the investigation order must validate the core prediction modalities—spatial and spectral—under controlled conditions before exposing them to full inter-site and cross-domain complexity. The priority strategy therefore emphasizes early robustness proofs and clear isolation of failure modes, enabling confident integration into higher-order supersets.

Rationale for Investigation Prioritization. The strategy should focus on de-risking the fundamental modalities of prediction—spatial and spectral—independently before integrating them. Topic 3 (Intra-Site Spatial / Low SNR) is the preferred starting point because it provides an ideal controlled environment to validate spatial prediction techniques. Since both source and target beams originate from collocated or adjacent sectors on the same site, the underlying geometrical correlations are strong and relatively deterministic compared to inter-site scenarios. By explicitly targeting low-SNR (cell-edge) conditions, Topic 3 addresses the most critical challenge for AI-based RRM: robustness when physical measurements are unreliable. Demonstrating that AI can reconstruct accurate beam-level quality from sparse, noisy inputs within a known geometry is a necessary prerequisite before tackling more complex and less predictable inter-site spatial relationships.

In parallel, Topic 2 (Frequency Domain Prediction) addresses the second fundamental pillar: cross-band spectral correlation. It isolates the challenge of mapping propagation characteristics from a more robust low band (FR1) to a more volatile high band (FR2), which is central to 6G efficiency goals where continuous FR2 monitoring is power- and overhead-intensive. Investigating this topic independently validates the ability of AI/ML to learn frequency-dependent pathloss and blockage behavior, directly targeting the primary RAN4 pain point of inter-frequency measurement gaps. While Topic 1 (Inter-Site Spatial Prediction) is important, it introduces high geometric complexity at an early stage. Because its core benefit—measurement and scheduling overhead reduction via prediction—is functionally aligned with Topic 3, Topic 3 provides a safer and cleaner spatial proof-of-concept. Moreover, the combined insights from Topic 3 (robust spatial reconstruction) and Topic 2 (non-collocated frequency-domain inference) form the necessary building blocks to move directly into Topic 4, the superset that jointly addresses inter-site geometry and inter-frequency prediction.

Figure 1: AI/ML for 6R RRM Topics and dependencies

Proposed Investigation Roadmap: AI/ML for 6G RRM. Based on the rationale above, a phased investigation roadmap is proposed to systematically build complexity while validating performance benefits at each stage. Step 1 focuses on foundational pillar investigation in a parallel track, with the objective of establishing baseline confidence in the two core prediction modalities independently. This includes Topic 2 (Frequency Domain Prediction), which aims to demonstrate that robust FR1 measurements can reliably predict FR2 cell coverage and spatial alignment, with success measured by a significant reduction or elimination of inter-frequency measurement gaps while maintaining FR2 accessibility. In parallel, Topic 3 (Spatial Domain Intra-Cell / Low SNR) targets proof that AI can accurately infer complete beam sets from sparse inputs within a collocated geometry under challenging cell-edge noise conditions. Success in this topic is defined by reliable beam recovery at low SNR where legacy reporting fails, thereby reducing the need for exhaustive Rx sweeping and easing scheduling restrictions.

Step 2 and Step 3 extend these validated capabilities toward realistic and proactive 6G deployment scenarios. Step 2 (Integration and Superset Validation) centers on Topic 4 (Spatio-Spectral Prediction), which acts as the superset of Topic 1 and Topic 2 by combining spatial and spectral inference. The objective is to use FR1 measurements from serving and neighbor cells to predict precise Tx and Rx beam quality on non-collocated FR2 neighbor sites, with success measured by achieving measurement-free inter-site, inter-frequency mobility and maximizing spectral efficiency through the removal of both spatial and spectral measurement overhead. Building on this, Step 3 (Temporal Extension) introduces Topic 5 (Spatio-Temporal-Spectral Prediction) as the superset of Topic 4 with an added time dimension. The objective is to forecast future beam quality based on user mobility trends to enable proactive RRM, with success defined by enabling conditional handover with near-zero interruption and preemptively avoiding beam failure through predictive Rx beam alignment.

Regarding the Dynamic Adaptation of Measurement Procedure, current 5G NR measurement procedures were designed under the assumption that radio conditions evolve gradually and that exhaustive, periodic measurements across configured cells, beams, and frequency layers are necessary to ensure mobility robustness. While this assumption was acceptable for sub-6 GHz deployments with limited spatial resolution, it becomes increasingly inefficient and, in some cases, unsafe for 5G evolution toward FR2 and early 6G systems. Narrow beams, dense multi-TRP deployments, and multi-frequency operation dramatically increase the measurement search space, forcing UEs to perform frequent and wide-scope measurements even when the underlying radio environment is stable.

This rigid, worst-case-driven measurement paradigm leads to excessive power consumption, unnecessary measurement gaps, increased latency in detecting relevant mobility candidates, and reduced user throughput. Importantly, the current framework lacks any mechanism to differentiate between environments that require dense, frequent measurements (e.g., high mobility, beam-volatile scenarios) and those that are inherently stable (e.g., stationary or slowly varying channels). As a result, measurement procedures are applied uniformly, independent of actual space-time-frequency channel dynamics observed at the UE.

Extending 5G toward 6G therefore requires a principled evolution of measurement procedures that allows adaptation based on predicted channel stability and relevance, while preserving the existing network-controlled RRM architecture and safety guarantees. AI/ML-assisted inference at the UE can be used to assess stability and relevance across spatial, temporal, and frequency domains, enabling dynamic adjustment of when measurements are performed and what measurement scope is required. Such adaptation does not replace legacy measurements, but complements them by allowing conservative reduction when confidence is high and automatic fallback when uncertainty increases.

Figure 2: RRM AI/ML Investigation Prioritization

- We propose defining the study's technical scope by structuring each AI/ML RRM prediction use case into Variant A (Tx-only prediction) and Variant B (joint Tx+Rx beam prediction). This inclusion is critical to leverage the significant measurement overhead and gap reductions identified from skipping unnecessary Rx beam sweeps

- We propose organizing the study into two independent investigation tracks. Track 1 follows a phased model development roadmap: Step 1 validates foundational spatial (T3, including cluster variants) and spectral cell-level (T2) prediction. Step 2 integrates these into the spatio-spectral superset (T4), followed by Step 3 extending into the time domain (T5). Independently, Track 2 investigates "Dynamic RRM Measurement Adaptation" mechanisms, focusing on defining triggers and scenarios for relaxing measuring legacy procedures, and evaluating system benefits.

Discussion On AI/ML use case for RRM : Track-2

We will present some use cases for Track-2: Dynamic Measurement Adaptation and the role of the AI/ML.

(1) Assistance-Aided Activation of AI-Based Inter-Frequency Prediction

Inter-frequency mobility is a critical component of NR network operation, ensuring seamless service continuity across different carrier frequencies and radio access technologies. Traditionally, as defined in RAN4 specifications (e.g., TS 38.133), inter-frequency measurements require the network to configure measurement gaps (MGs). During these gaps, the UE tunes its RF chain away from the serving cell to measure target frequencies. While necessary, measurement gaps interrupt DL/UL data transmission, degrade system throughput, and increase latency.

Recent studies in RAN1 (TR 38.843) have demonstrated the feasibility of using AI/ML models at the UE to predict radio conditions on a target frequency based on measurements taken on the currently active reference frequency (serving cell). If successful, this allows the UE to generate inter-frequency RRM reports (e.g., RSRP, SINR) without requiring measurement gaps, offering significant spectral efficiency gains.

However, the fundamental premise of this approach, that the reference channel and target channel are sufficiently correlated, is not universally valid. The spatial and temporal correlation across frequencies depends heavily on:

- Frequency Separation: The correlation between FR1 (e.g., 2 GHz) and FR2 (e.g., 28 GHz) is often negligible due to vastly different propagation characteristics and blockage sensitivity.

- Local Geometry: Specific obstacles in the UE’s immediate vicinity may block the target frequency while leaving the reference frequency unaffected, instantaneously breaking correlation.

- UE Mobility State: Rapid changes in orientation or location can alter multipath profiles faster than the model can adapt.

Currently, the gNB (RAN2 protocol entity) has no standardized mechanism to determine the instantaneous reliability of the UE's AI-based cross-frequency inference. The gNB faces a binary deployment choice:

- Risky Activation: Blindly enable AI-based reporting to eliminate gaps, risking inaccurate measurements, RLF (Radio Link Failure), or ping-pong handovers when correlation is weak.

- Conservative Fallback: Permanently disable AI and rely on legacy measurement gaps, forfeiting potential capacity gains even when conditions for AI prediction are ideal.

To overcome this, a dynamic mechanism is required where the network retains control over measurement configuration but is assisted by UE-side visibility into the local feasibility of AI prediction.

The Exact Role of AI/ML and the Proposed Indicator: The AI Confidence Assessment

Crucially, this indicator is not the predicted measurement value itself (e.g., RSRP), but rather meta-data about the reliability of the UE's prediction engine in the current local context. This dynamic confidence assessment ensures the network only utilizes AI inference when performance is guaranteed to be reliable.

It could take the form of:

- Option A (Binary): AI_Reliability_Flag (0 = Unreliable/Correlation Lost; 1 = Reliable).

- Option B (Multi-level): A 2-bit indicator defining confidence zones (e.g., High Confidence, Medium Confidence, Low Confidence, Out-of-Distribution).

Figure 3: Assistance-Aided Activation of AI-Based Inter-Frequency Prediction

- Investigate the use of a UE-signaled "AI Prediction Confidence Level" as a primary trigger for dynamic measurement adaptation mechanisms. This enables the network to adaptively switch between gap-less AI inference and legacy measurement gap configurations based on real-time local model reliability.

- (2) Prioritized Identification and Monitoring of Inter-Frequency Candidates

In dense NR networks deployed across multiple frequency layers, UEs are often configured with large lists of potential inter-frequency target cells to ensure seamless mobility. Current standardized procedures typically treat all carrier cells in this configured set uniformly. This forces the UE to expend equal effort, in terms of measurement gaps, power consumption, on every carrier, regardless of whether a specific candidate is a strong, likely handover target or a weak, redundant one or not detectable in the UE’s region. This uniform, worst-case approach decouples measurement effort from actual mobility value. Consequently, as network density increases, the aggregate measurement overhead scales unsustainably, wasting UE resources on low-value targets while failing to focus efforts on the small subset of cells that are truly critical for maintaining connectivity.

The objective of this proposal is to investigate a dynamic prioritization framework that decouples measurement effort from the size of the configured candidate list. By differentiating inter-frequency candidates based on their relative value to imminent mobility decisions, the framework aims to:

- Reduce Aggregate Overhead: Significantly lower the total measurement gap time, power consumption, and signaling burden on the UE by reducing the monitoring intensity for low-value candidates.

- Maintain Mobility Robustness: Ensure that physical measurement resources are concentrated on a limited set of high-value, decision-critical targets to secure primary mobility paths with legacy-grade robustness.

- Enable Intelligent Monitoring: Leverage AI/ML-assisted inference and infrequent validation to maintain sufficient awareness of lower-value candidates without the high cost of continuous physical measurement.

In this use case, the AI/ML model deployed at the UE functions primarily as an intelligent, context-aware classifier and ranking engine.

The AI/ML engine operates by digesting real-time radio conditions and contextual history to classify configured frequency layers into actionable priority categories:

- Inputs: The model ingests current serving cell metrics (e.g., L1-RSRP, SINR trends, Doppler shift/velocity estimates), rough geographical context (if available), and crucial historical data, such as previous successful handover targets along similar routes or time-of-day network loading patterns.

- Processing (Classification): The model processes these inputs to determine the probability that a specific neighboring frequency layer contains a viable cell that will trigger a handover event in the near horizon.

- Outputs (Priority Labels): The AI assigns a priority status to each configured inter-frequency layer. For example:

  - High-Value/Critical: Layers with a high historical probability of being a handover target in this specific context.

  - Opportunistic/Low-Value: Layers that are historically weak or absent in the current environment, unlikely to yield better targets than the serving cell.

Figure 4: Track2: Prioritized Identification and Monitoring of Inter-Frequency Candidates

- Investigate a UE-assisted AI/ML prioritization framework that classifies inter-frequency candidates into high-value and opportunistic categories based on handover utility and historical context. This enables the network to dynamically concentrate physical measurement resources on decision-critical targets while reducing the monitoring overhead for low-priority layers.

- (3) UE-Assisted Context-Aware Beamwidth and Stability Indicator

In FR2/FR3 deployments with narrow beamforming, beam stability can change on time scales that are shorter than conventional measurement reporting, filtering, and network-side reaction loops. Sudden UE motion, rotation, or local blockage can cause rapid beam misalignment or link degradation before the gNB can detect and respond based on standard radio measurements such as RSRP or SINR. As a result, beamwidth selection and beam management based solely on delayed radio measurements can be sub-optimal, leading to increased beam failure events, interruption, and reduced mobility robustness. This creates a timing and observability gap between the UE’s instantaneous link dynamics and the network’s reactive RRM decisions.

The objective is to investigate a UE-assisted framework in which the UE predicts short-term beam stability using AI/ML and reports a compact beam stability and/or beamwidth adaptation indicator to the network. This indicator is used by the gNB as an additional input to dynamically adapt beamwidth selection and beam management measurement configurations, such as measurement periodicity. The goal is to enable proactive network measurement adaptation that balances performance and robustness, reducing reliance on purely reactive decisions based on delayed radio measurements and improving mobility and beam management reliability in narrow-beam deployments.

The UE-side AI/ML model predicts short-term beam stability based on a combination of UE-available information and radio context. The specific model architecture and feature set are implementation specific. Conceptually, the inputs may include:

- Recent beam-level radio measurements, such as beam-specific RSRP, SINR, or CSI-related metrics, capturing short-term beam quality trends

- Temporal dynamics of beam measurements, including short-term variation, or beam switching history

- Mobility-related indicators derived from radio behavior, such as Doppler-related effects, rapid RSRP fluctuation, or handover/beam failure precursors

- UE-local context signals available at the UE that are correlated with short-term link stability

The UE AI/ML model produces compact, standardized outputs that summarize predicted short-term beam stability and suitability of narrow-beam operation. Conceptually, the outputs may include one or more of the following:

- Beam Stability Indicator
A discrete or quantized indicator representing the predicted short-term stability of the current or candidate narrow-beam configuration (e.g., Stable / Marginal / Unstable).

- Beamwidth Preference Indicator
An indicator reflecting whether narrow-beam operation is predicted to be suitable or whether wider, more robust beams are recommended (e.g., Narrow-Beam-Suitable vs Wide-Beam-Recommended).

- Time-to-Degradation / Time-to-Failure Indicator (Optional)
A coarse estimate or class-based indicator of the predicted time horizon over which the current narrow-beam configuration is expected to remain reliable (e.g., Short / Medium / Long, or quantized time buckets).

- Prediction Reliability or Confidence Metric (Optional)
An associated confidence or reliability measure indicating the trust level of the prediction, enabling the network to apply risk-aware policies when adapting beam and measurement configurations.

Figure 4: Track2: UE-Assisted Context-Aware Beamwidth and Stability Indicator

- Investigate a UE-assisted framework in which the UE predicts short-term beam stability using AI/ML and reports a compact beam stability and/or beamwidth adaptation indicator. The gNB uses this indicator to dynamically adapt beamwidth selection and beam management measurement configurations.

Conclusions

In this paper, we discussed candidate AI/ML-based use cases for 6G RRM, highlighting approaches to reduce measurement gaps, lower UE power consumption, increase throughput, and enhance mobility robustness. We made the following proposals:

- We propose defining the study's technical scope by structuring each AI/ML RRM prediction use case into Variant A (Tx-only prediction) and Variant B (joint Tx+Rx beam prediction). This inclusion is critical to leverage the significant measurement overhead and gap reductions identified from skipping unnecessary Rx beam sweeps

- We propose organizing the study into two independent investigation tracks. Track 1 follows a phased model development roadmap: Step 1 validates foundational spatial (T3, including cluster variants) and spectral cell-level (T2) prediction. Step 2 integrates these into the spatio-spectral superset (T4), followed by Step 3 extending into the time domain (T5). Independently, Track 2 investigates "Dynamic RRM Measurement Adaptation" mechanisms, focusing on defining triggers and scenarios for relaxing measuring legacy procedures, and evaluating system benefits.

- Investigate the use of a UE-signaled "AI Prediction Confidence Level" as a primary trigger for dynamic measurement adaptation mechanisms. This enables the network to adaptively switch between gap-less AI inference and legacy measurement gap configurations based on real-time local model reliability.

- Investigate a UE-assisted AI/ML prioritization framework that classifies inter-frequency candidates into high-value and opportunistic categories based on handover utility and historical context. This enables the network to dynamically concentrate physical measurement resources on decision-critical targets while reducing the monitoring overhead for low-priority layers.

- Investigate a UE-assisted framework in which the UE predicts short-term beam stability using AI/ML and reports a compact beam stability and/or beamwidth adaptation indicator. The gNB uses this indicator to dynamically adapt beamwidth selection and beam management measurement configurations.

References

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR, Draft Technical Report, RAN #109.

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Technical Report