3GPP TSG-RAN WG4 Meeting #118	                                                                      R4-2600541

Gothenburg, Sweden, Feb. 09-13, 2026

Agenda Item:	6.11.3

Source: 	Apple

Title: 	RRM performance requirements for positioning accuracy

Document for:	Discussion

# Discussion on potential RAN4 requirement for performance monitoring

During prior RAN1 meetings the following agreements were reached:

RAN1#116bis Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. RAN1#118 For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. RAN1#119 Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B RAN1#120bis Agreement For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method. RAN1#121 Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - There is no consensus in RAN1 to support Option B (including Option B-1 and B-2).

Performance monitoring is essential for AI/ML-enabled positioning systems to ensure reliable operation and maintain positioning accuracy over time. As AI/ML models are deployed in the field, their performance can degrade due to various factors such as changes in the radio environment, hardware variations, or distribution shifts from the training data. Without proper performance monitoring requirements, there is no systematic way to detect when a model is no longer performing adequately, which could lead to unacceptable positioning errors and degraded user experience. Defining clear performance monitoring requirements ensures that both the network and UE can detect performance degradation in a timely manner and take appropriate actions such as model switching, retraining, or fallback to legacy positioning methods.

From a standardization perspective, performance monitoring requirements are necessary to ensure interoperability between different vendors' implementations and to establish baseline expectations for AI/ML positioning systems. The RAN1 agreements have identified feasible options for monitoring metric calculation in both Case 1 (UE-side positioning) and Case 3a (network-side positioning), with Option A being supported for Case 1 where the target UE performs monitoring metric calculation. However, without RAN4 requirements that specify timing constraints and performance thresholds, implementations may vary significantly in their responsiveness to model degradation. These requirements would define critical aspects such as the maximum delay for monitoring metric calculation, the latency for signaling monitoring outcomes to the LMF, and potentially threshold values that trigger monitoring actions.

The complexity of the monitoring procedures, particularly for Case 1 Option A-1 where ground truth labels are generated by LMF and provided to the target UE, introduces multiple signaling exchanges that could impact overall system latency. RAN4 requirements would ensure that these procedures do not introduce excessive delays that could affect the real-time nature of positioning services. Furthermore, defining these requirements will provide clear guidance for implementation and testing, enabling the industry to develop AI/ML positioning solutions that meet consistent quality standards across different deployment scenarios.

For Case 1 we can consider the following delay components:

1. Maximum Delay for Ground Truth Label Delivery from LMF to Target UE

In Case 1, the AI/ML model operates at the UE side to predict the UE's own position based on channel measurements (e.g., PRS measurements). For label-based monitoring, the UE needs to compare its AI/ML predictions against "ground truth" to calculate monitoring metrics (e.g., prediction error, accuracy).

- The LMF generates the ground truth label (actual UE position) using traditional positioning methods or other reference data

- This ground truth must be delivered to the target UE so it can evaluate its own AI/ML model's performance

- Timing impact: If this delivery is too slow, the ground truth may not correspond temporally to the AI/ML prediction being evaluated (the radio environment may have changed), making the monitoring metric meaningless

- The delay should be short enough that the ground truth still represents the same positioning scenario as when the AI/ML model made its prediction

2. Maximum Delay for Monitoring Metric Calculation at the Target UE

Once the UE has both its AI/ML prediction and the ground truth label, it needs to calculate monitoring metrics (e.g., positioning error, confidence levels, KPIs).

- The UE compares: AI/ML predicted position vs. ground truth position

- Calculates metrics such as positioning error magnitude, whether error exceeds threshold, model confidence scores

- Timing impact: This processing delay affects how quickly the system can detect model degradation

- If the UE takes too long to compute metrics, it delays the detection of poor model performance, potentially allowing the system to continue using a degraded model for extended periods

- Fast calculation enables timely detection and response to performance issues

3. Maximum Delay for Monitoring Outcome Signaling from Target UE to LMF

After calculating monitoring metrics, the UE needs to report the monitoring outcome back to the LMF. Based on RAN1 agreements, this includes at least "an indication that the target UE cannot perform the Case 1 positioning method."

- The UE informs the LMF about monitoring results: model is performing well, model is degraded, or model cannot perform Case 1 positioning

- Timing impact: This signaling delay determines how quickly the network can react to model performance issues

- Fast signaling enables the LMF to:

  - Switch to alternative positioning methods (e.g., fallback to legacy positioning)

  - Trigger model updates or retraining

  - Initiate model switching to a different AI/ML model

  - Adjust positioning service parameters

- Slow signaling means the network remains unaware of UE-side model issues, potentially degrading positioning service quality

## End-to-End Impact on Case 1 AI/ML Positioning

## The total monitoring loop delay is the sum of these three components:

## Total Monitoring Delay = Ground Truth Delivery + Metric Calculation + Outcome Signaling

Proposal 1: For AI/ML positioning Case 1 performance monitoring, RAN4 should define delay requirements for at least Option A-1, covering:

- Maximum delay for ground truth label delivery from LMF to target UE

- Maximum delay for monitoring metric calculation at the target UE

- Maximum delay for monitoring outcome signaling from target UE to LMF

# Conclusions

In this contribution, we have got the following proposals:

Proposal 1: For AI/ML positioning Case 1 performance monitoring, RAN4 should define delay requirements for at least Option A-1, covering:

- Maximum delay for ground truth label delivery from LMF to target UE

- Maximum delay for monitoring metric calculation at the target UE

- Maximum delay for monitoring outcome signaling from target UE to LMF

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface