3GPP TSG-RAN WG4 Meeting #118                                       	                       R4-2600568

# Gothenburg, Sweden, February 9 – February 13, 2026

Agenda item:	6.1.2

Source:	Apple Inc.

Title:	On UE Demodulation Performance Requirements for 6Rx UEs

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

After finalizing the discussion on the maximum number of MIMO layers for 6Rx handheld UEs, in this paper we propose the scope of the 6Rx Demodulation Performance and CSI Reporting Requirements.

# Applicability of Requirements

Almost complete progress has been achieved and the rest of the open items were all closed during RAN4#117 in Dallas. For this last meeting, the only topic that needs to be concluded is with respect to how to capture the applicability rules for 6 layer requirements and which language to use and in which sections of TS 38.101-4. The past RAN4#117 Chairman Notes capture the following:

Since maxNumberMIMO-LayersPDSCH-v1900 is a proper capability, common practice in TS 38.101-4 is to capture the applicability of every feature in Section 5.1.1 and corresponding subsections. From our point of view, we see no reason to deviate from this practice, since we believe it’s the best and clearest way to avoid any confusion across 3GPP specifications, and this should include a clear indication that the feature is supported only for FWA. RAN2 has already captured this fact in their specifications, and now it’s time for RAN4 to follow suit and capture the details of 6-layer support in its own specification.

Observation #1: Common practice in TS 38.101-4 is to thorougly describre applicabily of requirements in Section 5.1.1 and corresponding subsections, possibly including further “Applicability Notes”.

Based on the past way forward, we believe that we should capture test applicability in both sections 5.1.1 and 5.2. For a common reader, requirements should be straightforward to understand.

Hence, we propose a new “Option 3” which can propose both mechanisms stated above. Both options are valid, and they don’t necessarily oppose to each other.

Proposal #1: Explicitely capture that 6 MIMO layers requirements are only applicable to FWA devices in 38.101-4 in Section 5.1 Applicability of Requirements, by also using “Applicability Notes”, and via Section 5.2 including “Notes” in the specific requirements sections.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: Common practice in TS 38.101-4 is to thorougly describre applicabily of requirements in Section 5.1.1 and corresponding subsections, possibly including further “Applicability Notes”.

Proposal #1: Explicitely capture that 6 MIMO layers requirements are only applicable to FWA devices in 38.101-4 in Section 5.1 Applicability of Requirements, by also using “Applicability Notes”, and via Section 5.2 including “Notes” in the specific requirements sections.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024